| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0227 WG11 Number: m27564 |
| --- | --- |

| Title: | VPS extension with updates of profile-tier-level syntax structure |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Munsi Haque 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4099 munsi.haque@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This contribution presents some possible updates for the profile_tier_level syntax structure to support additional flexibility for HEVC extensions while being used in vps_extension syntax structure for each layer or operating point.

- On profile_tier_level Syntax Structure

Below is the profile_tier_level syntax structure present in  the current DIS [2].

| profile_tier_level( profilePresentFlag, maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| if( profilePresentFlag ) { |  |
| general_profile_space | u(2) |
| general_tier_flag | u(1) |
| general_profile_idc | u(5) |
| for( i = 0; i < 32; i++ ) |  |
| general_profile_compatibility_flag[ i ] | u(1) |
| general_reserved_zero_16bits [Ed. (GJS): Adjust semantics accordingly.] | u(16) |
| } |  |
| general_level_idc | u(8) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) { |  |
| if( profilePresentFlag ) |  |
| sub_layer_profile_present_flag[ i ] | u(1) |
| sub_layer_level_present_flag[ i ] | u(1) |
| if( profilePresentFlag && sub_layer_profile_present_flag[ i ] ) { |  |
| sub_layer_profile_space[ i ] | u(2) |
| sub_layer_tier_flag[ i ] | u(1) |
| sub_layer_profile_idc[ i ] | u(5) |
| for( j = 0; j < 32; j++ ) |  |
| sub_layer_profile_compatibility_flag[ i ][ j ] | u(1) |
| sub_layer_reserved_zero_16bits[ i ] | u(16) |
| } |  |
| if( sub_layer_level_present_flag[ i ] ) |  |
| sub_layer_level_idc[ i ] | u(8) |
| } |  |
| } |  |

The presence of the profile_tier_level related syntax parameters is currently conditioned (restricted) by an external parameter flag of profilePresentFlag used as a "passing" parameter in the profile_tier_level syntax structure in the current DIS [2]. The following table shows what parameters can be conditioned (restricted) when this syntax structure is used in vps_extension() syntax structure to support various HEVC extensions requirements.

Selectable syntax parameters in the Current profile_tier_level (DIS)

| profilePresentFlag | sub_layer_profile_present_flag | levelPresentFlag | sub_layer_level_present_flag | Syntax parameters- General | Syntax parameters- Sub-layer |
| --- | --- | --- | --- | --- | --- |
| 0 | - | - | 0 | general_level_idc | - |
| 0 | - | - | 1 | general_level_idc | sub_layer_level_idc |
| 1 | 0 | - | 0 | general_profile_tier_level | - |
| 1 | 1 | - | 0 | general_profile_tier_level | sub_layer_profile_tier |
| 1 | 0 | - | 1 | general_profile_tier_level | sub_layer_level_idc |
| 1 | 1 | - | 1 | general_profile_tier_level | sub_layer_profile_tier_level |

It is quite obvious from the chart above that one of the parameter-group, called "general_prfile_tier" that contains general syntaxes for "profile_tier only", cannot be selected alone for possible use in vps_extension() syntax structure. In other words, all possible choices may not be possible with the existing profile_tier_level syntax structure. This can be attributed to the absence of a "levelPresentFlag", another conditional flag, for the profile_tier_level syntax structure.

Profile, Tier and Level Constraints

In the current HEVC specifications, Profile and Tier related parameters may be varied per Layer or Operating Point based upon setting the ProfilePresentFlag to 1 or 0. The Level parameters are always adapted in General case and if sub_layer_level_present_flag is present, then sub-layer related level parameters can be changed.

However, from the Profile/Tier/Level descriptions in Annex-A, the Level parameters can be common to some profile/tier cases and may change for other cases. This contribution presents a possibility that for general case, Level parameters may change separately from Profile/Tier Parameter changes. In other words,  Level parameters could be varied by setting a new flag of LevelPresentFlag to 1 or 0.

## Updates on profile_tier_level Syntax Structure

The following chart shows how all possible combinations of profile_tier and level parameters in a modified profile_tier_level syntax structure are possible during its application in vps_extension() when the "levelPresentFlag" is present as a passing parameter similar to "profilePresntFlag" or as an added syntax inside the profile_tier_level syntax structure.

Selectable syntax parameters in the Updated profile_tier_level (Proposal)

| profilePresentFlag | sub_layer_profile_present_flag | levelPresentFlag | sub_layer_level_present_flag | Syntax parameters- General | Syntax parameters- Sub-layer |
| --- | --- | --- | --- | --- | --- |
| 0 | - | 0 | - | - | - |
| 0 | - | 1 | 0 | general_level_idc | - |
| 0 | - | 1 | 1 | general_level_idc | sub_layer_level_idc |
| 1 | 0 | 0 | - | general_profile_tier | - |
| 1 | 1 | 0 | - | general_profile_tier | sub_layer_profile_tier |
| 1 | 0 | 1 | 0 | general_profile_tier_level | - |
| 1 | 1 | 1 | 1 | general_profile_tier_level | sub_layer_profile_tier_level |

Key Observations:

- profilePresentFlag enables restrictions (presence) of profile and tier syntaxes in the syntax structure above for both general and sub-layer cases

- No such flag is present to affect (restrict) the LevelIDC syntax parameters for both general and sub-layer cases in the current profile_tier_level syntax structrue [2]. Thus no restrictions on the presence of LevelIdc can be imposed either for a Layer or an Operating Point (OP) in vps_extension(). It may be added that Level restriction is OP-specific in case of VPS, as commented in " JCTVC-K_Notes_dD".

- So the profile_tier_level syntax structure can be updated as follows: use of "levelPresentFlag" as a passing parameter similar to "profilePresentFlag", or insert levelPresentFlag as a new 1-bit syntax.

An updated profile_tier_level syntax structure is shown below with levelPresentFlag as a passing parameter, similar to profilePresentFlag.

| profile_tier_level( profilePresentFlag, levelPresentFlag, maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| if( profilePresentFlag ) { |  |
| general_profile_space | u(2) |
| general_tier_flag | u(1) |
| general_profile_idc | u(5) |
| for( i = 0; i < 32; i++ ) |  |
| general_profile_compatibility_flag[ i ] | u(1) |
| general_reserved_zero_16bits [Ed. (GJS): Adjust semantics accordingly.] | u(16) |
| } |  |
| if( levelPresentFlag ) |  |
| general_level_idc | u(8) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) { |  |
| if( profilePresentFlag ) |  |
| sub_layer_profile_present_flag[ i ] | u(1) |
| sub_layer_level_present_flag[ i ] | u(1) |
| if( profilePresentFlag && sub_layer_profile_present_flag[ i ] ) { |  |
| sub_layer_profile_space[ i ] | u(2) |
| sub_layer_tier_flag[ i ] | u(1) |
| sub_layer_profile_idc[ i ] | u(5) |
| for( j = 0; j < 32; j++ ) |  |
| sub_layer_profile_compatibility_flag[ i ][ j ] | u(1) |
| sub_layer_reserved_zero_16bits[ i ] | u(16) |
| } |  |
| if(levelPresentFlag && sub_layer_level_present_flag[ i ] ) |  |
| sub_layer_level_idc[ i ] | u(8) |
| } |  |
| } |  |

## On VPS Extension Syntax structure

The currently defined vps_extension() in [1] uses vps_profile_tier syntax structure with profilePresentFlag always set to 1 for all layers (i=1 to vps_max_layers_minus1), as shown below. That is there are no restrictions for profile_tier or level syntax parameters in General cases.

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_codec_flag | u(1) |
| scalability_mask | u(16) |
| for( i = 0; i <NumScalabilityTypes; i++ ) { |  |
| dimension_id_len_minus1[ i ] | u(3) |
| } |  |
| vps_nuh_layer_id_present_flag | u(1) |
| // layer specific information |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| // mapping of layer ID to scalability dimension IDs |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| for( j = 0; j <= num_dimensions_minus1; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| // layer dependency |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ]; j++ ) |  |
| ref_layer_id[ i ][ j ] | u(6) |
| } |  |
| } |  |

In the following sections, this contribution proposes two ways of restricting such syntax parameters of profile_tier and/or levels while the modified profile_tier_level syntax structure is being used in vps_extension() with either layer-specific or operation-point (OP) specific updates.

### Layer-specific updates of profile_tier_level in vps_extension()

| vps_extension( ) { | Descriptor |
| --- | --- |
| ................ Edited .................................. |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| vps_level_present_flag[ i ] | u(1) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_level_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| if (!vps_profile_present_flag[ i ]) |  |
| vps_profile_ref_layer_num[ i ] | u(8) |
| else |  |
| vps_profile_ref_layer_num[ i ] = i |  |
| if (!vps_level_present_flag[ i ]) |  |
| vps_level_ref_layer_num[ i ] | u(8) |
| else |  |
| vps_level_ref_layer_num[ i ] = i |  |
| } |  |
| ................ Edited .................................. |  |
| } |  |

In the above syntax structure, four new syntax parameters are introduced.

Semantics

vps_profile_present_flag[ i ]: When this flag equal to 1 specifies the profile information for Layer i is present in the profile_tier_level( ) syntax structure;  When this flag equal to 0 specifies that profile information for Layer i is not present in the profile_tier_level( ) syntax structure.

vps_profile_ref_layer_num[ i ]: When vps_profile_present_flag[ i ] equals to 0, profile information for Layer i is inferred to be equal to the profile information of  the Layer  indicated by vps_profile_ref_layer_num[ i ], a Layer-number used as an index to a Reference Profile-Tier-Level Look-up Table storing such information of previously sent  Layers.

vps_level_present_flag[ i ]: When this flag equal to 1 specifies the level information for Layer i is present in the profile_tier_level( ) syntax structure;  When this flag equal to 0 specifies that level information for Layer i is not present in the profile_tier_level( ) syntax structure.

vps_level_ref_layer_num[ i ]: When vps_level_present_flag[ i ] equals to 0, level information for Layer i is inferred to be equal to the level information of  the Layer  indicated by vps_level_ref_layer_num[ i ], a Layer-number used as an index to a Reference Profile-Tier-Level Look-up Table storing such information of previously sent  Layers.

The combined effect of the values of vps_profile_present_flag and vps_level_present_flag is shown in the chart below, where such syntax parameter flags are present in the modified vps_extension() syntax structure shown above.

| vps_profile_present_flag | vps_level_present_flag | Available Syntax parameters |
| --- | --- | --- |
| 0 | 0 | (Reference – Profile/Tier, LevelIdc) |
| 0 | 1 | (Reference – Profile/Tier), LevelIdc |
| 1 | 0 | Profile/Tier, (Reference - LevelIdc) |
| 1 | 1 | Profile/Tier, LevelIdc |

As an example, in case of both "vps_profile_present_flag” and "vps_level_present_flag" being 0 (above chart), the available syntax parameters are "(Reference – Profile/Tier, LevelIdc)", and this means all such parameters are available from the stored or "Reference" look-up tables when such parameters are sent earlier.  Such reference parameters are already explained in the two syntax parameters of vps_profile_ref_layer_num  and vps_level_ref_layer_num. It may be noted that use of  such "vps_profile_present_flag” and "vps_level_present_flag" helps in avoiding unknown conditions.

### OP-specific updates of profile_tier_level in vps_extension()

| vps_extension( ) { | Descriptor |
| --- | --- |
| ................ Edited .................................. |  |
| for( i = 0; i <= vps_num_op_sets_minus1; i++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| vps_level_present_flag[ i ] | u(1) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_level_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| if (!vps_profile_present_flag[ i ]) |  |
| vps_profile_ref_op_num[ i ] | u(8) |
| else |  |
| vps_profile_ref_op_num[ i ] = i |  |
| if (!vps_level_present_flag[ i ]) |  |
| vps_level_ref_op_num[ i ] | u(8) |
| else |  |
| vps_level_ref_op_num[ i ] = i |  |
| } |  |
| ................ Edited .................................. |  |
| } |  |

It may be added that similar OP-specific profile_tier_level syntax structure update  is also mentioned in "Profile & level signaling for operation points" in [3]. As before in "Level-specific" case, the semant

In the above syntax structure, four new syntax parameters are introduced, similar to "Level-specific update" case in the previous section.

Semantics

vps_profile_present_flag[ i ]: When this flag equal to 1 specifies the profile information for Operation point i (or OP i) is present in the profile_tier_level( ) syntax structure;  When this flag equal to 0 specifies that profile information for OP i is not present in the profile_tier_level( ) syntax structure.

vps_profile_ref_op_num[ i ]: When vps_profile_present_flag[ i ] equals to 0, profile information for OP i is inferred to be equal to the profile information of  the OP  indicated by vps_profile_ref_op_num[ i ], a OP-number used as an index to a Reference Profile-Tier-Level Look-up Table storing such information of previously sent  OPs.

vps_level_present_flag[ i ]: When this flag equal to 1 specifies the level information for OP i is present in the profile_tier_level( ) syntax structure;  When this flag equal to 0 specifies that level information for OP i is not present in the profile_tier_level( ) syntax structure.

vps_level_ref_op_num[ i ]: When vps_level_present_flag[ i ] equals to 0, level information for OP i is inferred to be equal to the level information of  the OP  indicated by vps_level_ref_op_num[ i ], a OP-number used as an index to a Reference Profile-Tier-Level Look-up Table storing such information of previously sent  OPs .

## Other updates for VPS and SPS

If this new flag of "LevelPresentFlag" is accepted for the "profile_tier_level" syntax structure, then it needs to be initialized in VPS and SPS as follows:

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ..... edited ....... |  |
| profile_tier_level( 1, 1, vps_max_sub_layers_minus1 ) |  |
| ..... edited ....... |  |
| } |  |

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ..... edited ....... |  |
| if (nuh_layer_id == 0) |  |
| profile_tier_level( 1, 1, sps_max_sub_layers_minus1 ) |  |
| ..... edited ....... |  |
| } |  |

# Conclusion

In this contribution, an update for the profile_tier_level syntax structure is made to support additional flexibilities in selecting profile_tier and level parameters for HEVC extensions in scalable video coding, multi-view coding and 3D video coding areas. After this update, this modified  syntax structure is used in vps_extension() syntax structure where the profile_tier and level syntax parameters can be restricted in two possible ways, either for each layer or for each operating point. It is believed that any one of these two ways is suitable for an adoption in vps_extension.

# Patent rights declaration(s)

Sony Electronics Inc. / Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- J. Boyce and Y-K. Wang, “NAL unit header and parameter set designs for HEVC extensions”, JCTVC-K1007, Shanghai, China, October 2012.

- B. Bross, et. al., “High Efficiency Video Coding (HEVC) text specification draft 9,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-K1003_v13, Shanghai, China, October 2012.

- J. Boyce, “VPS syntax for scalable and 3D extensions”, JCTVC-K0204, Shanghai, China, October 2012.