| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0229 |
| --- | --- |

| Title: | Inter-layer texture prediction for bit-depth and chroma format scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Texas Instruments Inc., USA Madhukar Budagavi Texas Instruments Inc., USA Minhua Zhou Texas Instruments Inc., USA | Tel: Email: Tel: Email: Tel: Email: | +1-214-480-1479 d-kwon@ti.com +1-214-480-1861 madhukar@ti.com +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

Since the encoding of high bit-depth video is supported in HEVC version 1 and the encoding of high chroma resolution video is currently studied in AhG7, it is worthwhile to study bit-depth and chroma format scalable coding in SHVC. In this contribution, as a starting point, the CU-level inter-layer texture prediction method is evaluated for bit-depth scalable video coding. The bit-depth scalable coding (e.g. 8-bit YUV4:2:0 1080p BL – 10-bit YUV4:2:0 1080p EL) and the combined bit-depth and spatial scalable coding (e.g. 8-bit YUV4:2:0 720p BL – 10-bit YUV4:2:0 1080p EL) are tested using the SMuC software and compared with simulcast. The CU-level inter-layer texture prediction is also evaluated for chroma format scalable video coding. For this purpose, it is implemented in the HM-8.0 AhG7 software. Then, the combined chorma format and bit-depth scalable coding (e.g. 8-bit YUV4:2:0 1080p BL – 10-bit YUV4:2:2 1080p EL) and the combined chorma format, bit-depth and scalable coding (e.g. 8-bit YUV4:2:0 720p BL – 10-bit YUV4:2:2 1080p EL) are compared with simulcast. Experimental results reportedly show that the CU-level inter-layer texture prediction result in significant BL+EL BD-rate gain for bit-depth and chroma scalability when compared with simulcast. Specifically, using the SMuC software, it results in the gain of AI - (Y: 46.2%, Cb: 47.3%, Cr: 47.3%), RA - (Y: 41.0%, Cb: 40.4%, Cr: 39.9%), LD-P - (Y: 37.2%, Cb: 37.7%, Cr: 37.2%) and AI - (Y: 35.2%, Cb: 35.5%, Cr: 35.7%), RA - (Y: 26.7%, Cb: 20.3%, Cr: 19.8%), LD-P - (Y: 19.8%, Cb: 15.3%, Cr: 14.5%) for bit-depth scalability and bit-depth + spatial scalability, respectively. And, using the HM-8.0 AhG7 software, it result in the gain of AI - (Y: 37.3%, Cb: 41.1%, Cr: 40.3%), RA - (Y: 28.5%, Cb: 33.4%, Cr: 30.2%), LD-P - (Y: 17.3%, Cb: 28.5%, Cr: 24.8%) and AI - (Y: 29.3%, Cb: 32.5%, Cr: 32.2%), RA - (Y: 20.1%, Cb: 21.8%, Cr: 19.6%), LD-P - (Y: 12.1%, Cb: 17.2%, Cr: 14.9%) for chroma format + bit-depth scalability and chroma format + bit-depth + spatial scalability, respectively.

# Introduction

The CU-level inter-layer texture prediction (ILTP) in the multi-loop scalable extension of HEVC has turned out very effective for spatial and SNR scalable coding with 8-bit YUV 4:2:0 video in that it provides significant coding gain with marginal impact to the single-layer HEVC codec architecture [1]. Since the encoding of high bit-depth video is supported in HEVC version 1, the bit-depth scalable coding also needs to be studied along with SNR and spatial scalable coding. And since the encoding of high chroma resolution video is being studied in AhG7, it is worthwhile to start investigating chroma scalable coding for early support. In this contribution, the same CU-level ILTP is evaluated for bit-depth and chroma format scalable coding as a starting point toward this end.

The CU-level ILTP is evaluated first for bit-depth scalable coding. The SMuC software is slightly modified to support bit-depth scalable coding and its performance for the bit-depth scalable coding and the combined bit-depth and spatial scalable coding is compared with simulcast. For chroma scalable coding, the CU-level ILTP is implemented in the HM-8.0 AhG7 software. Then its performance for the combined chroma format and bit-depth scalable coding and the combined chroma format, bit-depth and spatial scalable coding is compared with simulcast.

# Inter-layer texture prediction

In [1], the CU-level inter-layer texture prediction is proposed for SNR and spatial scalable coding so that each CU can be predicted from the collocated reference-layer reconstruction. As shown in Table 1, it requires marginal change of the coding unit syntax as well as marginal impact to the single-layer HEVC architecture, but provides significant coding gain over simulcast. For this reason, it has been adopted along with DCT-IF up-sampling filter [2] in the SMuC software for initial tool experiments.

Table 1. Coding unit syntax for enhancement layer

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| } |  |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, log2CbSize ) |  |
| else { |  |
| intra_bl[x0][y0] | ae(v) |
| if (!intra_bl [x0][y0]) |  |
| { |  |
| nCbS = ( 1 << log2CbSize ) |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) |  |
| part_mode | ae(v) |
| …… |  |
| …… |  |
| } |  |
| if( !pcm_flag ) { |  |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0])) |  |
| no_residual_syntax_flag | ae(v) |
| …… |  |
| } |  |
| } |  |
| } |  |

In [2], the following DCT-IF filters are proposed for the up-sampling of reference-layer reconstruction with accurate phases.

Table 2. Luma up-sampling filter

| Phase shift | Luma up-sampling filter | Scaling factor |
| --- | --- | --- |
| 0 | { 0, 0, 0, 64, 0, 0, 0, 0} / 64 | 2x, 1.5x |
| 1/3 | {-1, 4, -11, 52, 26, -8, 3, -1} / 64 | 1.5x |
| 1/2 | {-1, 4, -11, 40, 40, -11, 4, -1} / 64 | 2x |
| 2/3 | {-1, 3, -8, 26, 52, -11, 4, -1} / 64 | 1.5x |

Table 3. Chroma up-sampling filter

| Phase shift | Chroma up-sampling filter | Horizontal, Vertical Scaling factor |
| --- | --- | --- |
| 0 | {0, 64, 0, 0}/64 | 2x H, 1.5x H |
| 1/4 | {-4, 54, 16, -2}/64 | 1.5x V |
| 1/3 | {-5, 50, 22, -3}/64 | 1.5x H |
| 3/8 | {-6, 46, 28, -4}/64 | 2x V |
| 1/2 | {-4, 36, 36, -4}/64 | 2x H |
| 7/12 | {-4, 30, 43, -5}/64 | 1.5x V |
| 2/3 | {-3, 22, 50, -5}/64 | 1.5x H |
| 7/8 | { -2, 10, 58, -2 }/64 | 2x V |
| 11/12 | {-1, 5, 62, -2}/64 | 1.5x V |

2.1 Test video sequences

The input test sequences in this contribution are generated from 1080p 10-bit YUV4:2:2 sequences, which are available on TNT FTP site. These sequences are listed in Table 4.

Table 4. 1080p 10-bit YUV4:2:2 test sequences

Sequence name

Kimono1_1920x1080_24_10bit_422.yuv

ParkScene_1920x1080_24_10bit_422.yuv

BirdsInCage_1920x1080_60_10bit_422.yuv

DucksAndLegs_1920x1080_60_10bit_422.yuv

CrowdRun_1920x1080_50_10bit_422.yuv

OldTownCross_1920x1080_50_10bit_422.yuv

Seeking_1920x1080_50_10bit_422.yuv

From each of them, the following sequences are generated sequentially as follows.

1. 1080p 10-bit YUV4:2:0

From each of 1080p 10-bit YUV4:2:2, 1080p 10-bit YUV4:2:0 is generated by down-sampling chroma sample in vertical direction.

2. 1080p 8-bit YUV4:2:0

From each of 1080p 10-bit YUV4:2:0, 1080p 8-bit YUV4:2:0 is generated simply by bit shifting.

3. 720p 8-bit YUV4:2:0

From each of 1080p 8-bit YUV4:2:0, 720p 8-bit YUV4:2:0 is generated using the down-sampling filter proposed in [3].

2.2 Inter-layer texture prediction for bit-depth scalable coding

To evaluate the inter-layer texture prediction for bit-depth scalable coding, the SMuC software has been modified especially for the up-sampling filter to support bit-depth scalability. Since the 8-bit input sequence is generated by right bit shifting from 10-bit sequence, each sample is left shifted by 2 bits to increase bit-depth of reference-layer reconstruction. If there is spatial resolution change between layers, spatial up-sampling is done after bit shifting.

2.3 Inter-layer texture prediction for chroma format scalable coding

The inter-layer texture prediction and DCT-IF up-sampling filter in the SMuC software have been implemented in the HM-8.0 AhG7 software. When there are changes in bit-depth, chroma format and spatial resolution between layers, the up-sampling of the reference-layer reconstruction is done by the following step:

1. Bit-depth scaling (tone mapping)

Each sample is left shifted by 2 if there is bit-depth change. 2 is the bit-depth difference between layers.

2. Adjust vertical and horizontal scaling factors for chroma

Let’s denote SY,H and SY,V are luma horizontal and vertical scaling factors, respectively, and SUV,H and SUV,V are chroma horizontal and vertical scaling factors, respectively. SY,H and SY,V are one of 1, 1.5 and 2, which are scaling factors considered. Therefore, SUV,H and SUV,V can be one of 1, 1.5, 2, 3 and 4 depending on the chroma format change between layers.

3. Spatial up-sampling

The reference-layer reconstruction is up-sampled using the scaling factors, SY,H, SY,V, SUV,H and SUV,V. The exact phases for 3x and 4x horizontal chroma up-sampling are 0, 1/3, 2/3, 1/4, 1/2 and 3/4. The exact phases for 4x vertical chroma up-sampling are 0, 1/4, ½ and 3/4. All these phases are included in Table 3. However, Table 3 does not include the chroma phases for 3x vertical chroma up-sampling, i.e. SUV,V = 3. For this, we need 3 more filters for the chroma phases of 1/12, 5/12 and 3/4. The filter coefficients for them are shown in Table 5.

Table 5. Additional chroma up-sampling filter for 3x chroma vertical up-sampling

| Phase shift | Chroma up-sampling filter |
| --- | --- |
| 1/12 | {-2, 62, 5, -1}/64 |
| 5/12 | {-4, 43, 30, -4}/64 |
| 3/4 | {-2, 16, 54, -4}/64 |

# Experimental results

# The inter-layer texture prediction is evaluated using the sequences in Section 2.1 for 4 different types of scalability shown in Table 6.

Table 6. Type of scalability for experiments

| Type of scalability | BL | EL | Software |
| --- | --- | --- | --- |
| Bit-depth | 1080p 8-bit YUV4:2:0 | 1080p 10-bit YUV4:2:0 | SMuC |
| Bit-depth + 1.5x | 720p 8-bit YUV4:2:0 | 1080p 10-bit YUV4:2:0 | SMuC |
| Chroma + Bit-depth | 1080p 8-bit YUV4:2:0 | 1080p 10-bit YUV4:2:2 | HM-8.0 AHG7 |
| Chroma + Bit-depth + 1.5x | 720p 8-bit YUV4:2:0 | 1080p 10-bit YUV4:2:2 | HM-8.0 AHG7 |

For all experiments, AI, RA and LD-P configurations are used and the scalable coding using the inter-layer texture prediction is compared with simulcast anchor. BL QP is set to 16 and 30, and delta QP for EL is set to -2, 0, 2 and 4 for each BL QP. Tables 7 to 10 shows the BD-rate results for different types of scalability compared to simulcast.

# Table 7. BD-rate results for bit-depth scalability compared to simulcast anchor

|  | AI HEVC BitDepth |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Overall (EL+BL) | -46.2% | -47.3% | -47.3% |
| Overhead over Single Layer | 4.6% | 4.0% | 4.1% |
| Overall (EL) | -93.2% | -96.1% | -96.2% |
| Enc Time[%] | 105.7% |  |  |
| Dec Time[%] | 104.0% |  |  |
| BL Match | Matched |  |  |
|  | RA HEVC BitDepth |  |  |
|  | Y | U | V |
| Overall (EL+BL) | -41.0% | -40.4% | -39.9% |
| Overhead over Single Layer | 12.9% | 18.9% | 20.5% |
| Overall (EL) | -83.0% | -84.8% | -84.7% |
| Enc Time[%] | 115.6% |  |  |
| Dec Time[%] | 136.1% |  |  |
| BL Match | Matched |  |  |
|  | LD-P HEVC BitDepth |  |  |
|  | Y | U | V |
| Overall (EL+BL) | -37.2% | -37.7% | -37.2% |
| Overhead over Single Layer | 18.6% | 22.4% | 24.5% |
| Overall (EL) | -76.0% | -79.8% | -79.4% |
| Enc Time[%] | 117.1% |  |  |
| Dec Time[%] | 143.8% |  |  |
| BL Match | Matched |  |  |

# Table 8. BD-rate results for bit-depth + 1.5x spatial scalability compared to simulcast anchor

|  | AI HEVC BitDepth + 1,5x |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Overall (EL+BL) | -35.2% | -35.5% | -35.7% |
| Overhead over Single Layer | 7.1% | 6.3% | 5.9% |
| Overall (EL) | -62.8% | -62.7% | -63.1% |
| Enc Time[%] | 109.0% |  |  |
| Dec Time[%] | 104.4% |  |  |
| BL Match | Matched |  |  |
|  | RA HEVC BitDepth + 1.5x |  |  |
|  | Y | U | V |
| Overall (EL+BL) | -26.7% | -20.3% | -19.8% |
| Overhead over Single Layer | 20.2% | 31.4% | 32.3% |
| Overall (EL) | -48.5% | -36.5% | -35.6% |
| Enc Time[%] | 109.3% |  |  |
| Dec Time[%] | 132.4% |  |  |
| BL Match | Matched |  |  |
|  | LD-P HEVC BitDepth + 1.5x |  |  |
|  | Y | U | V |
| Overall (EL+BL) | -19.8% | -15.3% | -14.5% |
| Overhead over Single Layer | 30.7% | 37.6% | 39.0% |
| Overall (EL) | -37.2% | -28.0% | -26.6% |
| Enc Time[%] | 109.1% |  |  |
| Dec Time[%] | 142.2% |  |  |
| BL Match | Matched |  |  |

# Table 9. BD-rate results for chroma + bit-depth scalability compared to simulcast anchor

|  | AI HEVC Chroma + BitDepth |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Overall (EL+BL) | -37.3% | -41.1% | -40.3% |
| Overhead over Single Layer | 9.9% | 12.7% | 16.4% |
| Overall (EL) | -69.3% | -80.9% | -81.0% |
| Enc Time[%] | 101.0% |  |  |
| Dec Time[%] | 86.9% |  |  |
| BL Match | Matched |  |  |
|  | RA HEVC Chroma + BitDepth |  |  |
|  | Y | U | V |
| Overall (EL+BL) | -28.5% | -33.1% | -29.9% |
| Overhead over Single Layer | 25.4% | 32.9% | 42.6% |
| Overall (EL) | -54.7% | -68.3% | -63.9% |
| Enc Time[%] | 92.2% |  |  |
| Dec Time[%] | 103.3% |  |  |
| BL Match | Matched |  |  |
|  | LD-P HEVC Chroma + BitDepth |  |  |
|  | Y | U | V |
| Overall (EL+BL) | -17.3% | -28.5% | -24.8% |
| Overhead over Single Layer | 45.3% | 43.5% | 55.9% |
| Overall (EL) | -34.8% | -59.0% | -54.0% |
| Enc Time[%] | 94.2% |  |  |
| Dec Time[%] | 108.0% |  |  |
| BL Match | Matched |  |  |

# Table 10. BD-rate results for chroma + bit-depth + 1.5x spatial scalability compared to simulcast anchor

|  | AI HEVC Chroma + BitDepth + 1.5x |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Overall (EL+BL) | -29.3% | -32.5% | -32.2% |
| Overhead over Single Layer | 8.7% | 2.0% | 3.6% |
| Overall (EL) | -47.8% | -51.6% | -52.0% |
| Enc Time[%] | 107.8% |  |  |
| Dec Time[%] | 88.1% |  |  |
| BL Match | Matched |  |  |
|  | RA HEVC Chroma + BitDepth + 1.5x |  |  |
|  | Y | U | V |
| Overall (EL+BL) | -20.1% | -21.8% | -19.6% |
| Overhead over Single Layer | 23.8% | 19.6% | 23.8% |
| Overall (EL) | -34.1% | -35.1% | -31.9% |
| Enc Time[%] | 91.5% |  |  |
| Dec Time[%] | 102.4% |  |  |
| BL Match | Matched |  |  |
|  | LD-P HEVC Chroma + BitDepth + 1.5x |  |  |
|  | Y | U | V |
| Overall (EL+BL) | -12.1% | -17.2% | -14.9% |
| Overhead over Single Layer | 37.2% | 26.8% | 31.3% |
| Overall (EL) | -21.2% | -28.0% | -25.0% |
| Enc Time[%] | 93.5% |  |  |
| Dec Time[%] | 106.5% |  |  |
| BL Match | Matched |  |  |

# Conclusion

In this contribution, the inter-layer texture prediction is evaluated for bit-depth and chroma format scalability and it was shown that significant coding gain can be achievable even for the combined scalability of bit-depth, chroma format and spatial resolution. The bit-depth and chroma format scalable coding is expected to be improved further by other coding tools such as other tone-mapping process, etc. Therefore, it is recommend to investigate them in CE along with the current SHVC study.

# Reference

- D.-K. Kwon, M. Budagavi and M. Zhou, “Description of scalable video coding technology proposal by Texas Instruments Inc.”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-K0038, Oct. 2012.

- L. Alshina, H. Lakshman, J. Dong, J. Chen and A. Luthra, “Suggested up-sampling filter design for tool experiments on HEVC scalable extension,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-K0378, Oct. 2012.

- J. Dong, Y. He and Y. Ye, “Downsampling filters for anchor generation for scalable extensions of HEVC,” m24499, 100th MPEG meeting, Geneva, Switzerland, Apr. 2012.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).