| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0230 |
| --- | --- |

| Title: | Non-TE2: Improved inter-layer texture prediction signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Texas Instruments Inc., USA Madhukar Budagavi Texas Instruments Inc., USA Minhua Zhou Texas Instruments Inc., USA | Tel: Email: Tel: Email: Tel: Email: | +1-214-480-1479 d-kwon@ti.com +1-214-480-1861 madhukar@ti.com +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

In this contribution, the inter-layer texture prediction (IntraBL) signaling in the current SMuC software is modified to reportedly improve coding efficiency. First, it is proposed to signal whether IntraBL CU has coded coefficients using two existing flags for Intra (I) and Inter (P and B) slices, respectively. In Intra slice, it is allowed to signal no_residual_syntax_flag for IntraBL CU when it has no coded coefficients. In Inter slice, it is allowed to signal skip_flag for IntraBL CU, which means that skip_flag can be set to 1 either by the merge process or the inter-layer texture prediction. Second, in Inter slices, it is proposed to modify the merge process so as to support inter-layer texture prediction for PU smaller than 2Nx2N. IntraBL mode is added in the merge candidate list if the number of spatial neighboring candidates is less than 4. Inter-layer texture prediction gets enabled when the signaled merge_index specifies IntraBL mode. Experimental results with common test condition reportedly show that, when compared to the SMuC software (SMuC_0.1.1bf), the proposed methods result in luma BL+EL BD-rate gain of 0.2%, 0.3%, 0.4%, 0.4%, 0.2%, 0.7%, 0.9% and 0.5% for AI 2x, AI 1.5x, RA 2x, RA 1.5x, RA SNR, LD-P 2x, LD-P 1.5x and LD-P SNR, respectively.

# Introduction

In the current SMuC software, the inter-layer texture prediction is signaled in CU level using IntraBL flag. Even though it provides significant coding gain for enhancement layer, further improvements are possible due to the following reasons: Skipped IntraBL CU is not signaled separately and the inter-layer texture prediction is not allowed for PU smaller than 2Nx2N. Based on these observations, in this contribution, the SMuC inter-layer texture prediction signaling scheme is modified for better coding efficiency.

Whether IntraBL CU has coded coefficients is signaled using the existing flags. For I slice, no_residual_syntax_flag is allowed to be signaled for IntraBL CU as well. For P and B slice, skip_flag is signaled also for IntraBL CU. IntraBL flag is signaled always regardless of skip_flag, and when skip_flag is equal to 1, merge_index is signaled only if IntraBL is equal to 0.

For PU smaller than 2Nx2N, the inter-layer texture prediction is supported by modifying the merge process. Inter-layer texture prediction mode is added in the merge candidate list if the number of spatial neighboring candidates is less than 4. By doing so, the inter-layer texture prediction for PU smaller than 2Nx2N can be signaled by merge_index without an additional flag.

# Proposed methods

2.1 Skipped IntraBL CU signaling

When a CU is predicted from the collocated reconstruction of reference layer, there could be no coded residual signal because of the high correlation between them. Therefore, it is efficient to signal whether an IntraBL CU has coded coefficients or not. In the proposed method, skipped IntraBL CU is signaled by the existing flags as shown in Table 1, where IntraBL flag is denoted by intra_bl.

In I slice, intra_bl is signaled first. When intra_bl is equal to 1, PredMode and PartMode is set to INTRA and PART_2Nx2N. Otherwise, PredMode and PartMode need to be signaled. To indicate whether IntraBL CU has coded coefficients, it is proposed to signal no_residual_syntax_flag for Intra BL CU in I slice.

In P and B slice, skip_flag is followed by intra_bl always in the proposed method, while intra_bl is signaled only when skip_flag is equal to 0 in the SMuC software. It indicates that a CU can be skipped by either the merge process or the inter-layer texture prediction. Therefore, when skip_flag is equal to 1, merge_index is signaled only if intra_bl is equal to 0.

Table 1. Proposed coding unit syntax for enhancement layer

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| } |  |
| if( slice_type != I ) |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| intra_bl[x0][y0] | ae(v) |
| if( skip_flag[ x0 ][ y0 ] ) |  |
| if (!intra_bl[x0][y0]) |  |
| prediction_unit( x0, y0, log2CbSize ) |  |
| else { |  |
| if (!intra_bl [x0][y0]) |  |
| { |  |
| nCbS = ( 1 << log2CbSize ) |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( PredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = Log2MinCbSize ) |  |
| part_mode | ae(v) |
| …… |  |
| …… |  |
| } |  |
| if( !pcm_flag ) { |  |
| if( (PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0]) ) ¦¦ (intra_bl[x0][y0] && slice_type == I)) |  |
| no_residual_syntax_flag | ae(v) |
| if( !no_residual_syntax_flag ) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

2.2 Inter-layer texture prediction for PU using the modified merge process

The benefit of inter-layer texture prediction is not fully exploited by the CU-level IntraBL flag since it enables the inter-layer texture prediction only for 2Nx2N prediction block. In some regions of picture, the inter-layer texture prediction for non-square PU or NxN PU can provides additional coding gain especially for Inter slice. However, signaling IntraBL flag for every PU may not be efficient considering that it provides marginal gain on top of CU-level IntraBL flag. Therefore, in the proposed method, the inter-layer texture prediction for PU smaller than 2Nx2N is supported by modifying the merge process in Inter slice.

After the derivation process for spatial merging candidate, if numMergeCand is less than 4, the inter-layer texture prediction mode is added into the merge candidate list before the temporal merging candidate. For this mode, refIdxLx is set to -1, predFlagLx is set to 0 and mvLx is set to (0, 0) for x = 0 and 1. When the derived refIdxLx for x = 0 and 1 by merge_idx are -1, a PU is predicted from the collocated reference-layer reconstruction. By the proposed method, the inter-layer texture prediction can be enabled in PU level without introducing an additional flag.

Figure 1. Modified derivation process of merge candidate list

Figure 2. Modified PU decoding process by merge_idx

# Experimental results

The proposed methods are compared with HM-8,1 Simulcast under the TE-A2 common test conditions [1]. It was shown that the proposed methods resulted in the BL+EL BD-rate gain of (Y: 23.1%, Cb: 22.8%, Cr: 22.9%), (32.9%, 32.7%, 33.0%), (16.8%, 8.5%, 7.9%), (26.5%, 19.8%, 18.3%), (20.3%, 10.6%, 8.2%), (12.9%, 5.2%, 4.2%), (22.6%, 16.6%, 14.8%) and (13.6%, 5.4%, 2.2%) for AI 2x, AI 1.5x, RA 2x, RA 1.5x, RA SNR, LD-P 2x, LD-P 1.5x and LD-P SNR, respectively. It is summarized in Table 2.

Table 2. BD-rate result of the proposed method compare to HM-8.1 Simulcast anchor.

# The proposed methods are also compared with the SMuC software (SMuC_0.1.1bf). As shown in Table 3, the proposed methods resulted in luma BL + EL BD-rate gain of 0.2%, 0.3%, 0.4%, 0.4%, 0.2%, 0.7%, 0.9% and 0.5% for AI 2x, AI 1.5x, RA 2x, RA 1.5x, RA SNR, LD-P 2x, LD-P 1.5x and LD-P SNR, respectively.

Table 3. BD-rate result of the proposed method compared to SMuC_0.1.1_bf

# Conclusion

In this contribution, the inter-layer texture prediction signaling is improved by a couple of methods. The IntraBL CU that has no coded coefficients is signaled using two existing flags, no_residual_syntax_flag and skip_flag for Intra slice and Inter slice, respectively. Moreover, the inter-layer texture prediction for smaller PU is enabled by the modified merge process in Inter slice. The experimental shows that when compare to Simulcast, the proposed methods result in BL+EL BD-rate gain of (Y: 23.1%, Cb: 22.8%, Cr: 22.9%), (32.9%, 32.7%, 33.0%), (16.8%, 8.5%, 7.9%), (26.5%, 19.8%, 18.3%), (20.3%, 10.6%, 8.2%), (12.9%, 5.2%, 4.2%), (22.6%, 16.6%, 14.8%) and (13.6%, 5.4%, 2.2%) for AI 2x, AI 1.5x, RA 2x, RA 1.5x, RA SNR, LD-P 2x, LD-P 1.5x and LD-P SNR, respectively.

# Reference

- L. Wei, Y. He, D.-K. Kwon, J. Zan, H. Lakshman and J. W. Kang, “Description of Tool Experiment A2: Inter-layer texture prediction signaling in SHVC,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-K1102, Oct. 2012.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).