| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0231 |
| --- | --- |

| Title: | Inter-layer slice header syntax element prediction in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Texas Instruments Inc., USA Madhukar Budagavi Texas Instruments Inc., USA Minhua Zhou Texas Instruments Inc., USA | Tel: Email: Tel: Email: Tel: Email: | +1-214-480-1479 d-kwon@ti.com +1-214-480-1861 madhukar@ti.com +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

In many scalable video coding applications, all spatial and SNR layers are coded using the same GOP structure. It is usual for all layer pictures at the same temporal position to have the same slice type and the same temporal references. Therefore, in this contribution, it is proposed to signal inter_layer_rps_prediction_flag in enhancement-layer slice header (SH) so that reference picture set (RPS) and reference picture list modification syntax elements can be inferred from those of a collocated reference-layer SH. When the pictures at the same temporal position are coded using the identical reference picture lists, their weighted prediction (WP) parameters are likely to be identical to each other. Therefore, it is also proposed to signal inter_layer_wp_prediction_flag so that the WP parameters for all reference indices can be inferred from those of a collocated reference-layer SH. It is asserted that the slice headers of enhancement layer can be signaled more efficiently with the proposed flags.

# Introduction

Scalable video encoder usually encodes all spatial and SNR layers into the same GOP structure as shown in Figure 1, unless a specific constraint is imposed on each layer. It is common for all layer pictures at the same temporal position to have the same slice type, the same temporal reference pictures, etc. Moreover, when the picture at the same temporal position are coded using the identical reference picture lists, their WP parameters for the same reference indices are likely to be identical. Therefore many of slice header (SH) syntax elements of enhancement layer can be inferred from those of a reference layer.

In this contribution, it is proposed to signal a couple of flags in the enhancement-layer SH so as to remove redundant signaling of RPS, reference picture list modification syntax and WP parameters, which requires relatively large number of bits when signaled.

Figure 1. Example of two-layer scalable encoding. Usually the pictures at the same temporal position have the same slice type and the same reference pictures. Note that arrows (reference pictures) are shown only for some of pictures for simplicity.

# Proposed syntax elements

2.1 Inter-layer prediction of RPS and reference picture list modification

It is proposed to signal inter_layer_rps_prediction_flag in the slice headers of enhancement layer as shown in Table 1. When it is set to 1, RPS and reference picture list modification syntax elements are inferred from those of a collocated reference-layer SH, which is defined as

collocated reference-layer slice header is the slice header of the reference layer slice that includes the collocated CU of the first LCU of the enhancement-layer slice.

Table 1. Proposed signaling of RPS and reference picture list modification in enhancement layer

| slice_header( ) { | Descriptor |
| --- | --- |
| …… |  |
| if( !IdrPicFlag ) { |  |
| pic_order_cnt_lsb | u(v) |
| inter_layer_rps_prediction_flag | u(1) |
| if (!inter_layer_rps_prediction_flag) |  |
| { |  |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| Else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| if( num_long_term_ref_pics_sps > 0 ) |  |
| num_long_term_sps | ue(v) |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ ) { |  |
| if( i < num_long_term_sps ) |  |
| lt_idx_sps[ i ] | u(v) |
| else { |  |
| poc_lsb_lt[ i ] | u(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| …… |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| if( sps_temporal_mvp_enable_flag ) |  |
| slice_temporal_mvp_enable_flag | u(1) |
| if (!inter_layer_rps_prediction_flag) |  |
| { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| if( lists_modification_present_flag ) |  |
| ref_pic_list_modification( ) |  |
| } |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| …… |  |
| } |  |
| …… |  |
| } |  |

inter_layer_rps_prediction_flag equal to 1 specifies that RPS and reference picture list modification syntax elements are inferred from those of a reference layer, which is signaled by reference_layer_id.

2.1 Inter-layer prediction of WP parameters

It is proposed to signal inter_layer_wp_prediction_flag in the slice headers of enhancement layer as shown in Table2. When it is set equal to 1, WP parameters for all reference indices are inferred from those of a collocated reference-layer SH. Otherwise, WP parameters, i.e. pred_weight_table(), are signaled.

Table 2. Proposed signaling of WP parameters in enhancement layer

| slice_header( ) { | Descriptor |
| --- | --- |
| …… |  |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| { |  |
| inter_layer_wp_prediction_flag | u(1) |
| if (!inter_layer_wp_prediction_flag) |  |
| pred_weight_table( ) |  |
| } |  |
| …… |  |
| } |  |

inter_layer_wp_prediction_flag equal to 1 specifies that WP parameter syntax elements are inferred from those of collocated slice header in a reference layer, which is signaled by reference_layer_id.

# Conclusion

Two flags, inter_layer_rps_prediction_flag and inter_layer_wp_prediction_flag are proposed for the enhancement-layer slice header When inter_layer_rps_prediction_flag is set equal to 1, RPS and reference picture list modification syntax elements are directly inferred from those of a collocated reference-layer slice header. When inter_layer_wp_prediction_flag is set equal to 1, WP parameters are inferred from those of a collocated reference-layer slice header. The proposed flags are useful in reducing overhead signaling especially when multiple layers are coded into the same GOP structure, which is common in many applications.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).