| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0232_r6 |
| --- | --- |

| Title: | AHG6: On deblocking filter and parameters signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Andrey Norkin, Kenneth Andersson, Rickard Sjöberg Färögatan 6 16480 Stockholm | Tel: Email: | +46 10 714 84 02 andrey.norkin@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The proposal claims that tc offsets parameters, RD penalty of maximum transform size for intra CU’s in inter P and B slices together with modified strong deblocking filter significantly can reduce blocking artifacts in sequences with chaotic motion. The software is attached to the contribution.

# Problem Statement

Instances of blocking artifacts in HEVC have been reported for sequences with chaotic motion such as water. This contribution proposes approaches to reduce blocking artifacts in those sequences.

# Tc offset parameters

The non-normative encoding method of tc offset parameters [2] was adopted to HM at the Shanghai meeting. It was requested to determine appropriate deblocking parameters that improve the performance for those sequences.

The proposed deblocking parameters are listed below. First for low delay configurations:

#        Type POC QPoffset QPfactor tcOffsetDiv2 betaOffsetDiv2

Frame1:  B    1   3        0.4624   5            0

Frame2:  B    2   2        0.4624   3            0

Frame3:  B    3   3        0.4624   5            0

Frame4:  B    4   1        0.578    1            0

and for random access:

#        Type POC QPoffset QPfactor tcOffsetDiv2 betaOffsetDiv2

Frame1:  B    8   1        0.442    1            0

Frame2:  B    4   2        0.3536   3            0

Frame3:  B    2   3        0.3536   4            0

Frame4:  B    1   4        0.68     6            0

Frame5:  B    3   4        0.68     6            0

Frame6:  B    6   3        0.3536   4            0

Frame7:  B    5   4        0.68     6            0

Frame8:  B    7   4        0.68     6            0

# RD penalty on maximum TU size for intra CU’s in inter slices

The subjective quality improvement has been observed on some sequences (e.g. on Red kayak) when the maximum intra TU size is restricted to 16x16. To reduce the block artifacts that appear because of large intra blocks in inter-predicted slices, it is proposed to add to the next version of HM a possibility to restrict add a penalty on the maximum intra TU size in inter-predicted slices in the encoder. The feature was implemented in HM9.1 by using the following code.

#if COST_PENALTY_32X32_NEW

Int tuSize = pcCU->getSlice()->getSPS()->getQuadtreeTULog2MaxSize();

Int isIntraSlice = (pcCU->getSlice()->getSliceType() == I_SLICE);

Bool    bCheckFull2;

if(isIntraSlice)

bCheckFull2    = ( uiLog2TrSize  <= pcCU->getSlice()->getSPS()->getQuadtreeTULog2MaxSize() );

else

bCheckFull2 = ( uiLog2TrSize  <= min(tuSize,ENCODER_INTER_SLICE_INTRA_TU_LOG2_MAXSIZE) );

if( bCheckFirst && bCheckFull2 )

#else

if( bCheckFirst && bCheckFull )

#endif

{

bCheckSplit = false;

}

//----- determine rate and r-d cost -----

UInt uiSingleBits = xGetIntraBitsQT( pcCU, uiTrDepth, uiAbsPartIdx, true, !bLumaOnly, false );

#if COST_PENALTY_32X32_NEW

if((uiLog2TrSize==5) && !isIntraSlice)

{

uiSingleBits=uiSingleBits*4;

}

#endif

dSingleCost       = m_pcRdCost->calcRdCost( uiSingleBits, uiSingleDistY + uiSingleDistC );

# Modified strong deblocking filter

## Modified decisions for applying strong filter

Current strong filtering decisions do not allow application of the strong deblocking filter to the boundaries between the large intra-predicted blocks, where it should be applied in order to improve the subjective video quality. It is therefore proposed to modify the strong filtering decisions in order for strong deblocking filter to be applied to the block boundary between the large intra-blocks.

It is proposed to modify the decisions for application of the strong filtering as shown below. The following conditions are checked for strong filter based on lines i = 0, 3.

2*dpqi < (β>>2)			          (4)    Same as in HM9.0

(|p3i - p0i| + |q0i – q3i|) < (β>>2)      (5) // remove a condition that 4 samples from the block boundary have the same level

dp0 = ((| p3,i − 2*p2,i + p1,i | + | q3,i − 2*q2,i + q1,i |) <<1) < (β>>2)  	(5) // instead, add a condition that 4 pixels //from the block boundary form a straight line (flat or inclined),

|p0i – q0i| < ((5*tC + 1)>>1)  		      (6) // same as in HM9.0

The proposed decisions are explained in Figure 1. The proposed decisions enable applying strong filtering also when both sides of the block boundary have a form of ramp (similar to the normal filtering mode).

| (a) | (b) |
| --- | --- |

Figure 1 (a) Proposed decisions for strong filtering. Two checks are included: that pixel values q0, q1, q2 and q3 form a line (the same for p0, p1, p2 and p3) and that | p0 – q0 | is below threshold. (b) Block artifact from Riverbed. Filter decision matches the artifact.

## Modified strong filter

Since the proposed filtering decisions turn on the strong filtering in the areas where it is not turned on in the current HEVC deblocking (e.g. on the signal that has a form of ramp), it is also necessary to modify the strong filter. Since the current HEVC filter is designed to be applied to a signal that is flat on both sides of the block boundary, the HEVC strong filter would modify a ramp when applied to it. Therefore, it is proposed to modify the filter coefficients that correspond to pixels p1, q1, p2, q2 so that the filter would not modify a ramp.

The proposed modifications to the strong filter calculations are shown below.

p0’ = Clip3( p0−tc0, p0+tc0, ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4 ) >> 3 )   // same filter as in HM8.0
q0’ = Clip3( q0−tc0, q0+tc0, ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3 )   // same filter as in HM8.0

p1’ = Clip3( p1−2*tc, p1+2*tc, ( p2 + p1 + p0 + q0 + 2 ) >> 2 )
p2’ = Clip3( p2−2*tc, p2+2*tc, ( 2*p3 + 3*p2 + p1 + p0 + q0 + 4 ) >> 3 ) 
q1’ = Clip3( q1−2*tc, q1+2*tc, ( p0 + q0 + q1 + q2 + 2 ) >> 2 )
q2’= Clip3( q2−2*tc, q2+2*tc, ( p0 + q0 + q1 + 3*q2 + 2*q3 + 4 ) >> 3 )

p1' = Clip3(p1-tc1, p1+tc1, ((p3 + 2*p2+ 2*p1 + p0 + 2*q0 + 4)>>3))          
q1'   = Clip3(q1-tc1, q1+tc1, ((q3 + 2*q2+ 2*q1 +   q0+2*p0 + 4)>>3))
p2' = Clip3(p2-tc2, p2+tc2, ((3*p3 + 3*p2 + p1 + q0 + 4 )>>3))
q2' = Clip3(q2-tc2, q2+tc2, ((p0 +  q1 + 3*q2 + 3*q3 +4 )>>3))

The effect of applying HEVC strong filter and the proposed filter to a ramp is shown in Figure 2. One can see from the figure that HEVC filter significantly disports the ramp when applied to it, while the proposed filter modified the ramp much less.

| - Effect of HEVC strong filter applied to a ramp | - Effect of proposed strong filter applied to a ramp |
| --- | --- |

Figure 2 Comparison of modification of ramp by HEVC filer and the proposed filter. The original signal is shown in blue, the signal after modification by a filter is shown in red.  The HEVC filter (a) modifies the ramp stronger than the proposed filter (b).

The clipping has also been modified in order to avoid blurring of the reconstructed image by applying too strong filtering (since the strong filter is applied more often). Therefore, the clipping thresholds used in the equations above are reduced relative to the current strong filter in order to reduce amount of blurriness.

The following clipping values are used:  tc1 = ( (tc<<1)+tc+1 )>>2, tc0 = (tc1*3+1)>>1, tc2 = (tc1+1)>>1.

# Software description

The software is attached to the contribution (file JCTVC-L0232_HM9.1.patch). The software is attached in the form of a patch that should be be applied to HM9.1. The patch includes both the modifications to the configuration files and to the source code.

The proposed modifications are to be turned on with the help of the following macros:

#define STRONG_FILTER_AND_DECISIONS                 1

#define COST_PENALTY_32X32_NEW                          1

The first macro turns on the strong filter and strong filter decisions and the second macro turns on the penalty for 32x32 intra blocks in inter slices.

The configuration files are modified when applying the patch.

# Experimental results

The proposed approaches have been implemented based on HM9.1. The BD-rate of the proposed approaches versus HM9.0 has been obtained and is shown below. The encoding and decoding times were obtained on a heterogeneous computational cluster and are therefore not reliable.

## Results for adaptation of deblocking filter parameters

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 87% |  |  | 86% |  |  |
| Dec Time[%] | 82% |  |  | 84% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | 0.0% | -0.2% | 0.0% | 0.2% |
| Class B | -0.1% | 0.2% | 0.3% | -0.1% | 0.3% | 0.3% |
| Class C | 0.0% | 0.4% | 0.5% | -0.1% | 0.4% | 0.5% |
| Class D | 0.1% | 0.3% | 0.6% | 0.1% | 0.6% | 0.5% |
| Class E |  |  |  |  |  |  |
| Class F | 0.3% | 0.3% | 0.4% | 0.3% | 0.3% | 0.4% |
| Overall | 0.1% | 0.3% | 0.3% | 0.0% | 0.3% | 0.4% |
|  | 0.0% | 0.3% | 0.3% | 0.0% | 0.3% | 0.4% |
| Enc Time[%] | 89% |  |  | 89% |  |  |
| Dec Time[%] | 88% |  |  | 93% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | -0.1% | 0.2% | -0.1% |
| Class C | 0.0% | 0.3% | 0.2% | 0.0% | 0.2% | 0.3% |
| Class D | 0.2% | 0.1% | -0.4% | 0.2% | 0.0% | -0.3% |
| Class E | 0.1% | -0.1% | -0.5% | 0.2% | 0.3% | 0.1% |
| Class F | 0.5% | 0.0% | 1.0% | 0.5% | 0.2% | 0.0% |
| Overall | 0.1% | 0.0% | 0.1% | 0.1% | 0.2% | 0.0% |
|  | 0.1% | 0.0% | 0.1% | 0.1% | 0.2% | 0.0% |
| Enc Time[%] | 88% |  |  | 87% |  |  |
| Dec Time[%] | 90% |  |  | 96% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | 0.0% | 0.1% | -0.4% | 0.1% | 0.1% |
| Class C | -0.1% | 0.2% | 0.3% | -0.2% | 0.3% | 0.2% |
| Class D | 0.0% | 0.1% | 0.4% | 0.0% | 0.6% | 0.5% |
| Class E | -0.2% | 0.0% | -0.5% | -0.1% | 0.3% | 0.4% |
| Class F | 0.3% | 0.1% | 0.4% | 0.4% | 0.6% | 1.9% |
| Overall | -0.1% | 0.1% | 0.2% | -0.1% | 0.4% | 0.6% |
|  | -0.1% | 0.1% | 0.1% | -0.1% | 0.4% | 0.5% |
| Enc Time[%] | 89% |  |  | 89% |  |  |
| Dec Time[%] | 85% |  |  | 89% |  |  |

  - Results for proposed filter modification

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% |  |  | 102% |  |  |
| Dec Time[%] | 110% |  |  | 111% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | 0.0% | 0.0% | -0.2% | -0.2% |
| Class B | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class D | 0.0% | -0.3% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 95% |  |  | 106% |  |  |
| Dec Time[%] | 99% |  |  | 119% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.0% | 0.1% | 0.3% | 0.3% | -0.1% |
| Class C | 0.1% | 0.1% | 0.1% | 0.3% | 0.1% | 0.0% |
| Class D | 0.0% | -0.1% | 0.2% | 0.1% | -0.1% | 0.0% |
| Class E | 0.3% | 0.0% | -0.5% | 1.3% | 0.9% | 0.7% |
| Class F | 0.1% | -0.3% | 0.6% | 0.2% | -0.5% | 0.3% |
| Overall | 0.1% | -0.1% | 0.1% | 0.4% | 0.1% | 0.1% |
|  | 0.1% | -0.1% | 0.1% | 0.4% | 0.0% | 0.1% |
| Enc Time[%] | 105% |  |  | 96% |  |  |
| Dec Time[%] | 116% |  |  | 116% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.1% | 0.2% | -0.2% | -0.2% | 0.0% |
| Class C | 0.1% | 0.0% | 0.0% | 0.2% | 0.1% | 0.1% |
| Class D | 0.0% | 0.2% | -0.2% | 0.0% | 0.7% | -0.4% |
| Class E | -0.2% | -0.2% | 0.1% | 0.2% | 0.4% | 0.6% |
| Class F | -0.1% | -0.3% | -0.2% | 0.1% | -0.1% | 0.8% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.2% |
|  | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 105% |  |  | 106% |  |  |
| Dec Time[%] | 101% |  |  | 105% |  |  |

## Results for proposed filter modification and tc_offsets

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 105% |  |  | 108% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.2% |
| Class B | -0.1% | 0.3% | 0.3% | 0.1% | 0.3% | 0.3% |
| Class C | 0.0% | 0.4% | 0.5% | 0.0% | 0.4% | 0.4% |
| Class D | 0.1% | 0.3% | 0.7% | 0.1% | 0.6% | 0.5% |
| Class E |  |  |  |  |  |  |
| Class F | 0.2% | 0.2% | 0.3% | 0.2% | 0.3% | 0.3% |
| Overall | 0.0% | 0.2% | 0.3% | 0.1% | 0.3% | 0.3% |
|  | 0.0% | 0.3% | 0.3% | 0.1% | 0.3% | 0.4% |
| Enc Time[%] | 93% |  |  | 103% |  |  |
| Dec Time[%] | 95% |  |  | 116% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | 0.2% | 0.3% | 0.6% | 0.4% |
| Class C | 0.1% | 0.2% | 0.0% | 0.4% | 0.6% | 0.4% |
| Class D | 0.1% | 0.2% | 0.2% | 0.3% | 0.2% | 0.0% |
| Class E | 0.5% | 0.1% | -0.4% | 1.8% | 1.4% | 1.3% |
| Class F | 0.5% | 0.4% | 1.1% | 0.8% | 0.1% | 0.8% |
| Overall | 0.2% | 0.2% | 0.2% | 0.6% | 0.5% | 0.5% |
|  | 0.2% | 0.2% | 0.2% | 0.6% | 0.5% | 0.5% |
| Enc Time[%] | 104% |  |  | 90% |  |  |
| Dec Time[%] | 114% |  |  | 101% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.5% | 0.1% | 0.5% | -0.7% | 0.5% | -0.1% |
| Class C | -0.1% | 0.3% | 0.3% | 0.0% | 0.3% | 0.4% |
| Class D | 0.0% | 0.0% | 0.2% | 0.0% | 0.4% | 0.2% |
| Class E | -0.2% | -0.4% | -0.2% | 0.0% | 0.5% | 0.6% |
| Class F | 0.3% | 0.5% | 0.7% | 0.4% | 0.5% | 1.6% |
| Overall | -0.1% | 0.1% | 0.3% | -0.1% | 0.4% | 0.5% |
|  | -0.1% | 0.1% | 0.3% | -0.1% | 0.5% | 0.4% |
| Enc Time[%] | 105% |  |  | 106% |  |  |
| Dec Time[%] | 103% |  |  | 105% |  |  |

## Results for RD penalty on maximum TU size for intra CU’s in inter slices and deblocking parameters

|  | All Intra Main |  |  |  |  | All Intra HE10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U |  | V |  | Y |  | U |  | V |
| Class A (8bit) | 0,0% | 0,0% |  | 0,0% |  | 0,0% |  | 0,0% |  | 0,0% |
| Class B | 0,0% | 0,0% |  | 0,0% |  | 0,0% |  | 0,0% |  | 0,0% |
| Class C | 0,0% | 0,0% |  | 0,0% |  | 0,0% |  | 0,0% |  | 0,0% |
| Class D | 0,0% | 0,0% |  | 0,0% |  | 0,0% |  | 0,0% |  | 0,0% |
| Class E | 0,0% | 0,0% |  | 0,0% |  | 0,0% |  | 0,0% |  | 0,0% |
| Overall | 0,0% | 0,0% |  | 0,0% |  | 0,0% |  | 0,0% |  | 0,0% |
|  | 0,0% | 0,0% |  | 0,0% |  | 0,0% |  | 0,0% |  | 0,0% |
| Class F | 0,0% | 0,0% |  | 0,0% |  | 0,0% |  | 0,0% |  | 0,0% |
| Enc Time[%] | 100% |  |  |  |  | 100% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  | 100% |  |  |  |  |
|  | Random Access Main |  |  |  |  | Random Access HE10 |  |  |  |  |
|  | Y | U |  | V |  | Y |  | U |  | V |
| Class A (8bit) | -0,4% | 0,2% |  | 0,4% |  | -0,4% |  | 0,2% |  | 0,2% |
| Class B | 0,2% | 1,0% |  | 0,9% |  | 0,2% |  | 1,1% |  | 1,0% |
| Class C | 0,1% | 0,6% |  | 0,6% |  | 0,0% |  | 0,6% |  | 0,6% |
| Class D | 0,2% | 0,4% |  | 0,7% |  | 0,2% |  | 0,6% |  | 0,4% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0,1% | 0,6% |  | 0,7% |  | 0,1% |  | 0,7% |  | 0,6% |
|  | 0,1% | 0,6% |  | 0,7% |  | 0,1% |  | 0,7% |  | 0,7% |
| Class F | 0,5% | 0,8% |  | 0,8% |  | 0,5% |  | 0,7% |  | 0,6% |
| Enc Time[%] | 99% |  |  |  |  | 101% |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  | 104% |  |  |  |  |
|  | Low delay B Main |  |  |  |  | Low delay B HE10 |  |  |  |  |
|  | Y | U |  | V |  | Y |  | U |  | V |
| Class A |  |  |  |  |  |  |  |  |  |  |
| Class B | 0,1% | 0,8% |  | 1,0% |  | 0,1% |  | 0,7% |  | 0,8% |
| Class C | 0,0% | 0,4% |  | 0,3% |  | 0,1% |  | 0,3% |  | 0,4% |
| Class D | 0,1% | 0,0% |  | -0,1% |  | 0,2% |  | 0,4% |  | 0,2% |
| Class E | 0,2% | 0,2% |  | -0,1% |  | 0,3% |  | 0,5% |  | 0,6% |
| Overall | 0,1% | 0,4% |  | 0,3% |  | 0,1% |  | 0,5% |  | 0,5% |
|  | 0,1% | 0,4% |  | 0,3% |  | 0,1% |  | 0,4% |  | 0,5% |
| Class F | 0,8% | 0,7% |  | 1,8% |  | 0,9% |  | 0,7% |  | 1,7% |
| Enc Time[%] | 99% |  |  |  |  | 98% |  |  |  |  |
| Dec Time[%] | 95% |  |  |  |  | 103% |  |  |  |  |
|  | Low delay P Main |  |  |  |  | Low delay P HE10 |  |  |  |  |
|  | Y |  | U |  | V | Y |  | U |  | V |
| Class A |  |  |  |  |  |  |  |  |  |  |
| Class B | 0,0% |  | 1,2% |  | 1,6% | -0,1% |  | 1,4% |  | 1,7% |
| Class C | -0,1% |  | 0,4% |  | 0,6% | -0,1% |  | 0,3% |  | 0,5% |
| Class D | 0,1% |  | 0,3% |  | 0,6% | 0,0% |  | 0,5% |  | 0,5% |
| Class E | -0,1% |  | 0,4% |  | 0,6% | 0,0% |  | 0,8% |  | 0,4% |
| Class F | 0,7% |  | 0,6% |  | 1,2% | 0,8% |  | 1,3% |  | 2,4% |
| Overall | 0,1% |  | 0,6% |  | 0,9% | 0,1% |  | 0,9% |  | 1,1% |
|  | 0,1% |  | 0,6% |  | 0,9% | 0,1% |  | 0,9% |  | 1,1% |
| Enc Time[%] | 102% |  |  |  |  | 100% |  |  |  |  |
| Dec Time[%] | 102% |  |  |  |  | 109% |  |  |  |  |

## Results for RD penalty on maximum TU size for intra CU’s in inter slices deblocking parameters and modified strong filter

|  | All Intra Main |  |  |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U |  |  | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% |  |  | 0.0% | 0.1% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | -0.1% | 0.0% |  |  | 0.0% | -0.1% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |  | 98% |  |  |
| Dec Time[%] | 106% |  |  |  |  | 109% |  |  |
|  | Random Access Main |  |  |  |  | Random Access HE10 |  |  |
|  | Y | U |  |  | V | Y | U | V |
| Class A (8bit) | -0.4% | 0.2% |  |  | 0.3% | -0.3% | 0.1% | 0.1% |
| Class B | 0.2% | 1.0% |  |  | 1.0% | 0.4% | 1.0% | 0.9% |
| Class C | 0.1% | 0.5% |  |  | 0.5% | 0.1% | 0.6% | 0.5% |
| Class D | 0.1% | 0.4% |  |  | 0.7% | 0.2% | 0.7% | 0.6% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.1% | 0.6% |  |  | 0.7% | 0.2% | 0.7% | 0.6% |
|  | 0.1% | 0.6% |  |  | 0.7% | 0.2% | 0.7% | 0.6% |
| Class F | 0.4% | 0.7% |  |  | 0.8% | 0.5% | 0.6% | 0.7% |
| Enc Time[%] | 109% |  |  |  |  | 108% |  |  |
| Dec Time[%] | 110% |  |  |  |  | 115% |  |  |
|  | Low delay B Main |  |  |  |  | Low delay B HE10 |  |  |
|  | Y | U |  |  | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 0.1% | 0.8% |  |  | 0.6% | 0.4% | 1.1% | 0.8% |
| Class C | 0.2% | 0.3% |  |  | 0.3% | 0.4% | 0.5% | 0.5% |
| Class D | 0.2% | -0.2% |  |  | 0.1% | 0.3% | 0.4% | 0.4% |
| Class E | 0.6% | 0.6% |  |  | 0.2% | 1.8% | 1.4% | 1.3% |
| Overall | 0.2% | 0.4% |  |  | 0.3% | 0.6% | 0.8% | 0.7% |
|  | 0.2% | 0.4% |  |  | 0.4% | 0.6% | 0.8% | 0.7% |
| Class F | 0.7% | 0.5% |  |  | 1.4% | 1.0% | 0.4% | 1.1% |
| Enc Time[%] | 107% |  |  |  |  | 106% |  |  |
| Dec Time[%] | 114% |  |  |  |  | 120% |  |  |
|  | Low delay P Main |  |  |  |  | Low delay P HE10 |  |  |
|  | Y |  | U | V |  | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.2% |  | 1.1% | 1.3% |  | -0.4% | 1.6% | 1.8% |
| Class C | 0.0% |  | 0.4% | 0.5% |  | 0.1% | 0.4% | 0.6% |
| Class D | 0.0% |  | 0.6% | 0.3% |  | 0.0% | 0.4% | 0.2% |
| Class E | -0.1% |  | 0.7% | 0.6% |  | 0.0% | 0.9% | 0.6% |
| Class F | 0.5% |  | 1.0% | 1.2% |  | 0.9% | 1.6% | 3.0% |
| Overall | 0.0% |  | 0.8% | 0.8% |  | 0.1% | 1.0% | 1.3% |
|  | 0.0% |  | 0.8% | 0.8% |  | 0.1% | 1.0% | 1.2% |
| Enc Time[%] | 106% |  |  |  |  | 105% |  |  |
| Dec Time[%] | 122% |  |  |  |  | 119% |  |  |

# Results on the AHG test sequences

The following results (PSNR and bitrates) were obtained on the AHG sequences (that are intended for showing the subjective quality improvements).

## HM9.1 with deblocking parameters

RA main configuration

| RA-Main | sequences | QP | item | bitrate | Ypsnr | Upsnr | Vpsnr |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | riverbed | 32 | hm9.0 | 4778.61 | 33.55 | 38.65 | 41.27 |
|  |  |  | hm9.1_dbl_par | 4774.92 | 33.57 | 38.65 | 41.28 |
|  |  | 37 | hm9.0 | 2359.65 | 30.82 | 38.00 | 40.89 |
|  |  |  | hm9.1_dbl_par | 2359.13 | 30.84 | 38.01 | 40.89 |
|  | DucksTakeOff, 25fps | 32 | hm9.0 | 9222.13 | 29.56 | 32.50 | 36.73 |
|  |  |  | hm9.1_dbl_par | 9218.91 | 29.56 | 32.48 | 36.73 |
|  |  | 37 | hm9.0 | 3957.91 | 27.27 | 31.56 | 36.27 |
|  |  |  | hm9.1_dbl_par | 3955.25 | 27.27 | 31.55 | 36.27 |
|  | RedKayak,570fr | 32 | hm9.0 | 3724.01 | 35.65 | 41.82 | 44.92 |
|  |  |  | hm9.1_dbl_par | 3721.25 | 35.65 | 41.82 | 44.91 |
|  |  | 37 | hm9.0 | 1721.03 | 32.95 | 41.01 | 44.12 |
|  |  |  | hm9.1_dbl_par | 1720.67 | 32.95 | 41.01 | 44.12 |
|  | WestWindEasy | 32 | hm9.0 | 3165.84 | 35.33 | 44.92 | 47.65 |
|  |  |  | hm9.1_dbl_par | 3162.79 | 35.34 | 44.92 | 47.65 |
|  |  | 37 | hm9.0 | 1489.67 | 32.50 | 43.88 | 46.78 |
|  |  |  | hm9.1_dbl_par | 1488.14 | 32.51 | 43.89 | 46.78 |
|  | F.ChinaSpeed | 32 | hm9.0 | 1312.51 | 35.06 | 40.72 | 40.63 |
|  |  |  | hm9.1_dbl_par | 1313.02 | 35.04 | 40.72 | 40.62 |
|  |  | 37 | hm9.0 | 680.96 | 31.92 | 39.24 | 39.06 |
|  |  |  | hm9.1_dbl_par | 681.05 | 31.91 | 39.24 | 39.06 |

LD main configuration

| LB-Main | sequences | QP | item | bitrate | Ypsnr | Upsnr | Vpsnr |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | riverbed | 32 | hm9.0 | 5863.00 | 34.64 | 38.96 | 41.40 |
|  |  |  | CE1_test2 | 5860.37 | 34.66 | 38.96 | 41.40 |
|  |  | 37 | hm9.0 | 2874.58 | 31.72 | 38.13 | 40.89 |
|  |  |  | CE1_test2 | 2874.65 | 31.74 | 38.13 | 40.90 |
|  | DucksTakeOff, 25fps | 32 | hm9.0 | 12344.34 | 30.35 | 32.24 | 36.47 |
|  |  |  | CE1_test2 | 12332.06 | 30.35 | 32.24 | 36.47 |
|  |  | 37 | hm9.0 | 5115.48 | 27.51 | 31.23 | 35.96 |
|  |  |  | CE1_test2 | 5103.77 | 27.53 | 31.23 | 35.97 |
|  | RedKayak,570fr | 32 | hm9.0 | 4607.96 | 36.44 | 41.90 | 44.88 |
|  |  |  | CE1_test2 | 4606.97 | 36.44 | 41.91 | 44.89 |
|  |  | 37 | hm9.0 | 2124.86 | 33.48 | 41.00 | 44.02 |
|  |  |  | CE1_test2 | 2126.73 | 33.49 | 40.96 | 43.98 |
|  | WestWindEasy | 32 | hm9.0 | 3762.90 | 36.22 | 44.73 | 47.41 |
|  |  |  | CE1_test2 | 3752.18 | 36.23 | 44.71 | 47.39 |
|  |  | 37 | hm9.0 | 1680.39 | 33.01 | 43.60 | 46.67 |
|  |  |  | CE1_test2 | 1674.83 | 33.02 | 43.59 | 46.65 |
|  | F.ChinaSpeed | 32 | hm9.0 | 1256.99 | 35.30 | 40.25 | 40.20 |
|  |  |  | CE1_test2 | 1259.04 | 35.27 | 40.24 | 40.19 |
|  |  | 37 | hm9.0 | 614.20 | 32.00 | 38.72 | 38.42 |
|  |  |  | CE1_test2 | 614.73 | 31.98 | 38.73 | 38.41 |

## Proposed strong filter and deblocking parameters

RA main configuration

| RA-Main | sequences | QP | item | bitrate | Ypsnr | Upsnr | Vpsnr |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | riverbed | 32 | hm9.0 | 4778.61 | 33.55 | 38.65 | 41.27 |
|  |  |  | hm9.1_filt_offsets | 4775.86 | 33.57 | 38.65 | 41.28 |
|  |  | 37 | hm9.0 | 2359.65 | 30.82 | 38.00 | 40.89 |
|  |  |  | hm9.1_filt_offsets | 2357.02 | 30.85 | 38.00 | 40.88 |
|  | DucksTakeOff, 25fps | 32 | hm9.0 | 9222.13 | 29.56 | 32.50 | 36.73 |
|  |  |  | hm9.1_filt_offsets | 9213.64 | 29.57 | 32.48 | 36.73 |
|  |  | 37 | hm9.0 | 3957.91 | 27.27 | 31.56 | 36.27 |
|  |  |  | hm9.1_filt_offsets | 3954.74 | 27.28 | 31.55 | 36.27 |
|  | RedKayak,570fr | 32 | hm9.0 | 3724.01 | 35.65 | 41.82 | 44.92 |
|  |  |  | hm9.1_filt_offsets | 3720.23 | 35.66 | 41.83 | 44.92 |
|  |  | 37 | hm9.0 | 1721.03 | 32.95 | 41.01 | 44.12 |
|  |  |  | hm9.1_filt_offsets | 1720.91 | 32.95 | 41.01 | 44.12 |
|  | WestWindEasy | 32 | hm9.0 | 3165.84 | 35.33 | 44.92 | 47.65 |
|  |  |  | hm9.1_filt_offsets | 3164.86 | 35.35 | 44.92 | 47.65 |
|  |  | 37 | hm9.0 | 1489.67 | 32.50 | 43.88 | 46.78 |
|  |  |  | hm9.1_filt_offsets | 1487.37 | 32.53 | 43.89 | 46.78 |
|  | F.ChinaSpeed | 32 | hm9.0 | 1312.51 | 35.06 | 40.72 | 40.63 |
|  |  |  | hm9.1_filt_offsets | 1311.68 | 35.05 | 40.72 | 40.63 |
|  |  | 37 | hm9.0 | 680.96 | 31.92 | 39.24 | 39.06 |
|  |  |  | hm9.1_filt_offsets | 680.64 | 31.92 | 39.24 | 39.06 |

LD main configuration

| LB-Main | sequences | QP | item | bitrate | Ypsnr | Upsnr | Vpsnr |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | riverbed | 32 | hm9.0 | 5863.00 | 34.64 | 38.96 | 41.40 |
|  |  |  | hm9.1_filt_offsets | 5855.86 | 34.65 | 38.96 | 41.40 |
|  |  | 37 | hm9.0 | 2874.58 | 31.72 | 38.13 | 40.89 |
|  |  |  | hm9.1_filt_offsets | 2871.50 | 31.75 | 38.13 | 40.91 |
|  | DucksTakeOff, 25fps | 32 | hm9.0 | 12344.34 | 30.35 | 32.24 | 36.47 |
|  |  |  | hm9.1_filt_offsets | 12332.37 | 30.36 | 32.24 | 36.47 |
|  |  | 37 | hm9.0 | 5115.48 | 27.51 | 31.23 | 35.96 |
|  |  |  | hm9.1_filt_offsets | 5102.36 | 27.54 | 31.23 | 35.96 |
|  | RedKayak,570fr | 32 | hm9.0 | 4607.96 | 36.44 | 41.90 | 44.88 |
|  |  |  | hm9.1_filt_offsets | 4604.97 | 36.44 | 41.91 | 44.88 |
|  |  | 37 | hm9.0 | 2124.86 | 33.48 | 41.00 | 44.02 |
|  |  |  | hm9.1_filt_offsets | 2123.53 | 33.50 | 40.99 | 44.01 |
|  | WestWindEasy | 32 | hm9.0 | 3762.90 | 36.22 | 44.73 | 47.41 |
|  |  |  | hm9.1_filt_offsets | 3750.18 | 36.22 | 44.71 | 47.39 |
|  |  | 37 | hm9.0 | 1680.39 | 33.01 | 43.60 | 46.67 |
|  |  |  | hm9.1_filt_offsets | 1673.29 | 33.03 | 43.59 | 46.66 |
|  | F.ChinaSpeed | 32 | hm9.0 | 1256.99 | 35.30 | 40.25 | 40.20 |
|  |  |  | hm9.1_filt_offsets | 1256.80 | 35.28 | 40.24 | 40.16 |
|  |  | 37 | hm9.0 | 614.20 | 32.00 | 38.72 | 38.42 |
|  |  |  | hm9.1_filt_offsets | 613.65 | 32.00 | 38.72 | 38.40 |

## RD penalty on maximum TU size for intra CU’s in inter slices and deblocking parameters

RA main configuration

| RA-Main | sequences | QP | Item | bitrate | Ypsnr | Upsnr | Vpsnr |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | riverbed | 32 | hm9.0 | 4778,61 | 33,5517 | 38,6472 | 41,2738 |
|  |  |  | hm9.1_pen32_offsets | 4940,9512 | 33,5028 | 38,6648 | 41,1758 |
|  |  | 37 | hm9.0 | 2359,65 | 30,8206 | 38,0034 | 40,8909 |
|  |  |  | hm9.1_pen32_offsets | 2440,1224 | 30,7621 | 38,0536 | 40,7003 |
|  | DucksTakeOff, 25fps | 32 | hm9.0 | 9222,13 | 29,5623 | 32,4952 | 36,7322 |
|  |  |  | hm9.1_pen32_offsets | 9302,9888 | 29,5461 | 32,4107 | 36,746 |
|  |  | 37 | hm9.0 | 3957,91 | 27,2664 | 31,558 | 36,2655 |
|  |  |  | hm9.1_pen32_offsets | 4013,8136 | 27,2523 | 31,5097 | 36,2684 |
|  | RedKayak,570fr | 32 | hm9.0 | 3724,0063 | 35,6519 | 41,8222 | 44,9172 |
|  |  |  | hm9.1_pen32_offsets | 3748,8779 | 35,5402 | 41,6641 | 44,5988 |
|  |  | 37 | hm9.0 | 1721,0261 | 32,9462 | 41,0137 | 44,1235 |
|  |  |  | hm9.1_pen32_offsets | 1733,9145 | 32,8022 | 40,8188 | 43,7849 |
|  | WestWindEasy | 32 | hm9.0 | 3165,8416 | 35,3253 | 44,916 | 47,6498 |
|  |  |  | hm9.1_pen32_offsets | 3202,9696 | 35,3181 | 44,8555 | 47,3958 |
|  |  | 37 | hm9.0 | 1489,6656 | 32,503 | 43,8785 | 46,7761 |
|  |  |  | hm9.1_pen32_offsets | 1506,8712 | 32,4918 | 43,7754 | 46,3829 |
|  | F.ChinaSpeed | 32 | hm9.0 | 1312,5082 | 35,0612 | 40,7215 | 40,6319 |
|  |  |  | hm9.1_pen32_offsets | 1319,1091 | 35,035 | 40,705 | 40,6076 |
|  |  | 37 | hm9.0 | 680,9587 | 31,9206 | 39,2391 | 39,057 |
|  |  |  | hm9.1_pen32_offsets | 684,1824 | 31,9006 | 39,2195 | 39,0346 |

LD main configuration

| LB-Main | sequences | QP | item | bitrate | Ypsnr | Upsnr | Vpsnr |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | riverbed | 32 | hm9.0 | 5863,004 | 34,6438 | 38,9627 | 41,404 |
|  |  |  | hm9.1_pen32_offsets | 6012,3856 | 34,5705 | 38,8881 | 41,2767 |
|  |  | 37 | hm9.0 | 2874,5784 | 31,7192 | 38,1301 | 40,8906 |
|  |  |  | hm9.1_pen32_offsets | 2957,0216 | 31,6628 | 38,1378 | 40,7544 |
|  | DucksTakeOff, 25fps | 32 | hm9.0 | 12344,344 | 30,347 | 32,2426 | 36,4681 |
|  |  |  | hm9.1_pen32_offsets | 12324,7064 | 30,3416 | 32,231 | 36,4552 |
|  |  | 37 | hm9.0 | 5115,48 | 27,5123 | 31,233 | 35,9641 |
|  |  |  | hm9.1_pen32_offsets | 5111,216 | 27,5261 | 31,213 | 35,9372 |
|  | RedKayak,570fr | 32 | hm9.0 | 4607,9558 | 36,4391 | 41,9046 | 44,8793 |
|  |  |  | hm9.1_pen32_offsets | 4625,0455 | 36,3291 | 41,7587 | 44,5872 |
|  |  | 37 | hm9.0 | 2124,8615 | 33,4845 | 40,9995 | 44,0153 |
|  |  |  | hm9.1_pen32_offsets | 2130,7634 | 33,3465 | 40,738 | 43,692 |
|  | WestWindEasy | 32 | hm9.0 | 3762,8984 | 36,2174 | 44,7311 | 47,4059 |
|  |  |  | hm9.1_pen32_offsets | 3770,072 | 36,192 | 44,6883 | 47,187 |
|  |  | 37 | hm9.0 | 1680,3904 | 33,0061 | 43,5975 | 46,6711 |
|  |  |  | hm9.1_pen32_offsets | 1690,528 | 32,9962 | 43,5049 | 46,3841 |
|  | F.ChinaSpeed | 32 | hm9.0 | 1256,9856 | 35,2955 | 40,246 | 40,2031 |
|  |  |  | hm9.1_pen32_offsets | 1263,695 | 35,2621 | 40,2292 | 40,1665 |
|  |  | 37 | hm9.0 | 614,1955 | 32,0014 | 38,7184 | 38,4243 |
|  |  |  | hm9.1_pen32_offsets | 618,5112 | 31,9761 | 38,7189 | 38,3809 |

## Proposed strong filter, RD penalty on maximum TU size for intra CU’s in inter slices and deblocking parameters

RA main configuration

| RA-Main | sequences | QP | Item | bitrate | Ypsnr | Upsnr | Vpsnr |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | riverbed | 32 | hm9.0 | 4778,61 | 33,5517 | 38,6472 | 41,2738 |
|  |  |  | hm9.1_filt_pen32_offsets | 4940,9216 | 33,5059 | 38,6645 | 41,178 |
|  |  | 37 | hm9.0 | 2359,65 | 30,8206 | 38,0034 | 40,8909 |
|  |  |  | hm9.1_filt_pen32_offsets | 2437,36 | 30,7749 | 38,0517 | 40,7129 |
|  | DucksTakeOff, 25fps | 32 | hm9.0 | 9222,13 | 29,5623 | 32,4952 | 36,7322 |
|  |  |  | hm9.1_filt_pen32_offsets | 9305,1712 | 29,5471 | 32,411 | 36,7455 |
|  |  | 37 | hm9.0 | 3957,91 | 27,2664 | 31,558 | 36,2655 |
|  |  |  | hm9.1_filt_pen32_offsets | 4011,0912 | 27,2564 | 31,5079 | 36,2711 |
|  | RedKayak,570fr | 32 | hm9.0 | 3724,0063 | 35,6519 | 41,8222 | 44,9172 |
|  |  |  | hm9.1_filt_pen32_offsets | 3748,6042 | 35,5476 | 41,6611 | 44,5983 |
|  |  | 37 | hm9.0 | 1721,0261 | 32,9462 | 41,0137 | 44,1235 |
|  |  |  | hm9.1_filt_pen32_offsets | 1732,8404 | 32,8152 | 40,8153 | 43,7833 |
|  | WestWindEasy | 32 | hm9.0 | 3165,8416 | 35,3253 | 44,916 | 47,6498 |
|  |  |  | hm9.1_filt_pen32_offsets | 3199,6152 | 35,318 | 44,8595 | 47,3963 |
|  |  | 37 | hm9.0 | 1489,6656 | 32,503 | 43,8785 | 46,7761 |
|  |  |  | hm9.1_filt_pen32_offsets | 1505,8352 | 32,5013 | 43,7799 | 46,4138 |
|  | F.ChinaSpeed | 32 | hm9.0 | 1312,5082 | 35,0612 | 40,7215 | 40,6319 |
|  |  |  | hm9.1_filt_pen32_offsets | 1316,8421 | 35,0417 | 40,7079 | 40,6057 |
|  |  | 37 | hm9.0 | 680,9587 | 31,9206 | 39,2391 | 39,057 |
|  |  |  | hm9.1_filt_pen32_offsets | 684,5832 | 31,9134 | 39,223 | 39,0341 |

LD main configuration

| LB-Main | sequences | QP | item | bitrate | Ypsnr | Upsnr | Vpsnr |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | riverbed | 32 | hm9.0 | 5863,004 | 34,6438 | 38,9627 | 41,404 |
|  |  |  | hm9.1_filt_pen32_offsets | 6007,3944 | 34,5698 | 38,8835 | 41,2771 |
|  |  | 37 | hm9.0 | 2874,5784 | 31,7192 | 38,1301 | 40,8906 |
|  |  |  | hm9.1_filt_pen32_offsets | 2952,2208 | 31,6779 | 38,1294 | 40,7536 |
|  | DucksTakeOff, 25fps | 32 | hm9.0 | 12344,344 | 30,347 | 32,2426 | 36,4681 |
|  |  |  | hm9.1_filt_pen32_offsets | 12329,68 | 30,3482 | 32,2308 | 36,4582 |
|  |  | 37 | hm9.0 | 5115,48 | 27,5123 | 31,233 | 35,9641 |
|  |  |  | hm9.1_filt_pen32_offsets | 5108,2112 | 27,5316 | 31,2144 | 35,936 |
|  | RedKayak,570fr | 32 | hm9.0 | 4607,9558 | 36,4391 | 41,9046 | 44,8793 |
|  |  |  | hm9.1_filt_pen32_offsets | 4622,6147 | 36,3408 | 41,7665 | 44,5864 |
|  |  | 37 | hm9.0 | 2124,8615 | 33,4845 | 40,9995 | 44,0153 |
|  |  |  | hm9.1_filt_pen32_offsets | 2131,9777 | 33,3648 | 40,7428 | 43,6514 |
|  | WestWindEasy | 32 | hm9.0 | 3762,8984 | 36,2174 | 44,7311 | 47,4059 |
|  |  |  | hm9.1_filt_pen32_offsets | 3765,6888 | 36,1887 | 44,6753 | 47,1781 |
|  |  | 37 | hm9.0 | 1680,3904 | 33,0061 | 43,5975 | 46,6711 |
|  |  |  | hm9.1_filt_pen32_offsets | 1685,2176 | 33,0079 | 43,5037 | 46,3633 |
|  | F.ChinaSpeed | 32 | hm9.0 | 1256,9856 | 35,2955 | 40,246 | 40,2031 |
|  |  |  | hm9.1_filt_pen32_offsets | 1262,9208 | 35,2738 | 40,2315 | 40,1849 |
|  |  | 37 | hm9.0 | 614,1955 | 32,0014 | 38,7184 | 38,4243 |
|  |  |  | hm9.1_filt_pen32_offsets | 618,0619 | 31,9843 | 38,715 | 38,383 |

# Conclusions

The proposal claims that deblocking artifacts for sequences with chaotic motions can be significantly reduced by suggested modifications.

It is proposed to adopt the tc_offset modifications to the common test conditions. It is also proposed to adopt the encoder modifications to the next version of HM software and the proposed deblocking filter modifications into the software and HEVC FDIS.

# References

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-J1100, Stockholm, Sweden, July 2012

- A. Norkin, “Non-CE1: Non-normative improvement to deblocking filtering”, JCTVC-K0289, Shanghai, China, October 2012

- A. Norkin, “CE1: Reduction of block artifacts in HEVC”, JCTVC-K0186, Shanghai, China, October 2012

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).