| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0233 |
| --- | --- |

| Title: | Multi-Type Inter Layer Sample Adaptive Offset Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Pu Jianle Chen Krishna Rapaka Marta Karczewicz 5775 Morehouse Dr. San Diego, CA 92121, USA | Tel: Email: | +1-858-651-7749 Wei.Pu@qti.qualcomm.com cjianle@qti.qualcomm.com krapaka@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the results of multi-type inter layer sample adaptive offset filter (MT-IL-SAO) for SHVC. MT-IL-SAO is applied to the base layer reconstructed pictures for the SNR salability case and the up-sampled base layer reconstructed pictures for the spatial scalability case. The major differences between SAO in HEVC and the proposed MT-IL-SAO are: 1) MT-IL-SAO is applied in picture basis while SAO in HEVC is CTU based; 2) MT-IL-SAO allows more than one SAO type for one single picture; 3) MT-IL-SAO uses one pass parallel filtering at the decoder for multiple SAO types. Compared with SHVC base software v0.1.1, MT-IL-SAO with two types of SAO shows BD-rate reduction of 0.3%, 0.1%, 0.4%, 0.1%, 0.5%, 0.5%, 0.4%, 0.7% for AI 2x, AI 1.5x, RA 2x, RA 1.5x, RA SNR, LD-P 2x, LD-P 1.5x, LD-P SNR, respectively.

# Introduction

In Shanghai meeting in October 2012, TE B4.3 was established to evaluate the performance of inter layer SAO [1]. This contribution proposes an alternative solution of inter layer SAO.

# Technical Description

## Overview

For inter layer SAO, picture based SAO can be used to reduce signaling overhead. However, in one picture, the texture may have more than one edge direction or it may have bands other than the consecutive four bands signaled in the sample adaptive offset parameters, which need to be corrected. It is beneficial to use multiple passes of SAO for picture based method.

## MT-IL-SAO Algorithm

Straightforward cascaded multi-pass inter layer SAO framework is illustrated in Fig. 1 and Fig.2. In the cascaded SAO, the second round of SAO input is the output of the first round of SAO, which introduces additional processing delay and extra memory access. To overcome this defect, Fig. 3 illustrates the recommended encoder side framework of MT-IL-SAO.

Base layer reference picture

SAO Pixel classification (1st)

SAO Offset Calculation (1st)

Apply 1st SAO base layer reference picture

SAO Offset Calculation (2nd)

SAO Pixel classification (2nd)

Apply 2nd SAO to base layer reference picture

Fig. 1. Cascaded SAO encoder architecture

Base layer reference picture

Apply 1st SAO

SAO Pixel classification (1st)

Apply 2nd SAO

SAO Pixel classification (2nd)

Fig. 2. Cascaded SAO decoder architecture

Base layer reference picture

SAO Pixel classification (1st)

SAO Offset Calculation (1st)

Apply 1st SAO to base layer reference picture

SAO Offset Calculation (2nd)

Pixel SAO classification (2nd)

Apply 2nd SAO to base layer reference picture

Fig. 3. MT-IL-SAO encoder architecture

Compared with Fig. 1, the SAO pixel classification is always performed based on the base layer reference picture. This modification favors the decoder for parallelism, as illustrated in Fig. 4. Both the first and second SAO can be processed once using the following equation.

saoPicture[i][j] = Clip3(0, (1<<bitDepth)-1, Clip3(0, (1 << bitDepth) − 1, recPicture[i][j] + SaoOffsetVal[ cIdx ][ saoIdx1 ]) + SaoOffsetVal[ cIdx ][ saoIdx2 ])

where saoIdx1 and saoIdx2 are based on the pixel classification result using the base layer reference picture as input.

Base layer reference picture

Apply SAO (1st+2nd)

SAO Pixel classification (1st, 2nd)

Fig. 4. MT-IL-SAO decoder architecture

## Syntax Description

## The syntax parameters of the proposed MT-IL-SAO are signaled in the slice header. Table 1 shows the syntax parameter signaled for MT-IL-SAO with maximum 2 SAO-types applied. The syntax elements of each round SAO are the same to the existing LCU-based SAO syntax except that the merge_flag is removed.

Table 1. Syntax of MT-IL-SAO

| parallel_sao { | Descriptor |
| --- | --- |
| for( rIdx = 0; rIdx < 2; rIdx++) |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| if( cIdx = = 0 ) |  |
| sao_type_idx_luma | ae(v) |
| else if( cIdx = = 1 ) |  |
| sao_type_idx_chroma | ae(v) |
| if( SaoTypeIdx[ cIdx ] != 0 ) { |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset_abs[ cIdx ] [ i ] | ae(v) |
| if( SaoTypeIdx[ cIdx ] = = 1 ) { |  |
| for( i = 0; i < 4; i++ ) |  |
| if( sao_offset_abs[ cIdx ][ i ] != 0 ) |  |
| sao_offset_sign[ cIdx ][ i ] | ae(v) |
| sao_band_position[ cIdx ] | ae(v) |
| } else { |  |
| if( cIdx = = 0 ) |  |
| sao_eo_class_luma | ae(v) |
| if( cIdx = = 1 ) |  |
| sao_eo_class_chroma | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

# Simulation Results

The proposed method is implemented using SHVC base software v0.1.1 and the simulation is done under SHVC common test conditions defined by AHG10. The results of MT-IL-SAO with maximum 2 SAO-types are shown in Table 2. Compared with SHVC base software v0.1.1, MT-IL-SAO achieves BD rate saving of 0.3%, 0.1%, 0.4%, 0.1%, 0.5%, 0.5%, 0.4%, 0.7% for AI 2x, AI 1.5x, RA 2x, RA 1.5x, RA SNR, LD-P 2x, LD-P 1.5x, LD-P SNR, respectively, with unnoticeable encoding time increase. The average decoding time increase is around 10%-20%.

Table 2.  Experimental results of MT-IL-SAO (Two SAO Types)

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- J. Chen, A. Segall, E. Alshina, S. Liu, J. Dong, and J. Park, “Description of Tool Experiment B4: Inter-layer filtering,” Document of Joint Collaborative Team on Video Coding, JCTVC-K1104, Oct. 2012.