| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0235 |
| --- | --- |

| Title: | TE3: Cross-check results of JCTVC-L0036 on TE3-4.1.2 Intra prediction based on reconstructed base layer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Krishna Rapaka 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +858 658 2042 krapaka@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports cross-check results of JCTVC-L0036 on TE3-4.1.2 Intra prediction based on reconstructed base layer. The simulation results reportedly matched those provided by the proponents.

# Introduction

In the TE3-4.1.2, Intra prediction based on reconstructed base layer a DC adjustment method has been proposed for enhancement layer Intra Prediction blocks of size 8x8 and larger. It has been reported that a delta DC value is added to the predicted enhancement layer sample in order to match the DC of the EL block with that of collocated BL block. The process has been represented as follows:

P(x,y) = P’(x,y) + dc_delta

Where:

P(x,y) denotes the sample to be predicted in the enhancement layer picture.

P’(x,y) denotes a predicted enhancement layer sample after HEVC (version 1) intra prediction process

dc_delta denotes the difference of base layer prediction block’s DC and P’(x,y) block’s DC component.

# Experimental results

We received the source code implemented on SMUC 0.1.1 from the proponents, and did a code study to verify that the proposed method is implemented as described. We used the common conditions generated from AHG10 in our experiments and ran simulations for AI cases (AI 2x and AI 1.5x).

For the below configuration the results match with the results provided by the proponents

#define TE3_MIX_INTRA_GRADIENT           0

#define TE3_MIX_INTRA_DC                 1

#define TE3_MIX_INTRA_DC_MIN_BLK_SIZE    8

#define TE3  ( TE3_MIX_INTRA_GRADIENT || TE3_MIX_INTRA_DC )

The Test results are as follows:

# Conclusion

In this contribution, we have presented the results of our cross-check of JCTVC-L0036. The implemented algorithm agrees with the proponent’s description, and the simulation results also match that provided by the proponents.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).