| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0236 |
| --- | --- |

| Title: | Non-TE2: Cross-check Results of Improved inter-layer texture prediction signaling (JCTVC-L0230) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Zhan Ma Felix Fernandes 1301 E. Lookout Drive, Richardson, TX 75082, U.S.A | Tel: Email: | +1-972-761-7134 zhan.ma@sta.samsung.com felix.f@sta.samsung.com |
| Source: | Samsung Electronics |  |  |

_____________________________

# Abstract

This contribution reports the cross-check results of Non-TE2: Cross-check Results of Improved inter-layer texture prediction signaling (JCTVC-L0230) proposed by Texas Instruments. All the BD-rate results matched exactly those provided by Texas Instruments.

# Introduction

Proposal is implemented with following features:

- Intra-BL Skip is inferred by no_residual_syntax_flag in intra Slice, while skip_flag and intraBL_flag are both used to signal intraBL Skip mode (to avoid using additional syntax)

- Merge mode is extended in inter slice to add inter-layer texture prediction for PU less than 2Nx2N.

- 4x4 IntraBL transform is fixed.

# Cross-check results

Proponent provided the source code and simulation results. The cross-check of JCTVC-L0230 is carried out by setting the macro definitions in the provided code as in Table 1.

Table 1 macro definitions

#define INTRA_BL_SKIP                    1

#define INTRA_BL_MERGE                   1

#define IBL_4x4_DST                       1

All the BD-rate results exactly matched the results provided by proponent. Table 2 and 3 shows the cross-check results. Please note that HM8.1 simulcast anchor is re-used from proponent’s data. Hence the time information in Table 3 is not accurate. More detailed results can be found in the Excel attachment. YUV files were written when the decoding times were measured. Both Encoding and Decoding were done on a Linux cluster with uniform computing nodes.

In addition, additional test is performed to cross-check the case where INTRA_BL_MERGE == 0. Result matches the data provided by the proponent, as shown in Table 4.

Table 2 Cross-check results for SMuC0.1.1bf anchor

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -0.1% | 0.3% | 0.3% |  |  |  |  |  |  |
| Class B | -0.2% | 0.4% | 0.3% | -0.3% | 0.6% | 0.3% |  |  |  |
| Overall (EL+BL) | -0.2% | 0.4% | 0.3% | -0.3% | 0.6% | 0.3% |  |  |  |
| Overall (EL) | -0.4% | 0.8% | 0.6% | -1.1% | 2.4% | 1.3% |  |  |  |
| Enc Time[%] | 101.9% |  |  | 101.9% |  |  |  |  |  |
| Dec Time[%] | 87.8% |  |  | 84.9% |  |  |  |  |  |
| Enc Mem[%] | 100.5% |  |  | 100.2% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.4% | 0.0% | 0.0% |  |  |  | -0.2% | 0.3% | 0.5% |
| Class B | -0.4% | 0.7% | 0.8% | -0.4% | 0.8% | 0.8% | -0.2% | 1.3% | 1.5% |
| Overall (EL+BL) | -0.4% | 0.5% | 0.6% | -0.4% | 0.8% | 0.8% | -0.2% | 1.0% | 1.2% |
| Overall (EL) | -0.8% | 0.9% | 0.9% | -1.5% | 2.0% | 1.5% | -0.5% | 1.8% | 2.1% |
| Enc Time[%] | 96.5% |  |  | 93.2% |  |  | 97.7% |  |  |
| Dec Time[%] | 93.9% |  |  | 90.1% |  |  | 93.5% |  |  |
| Enc Mem[%] | 100.3% |  |  | 100.2% |  |  | 100.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.7% | 0.1% | 0.0% |  |  |  | -0.5% | 0.5% | 0.6% |
| Class B | -0.7% | 0.8% | 1.0% | -0.9% | 0.8% | 1.0% | -0.6% | 1.3% | 1.7% |
| Overall (EL+BL) | -0.7% | 0.6% | 0.7% | -0.9% | 0.8% | 1.0% | -0.5% | 1.0% | 1.4% |
| Overall (EL) | -1.2% | 1.1% | 1.1% | -2.6% | 2.1% | 2.1% | -1.0% | 1.6% | 1.9% |
| Enc Time[%] | 97.4% |  |  | 93.4% |  |  | 98.3% |  |  |
| Dec Time[%] | 94.5% |  |  | 89.7% |  |  | 93.6% |  |  |
| Enc Mem[%] | 100.2% |  |  | 100.3% |  |  | 100.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 3 Cross-check results for HM8.1 simulcast anchor

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -27.2% | -27.1% | -27.9% |  |  |  |  |  |  |
| Class B | -21.5% | -21.0% | -20.9% | -32.9% | -32.7% | -33.0% |  |  |  |
| Overall (EL+BL) | -23.1% | -22.8% | -22.9% | -32.9% | -32.7% | -33.0% |  |  |  |
| Overall (EL) | -35.3% | -34.9% | -35.2% | -58.6% | -57.8% | -58.3% |  |  |  |
| Enc Time[%] | 110.4% |  |  | 107.5% |  |  |  |  |  |
| Dec Time[%] | 94.4% |  |  | 91.6% |  |  |  |  |  |
| Enc Mem[%] | 119.7% |  |  | 118.7% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -18.8% | -8.0% | -9.5% |  |  |  | -21.7% | -12.0% | -11.5% |
| Class B | -16.0% | -8.7% | -7.3% | -26.5% | -19.8% | -18.3% | -19.7% | -10.0% | -6.8% |
| Overall (EL+BL) | -16.8% | -8.5% | -7.9% | -26.5% | -19.8% | -18.3% | -20.3% | -10.6% | -8.2% |
| Overall (EL) | -25.9% | -13.2% | -12.5% | -48.0% | -37.3% | -34.7% | -32.0% | -17.8% | -14.3% |
| Enc Time[%] | 96.8% |  |  | 93.4% |  |  | 96.3% |  |  |
| Dec Time[%] | 134.2% |  |  | 127.5% |  |  | 103.4% |  |  |
| Enc Mem[%] | 116.4% |  |  | 114.9% |  |  | 103.8% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -14.1% | -2.2% | -3.3% |  |  |  | -14.4% | -7.3% | -6.2% |
| Class B | -12.4% | -6.5% | -4.6% | -22.6% | -16.6% | -14.8% | -13.3% | -4.6% | -0.6% |
| Overall (EL+BL) | -12.9% | -5.2% | -4.2% | -22.6% | -16.6% | -14.8% | -13.6% | -5.4% | -2.2% |
| Overall (EL) | -20.5% | -8.2% | -6.8% | -41.9% | -31.6% | -28.3% | -21.9% | -10.6% | -6.0% |
| Enc Time[%] | 97.3% |  |  | 93.7% |  |  | 96.4% |  |  |
| Dec Time[%] | 139.3% |  |  | 131.8% |  |  | 107.1% |  |  |
| Enc Mem[%] | 117.1% |  |  | 115.8% |  |  | 104.5% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 4 Cross-check results with INTRA_BL_MERGE disabled

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.2% | 0.1% |  |  |  | 0.1% | 0.4% | 0.5% |
| Class B | -0.2% | 0.6% | 0.6% | -0.2% | 0.8% | 0.7% | -0.1% | 1.0% | 1.0% |
| Overall (EL+BL) | -0.2% | 0.5% | 0.4% | -0.2% | 0.8% | 0.7% | 0.0% | 0.8% | 0.9% |
| Overall (EL) | -0.4% | 0.9% | 0.8% | -1.1% | 2.2% | 1.4% | -0.2% | 1.6% | 1.6% |
| Enc Time[%] | 96.3% |  |  | 92.8% |  |  | 97.4% |  |  |
| Dec Time[%] | 93.6% |  |  | 90.4% |  |  | 90.7% |  |  |
| Enc Mem[%] | 100.0% |  |  | 100.0% |  |  | 100.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.2% | 0.2% | 0.2% |  |  |  | -0.1% | 0.5% | 0.6% |
| Class B | -0.3% | 0.8% | 0.9% | -0.5% | 1.0% | 0.9% | -0.2% | 0.9% | 1.1% |
| Overall (EL+BL) | -0.3% | 0.6% | 0.7% | -0.5% | 1.0% | 0.9% | -0.2% | 0.8% | 0.9% |
| Overall (EL) | -0.6% | 1.2% | 1.1% | -1.9% | 2.9% | 2.2% | -0.5% | 1.4% | 1.5% |
| Enc Time[%] | 97.3% |  |  | 93.1% |  |  | 98.0% |  |  |
| Dec Time[%] | 94.0% |  |  | 89.3% |  |  | 90.7% |  |  |
| Enc Mem[%] | 100.0% |  |  | 100.1% |  |  | 100.0% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# Conclusion

The results presented by Texas Instruments (JCTVC-L0230) are verified by Samsung Electronics.

# Reference

[1] D.-K. Kwon, M. Budagavi, M. Zhou, “Non-TE2: Improved inter-layer texture prediction signaling”, JCTVC-L0230, Geneva, CH, Jan. 2013