| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0238 |
| --- | --- |

| Title: | TE5: Cross-check results of JCTVC-L0031 on TE5-5.1.3 Inter-layer intra mode prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Krishna Rapaka 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +858 658 2042 krapaka@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the results of a cross-check of JCTVC-L0031 on TE5-5.1.3 Inter-layer intra mode prediction. The simulation results reportedly matched those provided by the proponents.

# Introduction

In this Test TE5-5.1.3, an Inter-layer intra mode prediction method has been proposed. In this method, the difference of Intra prediction mode at enhancement layer with respect to the mode of co-located base layer is coded. The following binarization of difference values has been proposed:

- First bin indicates whether the difference is equal to zero, i.e. base layer intra prediction mode is equal to enhancement layer intra prediction mode or not.

- Second bin indicates the sign of the difference (either positive or negative)

- Third bin indicates whether the absolute value of the difference is equal to 1 or not.

- Fourth bin indicates whether the absolute value of the difference is equal to 2 or not.

- Since the difference values ranges from -34 to 34 and values 0, +-1 and +-2 are covered by the previous bins and the sign of the difference value is indicated by the second bin, there are a total of 32 absolute values that the difference can take. The remaining values can be coded by assigning absolute value of the difference minus 3 to 5 bins with equal probability (i.e. in bypass mode).

# Experimental results

We received the two source code (namely TE5_5.1.3_intraMode and TE5_5.1.3_intraMode-diagScanOnly) implemented on SMUC 0.1.1 from the proponents, and did a code study to verify that the proposed method is implemented as described. We used the common conditions generated from AHG10 in our experiments and ran simulations for the AI cases (AI-2x & AI-1.5x).

In Source Code named TE5_5.1.3_intraMode, for the below configuration the results match with the results provided by the proponents

#if !AVC_BASE

#define SVC_MVP                          0

#define SVC_BL_CAND_INTRA                0

#define DIFF_INTRA_DIR_CODING            1

#endif

#endif

The Test results for the above configuration are as follows:

In Source Code named TE5_5.1.3_intraMode-diagScanOnly, for the below configuration the results match with the results provided by the proponents :

#if !AVC_BASE

#define SVC_MVP                          0

#define SVC_BL_CAND_INTRA                0

#define DIFF_INTRA_DIR_CODING            1

#define COEFF_SCAN_IDX_DIAG_ONLY         1

#endif

#endif

The Test results for the above configuration are as follows:

# Conclusion

In this contribution, we have presented the results of our cross-check of JCTVC-L0031. The implemented algorithm agrees with the proponent’s description, and the simulation results also match that provided by the proponents.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).