| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0239 |
| --- | --- |

| Title: | nonTE5 : Assigning intra prediction mode to inter layer intra predicted blocks in enhance layer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungsun Kim, Joonyoung Park, Byeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea Mei Guo, Shan Liu, Shawmin Lei MediaTek USA 2860 Junction Ave, San Jose, CA 95134, USA | Email: Email: | jungsun.kim@lge.com mei.guo@mediatek.com shan.liu@mediatek.com |
| Source: | LG Electronics and MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to assign intra prediction mode to the blocks predicted by inter layer intra prediction in enhance layer. The prediction mode is derived as the intra prediction mode of the corresponding intra block in base layer. Or it is derived as DC when the corresponding block is not intra predicted. Intra prediction modes are assigned to both luma and chroma block. It reports -0.6%, -0.3%, -0.3% gain for AI 2x and -0.6%, -0.1%, -0.4% gain for overall bit-stream in AI 1.5x. It also reports -0.3%, -0.1%, -0.2% gain, -0.3%, 0.1%, -0.1% gain and -0.2%, 0.0%, -0.1% gain for RA 2x, RA 1.5x and RA SNR cases respectively. And 0.1%, -0.2%, -0.3% gain, -0.1%, 0.0%, -0.1% gain and -0.1%, -0.2%, -0.3% gain for LD 2x, LD 1.5x and LD SNR cases respectively.

# Introduction

In SMuC0.1.1, if block is coded by inter layer texture prediction,

- Transform for 4x4 Luma TU is DCT.

- Scanning order is upright.

- Intra prediction modes as a neighbour block for inferring MPMs are set as DC.

- The current reference software SMuC 0.1.1 has a bug caused losses in bd-rate. In SMuC 0.1.1, forward transform for inter layer intra predicted block is DCT and inverse transform is DST. It was reported previously and the bug is fixed as inverse transform is set to DCT. The modification brings 0.0 – -0.3% gains for each case.

- Proposed methods

We propose to code above three features as in intra prediction. And the prediction mode used for the cases is set as the intra prediction mode of the block corresponds to the center 4x4 unit within the current block for both luma and chroma component. The center 4x4 unit is located as Fig. 1. When the corresponding block is not available or is not intra predicted, the mode is set as DC.

Fig. 1 Intra prediction mode of the block which corresponds to 4x4 unit A is assigned to the current block

Therefore, if block is coded by inter layer texture prediction,

- Transform for 4x4 Luma TU is DST.

- Scanning order is upright, vertical or horizontal depending on the intra prediction mode assigned.

- Intra prediction modes as a neighbour block for MPMs are set as the intra prediction mode assigned.

# Simulation Results

The experimental result is shown in Table 1. The proposed method is implemented on top of SMuC0.1.1 provided. The detailed results are described in the accompanied excel files. The anchor of the test is SMuC0.1.1. Assigning intra prediction mode to inter layer intra predicted blocks brings -0.6%, -0.3%, -0.3% gain for AI 2x and -0.6%, -0.1%, -0.4% gain for AI 1.5x. It shows -0.3%, -0.1%, -0.2% gain, -0.3%, 0.1%, -0.1% gain and -0.2%, 0.0%, -0.1% gain for RA 2x, RA 1.5x and RA SNR cases respectively. It also shows -0.1%, -0.2%, -0.3% gain, -0.1%, 0.0%, -0.1% gain and -0.1%, -0.2%, -0.3% gain for LD 2x, LD 1.5x and LD SNR cases respectively.

Table 2 shows the results of limitation of scanning order to up-right scan in enhance layer to prevent parsing error caused by incorrect derivation of intra prediction mode from base layer.

Table 3 shows the results comparing SMuC0.1.1bf which corrects the misalignment between forward and inverse transform.

## Test Environment

Encoding and decoding was performed in cluster system and the environment is as following:

OS: Linux version 2.6.18-274.el5 (gcc version 4.1.2 20080704 (Red Hat 4.1.2-50))

CPU: Intel Xeon Quad Core E31230 (SANDY BRIDGE3.2GHz)

Memory : 12GB DDR3 UDIMM Memory

## Results

Table 1 BD rate of the proposed method against SMuC0.1.1

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -0.5% | -0.2% | -0.1% |  |  |  |  |  |  |
| Class B | -0.6% | -0.3% | -0.4% | -0.6% | -0.1% | -0.4% |  |  |  |
| Overall (EL+BL) | -0.6% | -0.3% | -0.3% | -0.6% | -0.1% | -0.4% |  |  |  |
| Overall (EL) | -1.0% | -0.4% | -0.5% | -1.5% | 0.1% | -0.4% |  |  |  |
| Enc Time[%] | 100.9% |  |  | 100.1% |  |  |  |  |  |
| Dec Time[%] | 101.2% |  |  | 100.7% |  |  |  |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.2% | -0.1% |  |  |  | -0.2% | -0.1% | -0.1% |
| Class B | -0.3% | -0.1% | -0.2% | -0.3% | 0.1% | -0.1% | -0.3% | 0.1% | -0.1% |
| Overall (EL+BL) | -0.3% | -0.1% | -0.2% | -0.3% | 0.1% | -0.1% | -0.2% | 0.0% | -0.1% |
| Overall (EL) | -0.4% | -0.2% | -0.2% | -0.7% | 0.3% | -0.1% | -0.3% | 0.3% | 0.1% |
| Enc Time[%] | 99.8% |  |  | 100.3% |  |  | 101.9% |  |  |
| Dec Time[%] | 100.2% |  |  | 100.6% |  |  | 102.2% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.2% | -0.2% |  |  |  | -0.1% | -0.4% | -0.3% |
| Class B | -0.1% | -0.1% | -0.3% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | -0.3% |
| Overall (EL+BL) | -0.1% | -0.2% | -0.3% | -0.1% | 0.0% | -0.1% | -0.1% | -0.2% | -0.3% |
| Overall (EL) | -0.1% | -0.3% | -0.4% | -0.2% | 0.0% | -0.2% | -0.1% | -0.2% | -0.3% |
| Enc Time[%] | 99.6% |  |  | 100.8% |  |  | 98.3% |  |  |
| Dec Time[%] | 100.0% |  |  | 101.0% |  |  | 99.5% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 2 BD rate of the proposed method with limitation of scanning order in Enhance layer against SMuC0.1.1

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -0.3% | 0.1% | 0.0% |  |  |  |  |  |  |
| Class B | -0.4% | 0.0% | 0.0% | -0.5% | 0.2% | 0.0% |  |  |  |
| Overall (EL+BL) | -0.4% | 0.0% | 0.0% | -0.5% | 0.2% | 0.0% |  |  |  |
| Overall (EL) | -0.7% | 0.1% | 0.0% | -1.3% | 0.7% | 0.4% |  |  |  |
| Enc Time[%] | 99.7% |  |  | 100.9% |  |  |  |  |  |
| Dec Time[%] | 100.2% |  |  | 101.6% |  |  |  |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.1% | -0.1% |  |  |  | -0.2% | 0.0% | 0.0% |
| Class B | -0.2% | 0.2% | 0.2% | -0.3% | 0.2% | 0.1% | -0.2% | 0.3% | 0.2% |
| Overall (EL+BL) | -0.2% | 0.1% | 0.1% | -0.3% | 0.2% | 0.1% | -0.2% | 0.2% | 0.1% |
| Overall (EL) | -0.3% | 0.2% | 0.1% | -0.6% | 0.5% | 0.3% | -0.3% | 0.6% | 0.4% |
| Enc Time[%] | 100.3% |  |  | 98.8% |  |  | 102.1% |  |  |
| Dec Time[%] | 100.8% |  |  | 99.5% |  |  | 102.5% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.2% |  |  |  | -0.1% | -0.4% | -0.2% |
| Class B | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% |
| Overall (EL+BL) | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% |
| Overall (EL) | -0.1% | -0.1% | -0.2% | -0.1% | 0.1% | 0.0% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 99.7% |  |  | 100.1% |  |  | 99.0% |  |  |
| Dec Time[%] | 100.3% |  |  | 100.6% |  |  | 100.1% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 3 BD rate of the proposed method against SMuC0.1.1bf

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -0.2% | -0.1% | 0.1% |  |  |  |  |  |  |
| Class B | -0.3% | -0.2% | -0.2% | -0.2% | -0.2% | -0.3% |  |  |  |
| Overall (EL+BL) | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% | -0.3% |  |  |  |
| Overall (EL) | -0.4% | -0.2% | -0.2% | -0.6% | -0.3% | -0.5% |  |  |  |
| Enc Time[%] | 105.4% |  |  | 104.9% |  |  |  |  |  |
| Dec Time[%] | 104.4% |  |  | 104.0% |  |  |  |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.1% | 0.0% |  |  |  | -0.1% | -0.1% | 0.0% |
| Class B | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Overall (EL+BL) | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Overall (EL) | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% |
| Enc Time[%] | 100.6% |  |  | 101.2% |  |  | 99.6% |  |  |
| Dec Time[%] | 101.5% |  |  | 101.9% |  |  | 100.9% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.1% | 0.0% |  |  |  | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | -0.1% |
| Overall (EL+BL) | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | -0.1% |
| Overall (EL) | 0.0% | 0.0% | 0.0% | -0.1% | -0.2% | 0.1% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 99.6% |  |  | 101.1% |  |  | 98.7% |  |  |
| Dec Time[%] | 100.9% |  |  | 102.0% |  |  | 100.6% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).