#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( chroma_format_idc != 0 ) |  |
| chroma_pred_from_luma_enabled_flag | u(1) |
| … |  |
| } |  |

### Derivation process for chroma intra prediction mode

Input to this process is a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture.

Output of this process is the variable IntraPredModeC.

The chroma intra prediction mode IntraPredModeC is derived using intra_chroma_pred_mode[ xB ][ yB ] and IntraPredModeY[ xB ][ yB ] as specified in Table 2.

Table 1 – Specification of IntraPredModeC when chroma_pred_from_luma_enabled_flag is equal to 1

| intra_chroma_pred_mode[ xB ][ yB ] | IntraPredMode[ xB ][ yB ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 26 | 10 | 1 | X ( 0 <= X < 35 ) |
| 0 | 34 | 0 | 0 | 0 | 0 |
| 1 | 26 | 34 | 26 | 26 | 26 |
| 2 | 10 | 10 | 34 | 10 | 10 |
| 3 | 1 | 1 | 1 | 34 | 1 |
| 4 | 35 | 35 | 35 | 35 | 35 |
| 5 | 0 | 26 | 10 | 1 | X |

Table 2 – Specification of IntraPredModeC when chroma_pred_from_luma_enabled_flag is equal to 0

| intra_chroma_pred_mode[ xB ][ yB ] | IntraPredMode[ xB ][ yB ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 26 | 10 | 1 | X ( 0 <= X < 35 ) |
| 0 | 34 | 0 | 0 | 0 | 0 |
| 1 | 26 | 34 | 26 | 26 | 26 |
| 2 | 10 | 10 | 34 | 10 | 10 |
| 3 | 1 | 1 | 1 | 34 | 1 |
| 4 | 0 | 26 | 10 | 1 | X |

##### Specification of Intra_FromLuma (35) prediction mode

Inputs to this process are:

–	a sample location ( xB, yB ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

–	luma neighbouring samples pLM[ x, y ], with x = −1, y = 0... 2*nS − 1; and x = 0…2*nS − 1, y = − 1,

–	a variable nS specifying the transform block size.

Output of this process are:

–	predicted samples predSamples[ x, y ], with x, y = 0..nS−1.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS−1, are derived as the following ordered steps:

- Variable k0 and the sample array pY’ are derived as:

k0 = Max( 0, BitDepthC + Log2( nS ) − 14 )		(8-1)

when chroma_format_idc is equals to 3,

pY’[ x, −1 ] = ( PLM[ x−1, −1 ] + 2*PLM[ x, −1 ] + PLM[ x+1, −1 ] + 2 ) >> 2, with x = 0..nS−1	(8-60)

otherwise,

pY’[ x, −1 ] = ( PLM[ 2*x−1, −1 ] + 2*PLM[ 2*x, −1 ] + PLM[ 2*x+1, −1 ] + 2 ) >> 2, with x = 0..nS−1	(8-2)

pY’[ −1, y ] =  PLM[ −1, y ], with y = 0..nS−1		(8-61)

if chroma_format_idc is equals to 1,

pY’[ x, y ] = ( recSamplesL[ 2*x, 2*y ] + recSamplesL[ 2*x, 2*y+1 ] ) >> 1, with x, y = 0..nS−1	(8-3)

else if chroma_format_idc is equals to 2,

pY’[ x, y ] = ( recSamplesL[ 2*x, y ] + recSamplesL[ 2*x, y ] ) >> 1, with x, y = 0..nS−1	(8-4)

otherwise,

pY’[ x, y ] = recSamplesL[ x, y ], with x, y = 0..nS−1	(8-5)

- Variables l, c, ll, lc and k1 are derived as follows:

l = y=0nS−1pY'[−1, y]+x=0nS−1pY'[x, −1]>>k0		(8-6)

c = y=0nS−1p[−1, y]+x=0nS−1p[x, −1]>>k0		(8-7)

ll = y=0nS−1pY'[−1, y]2+x=0nS−1pY'[x, −1]2>>k0	(8-8)

lc = y=0nS−1pY'[−1, y]∗p[−1, y]+y=0nS−1pY'[x, −1]∗p[x,−1]>>k0	(8-9)

k1 = Log2( (2*nS) >> k0 )		(8-10)

- Variables a, b and k are specified by the following pseudo-code:

avgY’ 	=  l >> k1
errY’ =  l & ( ( 1 << k1 ) − 1 )
avgC = c >> k1
errC = c & ( ( 1 << k1 )  − 1 )
a1 = lc − ( 2 * nS * avgY’ * avgC + avgY’ * errC + avgC * errY’ )
a2 = ll − ( 2 * nS * avgY’2 + 2 * avgY’ * errY’ )
k2 = ( a1 = = 0) ? 0 : Max( 0, Floor( Log2( Abs( a1 ) ) ) − BitDepthC + 2 )
k3 = ( a2 = = 0) ? 0 : Max( 0, Floor( Log2( Abs( a2 ) ) ) − 5 )
k4 = k3 − k2 + BitDepthC − 2
a1s = a1 >> k2
a2s = a2 >> k3
lmDiv = ( 1 << ( BitDepthC + 4 ) + a2s / 2 ) / a2s
a2t = ( a2s < 32) ? 0 : lmDiv
a2t = Clip1C ( a2t )		(8-11)
if( a2s < 32 )
	a3 = 0
else if( a2s >= 32 && k4 >= 0 )
	a3 = ( a1s * a2t ) >> k4 )
else
	a3 = ( a1s * a2t ) << ( −k4 )
a4 = Clip3( − 28, 28 − 1, a3 )
a5 = a4 << 7
k5 = ( a5 = = 0 ) ? 0 : k5 = Floor( Log2( Abs( a5 ) + ( Sign2( a5 ) − 1)/2 ) ) − 5
k = 13 − k5
a = a5 >> k5
b = avgC − ( ( a * avgY’ ) >> k )

- The values of the prediction samples predSamples[ x, y ] are derived as:

predSamples[ x, y ] = Clip1C( ( ( pY’[ x, y ] * a ) >> k ) + b ), with x, y = 0..nS−1	(8-12)

Table 9-32 Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset

| intra_chroma_pred_mode | 0 | prefix and suffix as specified in subclause 9 | prefix: 1 suffix: na | prefix: Error: Reference source not found suffix: na | prefix: 0 suffix: na, (Bypass) |
| --- | --- | --- | --- | --- | --- |
|  | 1 |  | prefix: 1 suffix: na | prefix: Error: Reference source not found suffix: na | prefix: 2 suffix: na, (Bypass) |
|  | 2 |  | prefix: 1 suffix: na | prefix: Error: Reference source not found suffix: na | prefix: 4 suffix: na, (Bypass) |

#### Binarization process for intra_chroma_pred_mode

Input to this process is a request for a binarization for the syntax element intra_chroma_pred_mode and the chroma_pred_from_luma_enabled_flag.

Output of this process is the binarization of the syntax element.

The binarization of the syntax element intra_chroma_pred_mode consists of a prefix part and (when present) a suffix part. Table 9-35 specifies the binarization of the prefix part and the suffix part.

Table 9-35 – Specifcation of intra_chroma_pred_mode depending on chroma_pred_from_luma_enabled_flag

| Value of intra_chroma_pred_mode | chroma_pred_from_luma_enabled_flag = 1 |  | chroma_pred_from_luma_enabled_flag = 0 |  |
| --- | --- | --- | --- | --- |
|  | prefix | suffix | prefix | suffix |
| 5 | 0 | n/a | n/a | n/a |
| 4 | 10 | n/a | 0 | n/a |
| 0 | 11 | 00 | 1 | 00 |
| 1 | 11 | 01 | 1 | 01 |
| 2 | 11 | 10 | 1 | 10 |
| 3 | 11 | 11 | 1 | 11 |