| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0244 |
| --- | --- |

| Title: | Main 10 Still Picture Profile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung, Ali Tabatabai 1730 N. First Street, MD:3NW San Jose, CA 95112 USA | Tel: Email: | 1-408-352-4725 cheung.auyeung@am.sony.com ali.tabatabai@am.sony.com |
|  | Teruhiko Suzuki 2-10-1, Osaki, Shinagawa-ku Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2740 teruhikos@jp.sony.com |
| Source: | Sony Electronics Inc. and Sony Corporation |  |  |

_____________________________

# Abstract

The consumer market for picture display and capture is migrating towards Deep Color which supports 10 or more bits per color channel. Deep Color also provides a migration path for standalone digital cameras to compete with smart phones. Therefore, this contribution proposes a Main 10 Still Picture Profile to support current and newer generations of imaging consumer products.

# Introduction

The picture display and capture consumer market is migrating towards Deep Color which supports 10 or more bits per color channel. Examples of such devices include Sony PS3, Sony EX700 series TVs, recent Sony XBR series TVs, Dell u2410 monitors, Dell u2711 monitors, newer NVIDIA Quadro graphics cards for Windows, NVIDIA GeForce 8 and newer graphic cards for Linux, etc. In particular, Deep Color provides an important migration path for both standalone digital cameras and smart phones with build in camera.

This contribution proposes a Main 10 Still Picture Profile to support Deep Color for consumer products.

# Main 10 Still Picture Profile

Bitstreams conforming to the Main 10 Still Picture profile shall obey the following constraints:

- The bitstream shall contain only one picture.

- Sequence parameter sets shall have chroma_format_idc equal to 1 only.

- Sequence parameter sets shall have bit_depth_luma_minus8 in the range of 0 to 2, inclusive.

- Sequence parameter sets shall have bit_depth_chroma_minus8 in the range of 0 to 2, inclusive.

- Sequence parameter sets shall have sps_max_dec_pic_buffering[ sps_max_sub_layers_minus1 ] equal to 0 only.

- Log2CtbSizeY shall be in the range from 4 to 6, inclusive.

- When a picture parameter set has tiles_enabled_flag is equal to 1, it shall have entropy_coding_sync_enabled_flag equal to 0.

- When a picture parameter set has tiles_enabled_flag is equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 256 for all values of i in the range of 0 to num_tile_columns_minus1, inclusive, and RowHeightInLumaSamples[ j ] shall be greater than or equal to 64 for all values of j in the range of 0 to num_tile_rows_minus1, inclusive.

- The number of times read_bits( 1 ) is called in subclauses Error: Reference source not found and Error: Reference source not found when parsing coding_tree_unit( ) data for any coding tree unit shall be less than or equal to 4 * RawCtuBits / 3.

- The level constraints specified for the Main 10 Still Picture profile in subclause Error: Reference source not found shall be fulfilled.

Conformance of a bitstream to the Main 10 Still Picture profile is indicated by general_profile_idc being equal to 4 or general_profile_compatibility_flag[ 4 ] being equal to 1.

Decoders conforming to the Main 10 Still Picture profile at a specific level (identified by a specific value of general_level_idc) shall be capable of decoding all bitstreams for which the all of following conditions apply:

- general_profile_compatibility_flag[ 4 ] is equal to 1.

- general_level_idc represents a level lower than or equal to the specified level.

- general_tier_flag represents a tier lower than or equal to the specified tier.

# Patent rights declaration(s)

Sony Electronics Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).