| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0250 |
| --- | --- |

| Title: | Mixed chroma format coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Minhua Zhou | Email: | madhukar@ti.com zhou@ti.com |
| Source: | Texas Instruments |  |  |

_____________________________

# Abstract

Applications such as wireless displays, remote desktop, remote gaming, cloud computing etc. are becoming popular. Video generated in these applications typically has mixed content consisting of natural video, text, graphics etc. Natural video is usually in 4:2:0 format where as text and graphics requires 4:4:4 for higher fidelity. This leads to video which is inherently of mixed chroma format where different portions of video pictures have different chroma formats. Standard video coders (e.g. AVC and HEVC Main profile) are designed to operate on pictures that have a single chroma format. This contribution describes four different approaches in which mixed chroma format video can be coded and transmitted and provides BD-Rate results when 4:2:0 content is upsampled and coded as 4:4:4 video (AI: 14.2%, RA: 12.1%, LDB: 10.7%, LDP: 10.9%). It recommends that a study of tools that can code video in mixed chroma format at the video coding layer be done.

# Mixed chroma format coding

Applications such as wireless displays, remote desktop, remote gaming, cloud computing etc. are becoming popular. Video content in these applications typically has mixed content consisting of natural video, text, graphics etc. as illustrated in Figure 1.

Mixed format video

Combination

HEVC

Circuits

Figure 1: Example of video with mixed content consisting of natural video, text, and graphics.

Natural video is typically in 4:2:0 format where as text and graphics requires 4:4:4 for higher fidelity. This leads to video which is inherently of mixed chroma format where different portions of picture have different chroma formats. Mixed chroma format video can be coded and transmitted using one of the following four different approaches explained below. A video consisting of 4:2:0 natural video and 4:4:4 text/graphics is used an example to explain the different approaches.

## Coding at the highest resolution chroma format in the video

4:2:0 natural video is upsampled to 4:4:4 resolution such that the whole video frame is at 4:4:4 resolution. A 4:4:4 video coder such as AVC or HEVC with range extensions can then be used to code the video. This approach leads to inefficiency in coding of the 4:2:0 sub-part of the video. To quantify the coding loss, HEVC 4:2:0 common test condition video was upsampled to 4:4:4 format and coded using the HM-range-extensions software. Summary of BD-Rate performance is provided below.

Table 1: BD-Rate results. Anchor is HM-9.0 common test conditions videos at 4:2:0 and test is HM-range-extensions using common test conditions videos upsampled to 4:4:4.

|  | Class A-E |
| --- | --- |
| AI-Main | 14.2% |
| RA-Main | 12.1% |
| LB-Main | 10.7% |
| LP-Main | 10.9% |

## Coding at the lowest resolution chroma format in the video

Alternatively, the 4:4:4 sub-part of the video can be downsampled to 4:2:0 and then the whole video can be coded at 4:2:0 resolution. The disadvantage of this approach is that it leads to visual quality loss in the text/graphics part that typically contains strong color edges.

## Coding mixed chroma format video using system standards

Mixed chroma format video can also be treated as a combination of multiple video bitstream with each bitstream having a different chroma format. A system level standard such as T.120 can be used to multiplex the constituent video streams and transmit them to the receiver which can then decode them and display them together on the screen. A disadvantage of this approach is that several system level standards are required to put together the solution. Some solutions on the market require on the order of 20 different standards to implement the solution. The complexity of this might be acceptable on the desktop computer but on embedded devices the complexity burden can be high leading to slowdown in performance.

## Coding mixed chroma format video at video coding layer

Mixed chroma format video can be coded at the video coding layer by modifying the encoding and decoding process at the video coding layer. The potential advantage of this approach is that the different chroma formats in the video are coded at their original resolutions without requiring system level standards. Reference [1] presents one such approach.

# Conclusions

Applications such as wireless displays, remote desktop, remote gaming, cloud computing etc. are becoming popular. Video in these applications typically has mixed content consisting of natural video, text, graphics etc. Natural video is usually in 4:2:0 format where as text and graphics requires 4:4:4 for higher fidelity. This leads to video which is inherently of mixed chroma format where different portions of video pictures have different chroma formats. Existing solutions that upsample/downsample the mixed chroma format video into a single chroma format and then use traditional video coders to code the video in single chroma format lead to coding and/or visual quality loss. Alternatively, using system level standards to code the multi chroma format video as a collection of different bitstreams leads to complexity increase. Natural video and text/graphics have different source statistics. Hence different coding techniques might be suitable for differing source types. Because of these considerations it is recommended that further study of tools that can code video in mixed chroma format at the video coding layer be carried out.

# References

1. T. Lin, S. Wang, “Mixed Chroma Sampling-rate coding: combining the merits of 4:4:4 and 4:2:0 and increasing the value of past 4:2:0 investment,” JCTVC-H0065.

# Patent rights declaration(s)

Texas Instruments may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).