| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L-0251 |
| --- | --- |

| Title: | TE2-3.2.3 Infer Zero motion vector when referencing co-located base layer picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra, Jie Zhao, and Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 USA | Tel: Email: | +1-360-817-8451 misrak@sharplabs.com jzhao@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP Laboratories of America, Inc. |  |  |

_____________________________

# Abstract

This document reports the results for TE2 test 3.2.3 on inter-layer texture prediction signaling for the scalable extension of High efficiency video coding (HEVC) standard. In the proposed approach the reference index framework of SMuC 0.1.1 is enabled. Enabling the reference index framework adds the up-sampled base layer picture to the end of reference picture list 0 and reference picture list 1. The primary change proposed in this contribution is to infer zero motion vectors for any prediction unit referencing the up-sampled base layer picture. The proposed approach achieves the following average luma BD rate improvements with HM-8.1 simulcast anchors: 23.0% /32.9% (AI 2x/AI 1.5x), 15.3%/25.6%/18.6% (RA 2x/RA 1.5x/RA SNR), 12.5%/22.4%/13.3% (LD-P 2x/ LD-P 1.5x/LD-P SNR). The proposed approach achieves an additional BD rate improvement of 0.1% to 0.4% for the SMuC 0.1.1 reference index framework with HM 8.1 simulcast anchors.

# Introduction

The contribution proposes the use of a high level syntax based approach towards exploiting the up-sampled base layer pixel reconstructions for enhancement layer prediction. This approach is asserted to be easier to support [3] in the current multi-view draft high efficiency video coding (MV-HEVC) specification.

# Algorithm and implementation

The code is based on SMuC0.1.1 with the macro REF_IDX_FRAMEWORK enabled. Enabling the macro adds an up-sampled base layer is added to the end of reference picture list of enhancement layer. When referencing the co-located up-sampled base layer picture the motion vector is inferred to be equal to zero. This method saves the bits used to code motion vector, therefore improves coding efficiency.

# Test Result

The test conditions used correspond to those outlined in the TE2 test description document JCTVC-K1102 [1]. The anchor software used for measuring BD rate is the SMuC 0.1.1. BD rates are measured with respect to HM 8.1 simulcast anchors. The detailed BD rate results can be found in the attached excel.

## BD rate for proposed infer zero MV method with HM 8.1 simulcast anchors

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -27.1% | -26.5% | -27.3% |  |  |  |  |  |  |
| Class B | -21.3% | -20.4% | -20.2% | -32.9% | -33.0% | -33.2% |  |  |  |
| Overall (EL+BL) | -23.0% | -22.1% | -22.2% | -32.9% | -33.0% | -33.2% |  |  |  |
| Overall (EL) | -35.1% | -34.0% | -34.3% | -58.8% | -58.5% | -58.9% |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -17.6% | -5.4% | -6.8% |  |  |  | -20.5% | -9.2% | -8.1% |
| Class B | -14.4% | -5.8% | -3.9% | -25.6% | -18.9% | -17.0% | -17.9% | -6.2% | -1.7% |
| Overall (EL+BL) | -15.3% | -5.7% | -4.8% | -25.6% | -18.9% | -17.0% | -18.6% | -7.0% | -3.5% |
| Overall (EL) | -23.7% | -9.0% | -7.8% | -46.6% | -36.0% | -32.8% | -29.5% | -12.5% | -7.4% |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -13.7% | -1.5% | -2.7% |  |  |  | -14.0% | -6.7% | -5.7% |
| Class B | -12.0% | -5.7% | -3.9% | -22.4% | -16.9% | -15.1% | -13.0% | -5.6% | -1.6% |
| Overall (EL+BL) | -12.5% | -4.5% | -3.5% | -22.4% | -16.9% | -15.1% | -13.3% | -5.9% | -2.8% |
| Overall (EL) | -19.9% | -7.1% | -5.8% | -41.7% | -32.3% | -29.0% | -21.4% | -11.2% | -6.8% |

## BD rate for reference index framework with HM 8.1 simulcast anchors

As reference, we also obtain results of SMuC0.1.1 with REF_IDX_FRAMEWORK enabled. The detailed BD rate results can be found in the attached excel sheet.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -26.9% | -26.2% | -27.1% |  |  |  |  |  |  |
| Class B | -21.2% | -20.1% | -20.0% | -32.8% | -32.9% | -33.1% |  |  |  |
| Overall (EL+BL) | -22.8% | -21.9% | -22.0% | -32.8% | -32.9% | -33.1% |  |  |  |
| Overall (EL) | -34.8% | -33.7% | -33.9% | -58.5% | -58.2% | -58.7% |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -17.3% | -4.6% | -6.1% |  |  |  | -20.3% | -8.3% | -7.3% |
| Class B | -14.2% | -5.2% | -3.3% | -25.4% | -18.8% | -16.9% | -17.7% | -5.8% | -1.2% |
| Overall (EL+BL) | -15.1% | -5.1% | -4.1% | -25.4% | -18.8% | -16.9% | -18.5% | -6.5% | -2.9% |
| Overall (EL) | -23.4% | -8.0% | -6.9% | -46.4% | -35.9% | -32.7% | -29.2% | -11.7% | -6.5% |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -13.3% | -1.1% | -2.3% |  |  |  | -13.7% | -6.4% | -5.4% |
| Class B | -11.6% | -5.6% | -3.9% | -22.1% | -16.9% | -15.2% | -12.7% | -5.7% | -1.8% |
| Overall (EL+BL) | -12.1% | -4.3% | -3.5% | -22.1% | -16.9% | -15.2% | -13.0% | -5.9% | -2.9% |
| Overall (EL) | -19.4% | -6.8% | -5.7% | -41.2% | -32.1% | -29.1% | -20.9% | -11.2% | -6.9% |

## Additional BD rate improvement with proposed approach for reference index framework with HM 8.1 simulcast anchors

The additional BD rate is computed as the difference between BD rate listed in sections 3.1 and 3.2.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.2% | -0.2% |  |  |  |
| Class B | -0.1% | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% |
| Overall (EL+BL) | -0.1% | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% |
| Overall (EL) | -0.2% | -0.3% | -0.3% | -0.3% | -0.3% | -0.2% |

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.8% | -0.7% |  |  |  | -0.1% | -0.8% | -0.8% |
| Class B | -0.2% | -0.6% | -0.6% | -0.1% | -0.1% | -0.1% | -0.2% | -0.4% | -0.5% |
| Overall (EL+BL) | -0.2% | -0.6% | -0.6% | -0.1% | -0.1% | -0.1% | -0.2% | -0.5% | -0.6% |
| Overall (EL) | -0.3% | -0.9% | -0.9% | -0.2% | -0.1% | -0.1% | -0.3% | -0.8% | -0.8% |

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.4% | -0.4% | -0.3% |  |  |  | -0.3% | -0.3% | -0.3% |
| Class B | -0.3% | -0.1% | 0.0% | -0.3% | -0.1% | 0.1% | -0.3% | 0.1% | 0.3% |
| Overall (EL+BL) | -0.4% | -0.2% | -0.1% | -0.3% | -0.1% | 0.1% | -0.3% | 0.0% | 0.1% |
| Overall (EL) | -0.5% | -0.3% | -0.1% | -0.4% | -0.1% | 0.1% | -0.5% | -0.1% | 0.1% |

# Conclusions

This document reports TE2 3.2.3 result in which motion vector is inferred to 0 when referencing co-located up-sampled base layer picture. The software is based on SMuC0.1.1 with REF_IDX_FRAMEWORK enabled. The proposed approach achieves the following average luma BD rate improvements with HM-8.1 simulcast anchors: 23.0% /32.9% (AI 2x/AI 1.5x), 15.3%/25.6%/18.6% (RA 2x/RA 1.5x/RA SNR), 12.5%/22.4%/13.3% (LD-P 2x/ LD-P 1.5x/LD-P SNR). The proposed approach achieves an additional BD rate improvement of 0.1% to 0.4% for the SMuC 0.1.1 reference index framework with HM 8.1 simulcast anchors.

# References

- L. Guo, Y. He, D. Kown, J. Zan, H. Lakshman, J. Kang, “Description of Tool Experiment A2: Inter-layer Texture Prediction Signaling in SHVC,” JCTVC-K1102, Shanghai, CN, Oct. 2012

- K. Misra, J. Zhao, S.-H. Kim, S.Deshpande, A. Segall, “Description of scalable video coding technology proposal by Sharp,” JCTVC-K0031, Shanghai, CN, Oct. 2012

- Kemal Ugur, Miska M. Hannuksela, Jani Lainema, Dmytro Rusanovskyy, “Unification of scalable and multi-view extensions with HLS only changes,” JCTVC-L0188, Geneva, CH, Jan. 2013.

# Patent rights declaration

SHARP may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).