| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0252 |
| --- | --- |

| Title: | Non-TE4: Adaptive up-sampling of base layer picture using bilateral filters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao, Kiran Misra, and Andrew Segall 5750 NW Pacific Rim Blvd Vancouver WA 98683 USA | Tel: Email: | +1-360-817-8451 jzhao@sharplabs.com, misrak@sharplabs.com, asegall@sharplabs.com |
| Source: | SHARP Laboratories of America, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the use of non-linear and content adaptive “bilateral” filters for inter-layer prediction.  The “bilateral” filter is an adaptive filter that is asserted to preserve edges while smoothing texture and noise.  In this proposal, the filtering is performed following the upsampling operation and switched (enabled/disabled) on the CU basis.  Multiple filters can be enabled and signaled in the PPS.  Results are reported following TE-B4 test conditions, and show EL+BL rate improvements compared to SMuC 0.1.1 anchors of: -1.4% (AI 2x), -0.6% (AI 1.5x), -1.4% (RA 2x), -0.6% (RA 1.5x), -0.6% (RA SNR), -1.4% (LD-P 2x), -0.7 (LD-P 1.5x), and -0.9% (LD-P SNR).

# Introduction

We propose a non-linear and content adaptive upsampling process using SMuC0.1.1 with INTRA_BL framework. Here, when the base layer picture is used for reference, i.e. INTRA_BL mode is on, multiple versions of the upsampled base layer pictures are made available for inter-layer prediction. The first up-sampled base layer corresponds to the fixed up-sampler of SMuC 0.1.1, while a second up-sampled version is generated by performing bilateral filtering on the first up-sampled base layer picture. Additional versions may also be generated by changing the parameters of the bilateral filter.  At the encoder, the upsampler is selected at a CU level in checkRDCostIntraBL based on RD cost.

Signaling the filter is accomplished with an up-sampler index that is added to the CU.  The index is sent only when the INTRA_BL mode is selected.  The number of active filters and filter parameters are signalled in PPS or slice header.

# Bilateral Filter

A bilateral filter is a non-linear filter [1].  It reduces the noise in the upsampled base layer while preserving edges. We assert that this is beneficial for spatial scalability, as it removes low amplitude and high frequency content that is normally associated with noise, while preserving high amplitude and high frequency information corresponding to edges and structure.  This is achieved by replacing the intensity value at each pixel by a weighted average of intensity values from nearby pixels within a window. The weight is based on a Gaussian distribution and relative pixel intensities.

# We propose to apply bilateral filtering to the upsampled base layer picture. A window size of 5x5 is suggested.  For a pixel at location x,y, the bilateral filtered output is then expressed as:

Output(x,y)=f(G(a,b))∗Input(a,b)f(G(a,b))

where (a, b) denote the neighbor pixel location within the window around input pixel location (x, y), The sum operation is on all the pixels within the window. Input(a,b) denotes the input pixel intensity at (a,b), G(a,b) denotes the gradient of the input at (a,b) relative to input at (x,y), and f is a function that maps the gradient information to a weight value.

The form of function f() controls the smoothing of the filter.  For example, if f() is a constant, then the bilateral filter is equal to a linear Gaussian operator.  Here, we express the function f() as:

f(G(a,b))=K(a,b)∗e−(G(a,b)22∗sigmaR2)

Where sigmaR is a control parameter to adjust the weight based on the Gradient. K(a,b) denotes a spatial kernel, which is typically a Gaussian kernel, to adjust the weight based on the distance of location (a,b) to the input pixel location (x,y). In our proposal, f() is implemented as a LUT to reduce computational complexity.

# Syntax

For adaptive upsampling, we anticipate the support of multiple non-linear filters at the decoder.  Here, we propose syntax that is flexible for handling multiple upsampling filters, and can reduce the signalling overhead when a large number of upsampling filters are used.

Example syntax tables are shown in the tables below.  Here, the number of bilateral filters is signalled in PPS. A most_probable_upsampler_idx  is signalled in slice header.  At CU level, a mapped upsampler index with respect to most_probable_upsampler_idx  is signalled.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| … |  |
| log2_parallel_merge_level_minus2 | ue(v) |
| if( getLayerId() > 0 ) { |  |
| num_bilateral_filter | ue(v) |
| for( i = 0; i < num_bilateral_filter; i++ ) |  |
| bilateral_filter_idx[ i ] | ue(v) |
| } |  |
| num_extra_slice_header_bits | u(3) |
| slice_segment_header_extension_present_flag | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| … |  |
| if( !dependent_slice_segment_flag ) { |  |
| … |  |
| } |  |
| if( getLayerId() > 0 ) { |  |
| most_probable_upsampler_idx | ue(v) |
| } |  |
| if( tiles_enabled_flag ¦ ¦ entropy_coding_sync_enabled_flag ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| slice_segment_header_extension_length | ue(v) |
| for( i = 0; i < slice_segment_header_extension_length; i++) |  |
| slice_segment_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( getLayerId() > 0 ) { |  |
| cu_intra_bl_flag[ x0 ][ y0 ] | u(1) |
| if(cu_ intra_bl_flag[ x0 ][ y0 ] ) { |  |
| cu_mapped_upsampler_idx[ x0 ][ y0 ] | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

cu_mapped_upsampler_idx[ x0 ][ y0 ] specify the mapped up-sampler index. The array indices  x0 , y0  specify the location ( x0 , y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The up-sampler index is obtained using:

cu_upsampler_idx[ x0 ][ y0 ] = cu_mapped_upsampler_idx[ x0 ][ y0 ]
if( cu_mapped_upsampler_idx[ x0 ][ y0 ] = = 0 ) {
	cu_upsampler_idx[ x0 ][ y0 ] = most_probable_upsampler_idx
} else if(cu_mapped_upsampler_idx[ x0 ][ y0 ] <= most_probable_upsampler_idx ) {
	cu_upsampler_idx[ x0 ][ y0 ]--
}

Mapping upsampler index to bilateral filter

if( getLayerId() >0 ) {
	for( i = 0; i < num_bilateral_filter; i++ ) {
		upsampler_idx_minus1_to_bilateral_filter_idx[ i ] = bilateral_filter_idx[ i ]
	}
}

# Results

Results are reported in Table 1.  Here, we enable one bilateral filter at the decoder.  We conduct the evaluation using TE4 test condition [2]. As can be seen from the Table, the method provides improved performance at higher spatial scalability ratios.  Specifically, we observe an overall (EL+BL) bit-rate reduction of   -1.4% for AI, RA and LD-P 2X test conditions.  This is equivalent to an EL bit-rate reduction of -2.8%, -2.6% and -2.5% for AI, RA and LD-P 2X, respectively.  Bit-rate reductions for other test conditions are also observed and in the range of -.6% to -.9% (EL+BL) or -.8% to -1.9% (EL).

Table 1. Result of Proposed Adaptive Upsampling Method vs. SMuC0.1.1 Anchor

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -2.4% | -2.8% | -3.3% |  |  |  |
| Class B | -1.0% | -0.9% | -1.0% | -0.6% | -0.3% | -0.6% |
| Overall (EL+BL) | -1.4% | -1.4% | -1.7% | -0.6% | -0.3% | -0.6% |
| Overall (EL) | -2.8% | -2.8% | -3.3% | -1.9% | -0.2% | -1.0% |

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -2.6% | 0.3% | -1.9% |  |  |  | -0.5% | 0.1% | -0.9% |
| Class B | -0.9% | -0.1% | -0.5% | -0.6% | 0.3% | -0.4% | -0.6% | 0.1% | -0.2% |
| Overall (EL+BL) | -1.4% | 0.0% | -0.9% | -0.6% | 0.3% | -0.4% | -0.6% | 0.1% | -0.4% |
| Overall (EL) | -2.6% | 0.2% | -1.7% | -1.4% | 1.0% | -0.8% | -0.8% | 0.5% | -0.7% |

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -2.2% | -0.7% | -2.2% |  |  |  | -0.4% | -0.7% | -1.1% |
| Class B | -1.0% | -0.9% | -1.1% | -0.7% | -0.4% | -0.7% | -1.1% | -1.1% | -1.0% |
| Overall (EL+BL) | -1.4% | -0.8% | -1.4% | -0.7% | -0.4% | -0.7% | -0.9% | -0.9% | -1.0% |
| Overall (EL) | -2.5% | -1.5% | -2.6% | -1.5% | -1.1% | -1.7% | -1.3% | -1.7% | -1.9% |

.

# Conclusions

In this document, a non-linear and content adaptive bilateral filtering process is proposed.  Average bit-rate reductions (BL+EL) of 1.4% are observed for AI, RA and LDP 2X conditions relative to SMuC0.1.1 anchors for the S-HEVC test conditions.  We propose further study of the method in a TE/CE.

# References

- C. Tomasi and R. Manduchi, “Bilateral filtering for gray and color images,” IEEE International Conference on Computer Vision, pp.839-846, 1998.

- J. Chen, A. Segall et al, “Description of Tool Experiment B4: Inter-layer Filtering,” JCTVC-K1104, Shanghai, CN, Oct. 2012

- K. Misra, J. Zhao et al, “Description of scalable video coding technology proposal by Sharp,” JCTVC-K0031, Shanghai, CN, Oct. 2012

# Patent rights declaration(s)

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).