| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0254_r1 |
| --- | --- |

| Title: | AHG9: On RPS derivation and marking process for long-term reference pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Byeong Moon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130 Korea | Tel: Email: | - hendry.hendry@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This proposal claims that there is a case for which the current HEVC specification for RPS derivation and reference picture marking is broken. The case is when there is a long-term reference picture is supposed to be removed from the DPB but is not and other reference picture with the same POC LSB is marked as LTRP in the next picture. If the newly marked LTRP is signalled with delta_poc_msb_present_flag equals to 0, according to current marking process, a decoder will keep the old LTRP, which should have been removed, and discard the supposed to be new LTRP. This document proposed to avoid such case by changing the reference picture marking process long-term reference picture.

# Introduction

RPS derivation process and reference picture marking process in the current HEVC specification is specified as the following in Section 8.3.2 can be summarized as follow:

- Derive long-term reference pictures (LTRPs)

- All reference pictures included in RefPicSetLtCurr and RefPicSetLtFoll are marked as "used for long-term reference".

- Derive short-term reference pictures (STRPs)

- All reference pictures in the decoded picture buffer that are not included in RefPicSetLtCurr, RefPicSetLtFoll, RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetStFoll are marked as "unused for reference"

Currently, long-term reference pictures (LTRP) are indicated by their least significant bit (LSB) part of their POC value and information to derive the most significant bit (MSB) part of the POCs (i.e., delta_poc_msb_present_flag [i]) is signalled optionally. To ensure that the signalling mechanism would work for reference picture set (RPS) derivation and marking process, the HEVC standard mandates that delta_poc_msb_present_flag[ i ] shall equal to 1 for every long-term picture in the RPS when there is one or more reference picture in the DPB with the POC LSB equal to POC LSB of the LTRP.

# Problem Statement

A problem of the current LTRP signalling designed and the RPS derivation and marking process is that it is possible to mark a LTRP as ‘unused for reference’ in a picture that may be discarded without affecting the decodability of other pictures. When a LTRP is marked as ‘unused for reference’ in a safely discardable picture, and the picture is indeed discarded, the LTRP will stay in the DPB and affect the marking process in the next picture if there is another LTRP with the same POC LSB with delta_poc_msb_present_flag equal to 0.

Fig. 1 illustrates the problem that current reference picture set derivation and marking process for LTRP has. The scenario illustrated in Fig. 1 is as follow:

- Eight bits are used to signalled POC LSB of pictures in the sequence

- There are two reference pictures in Pic 266 that has the same POC LSB, they are reference picture 0 (i.e., reference picture with POC 0 and marked as LTRP) and reference picture 256 (i.e., reference picture with POC 256 and marked as STRP), thus, MSB cycle for LTRP is signalled.

- In Pic 267, reference picture 0 is removed from DPB.

- In Pic 268, reference picture 256 is marked as LTRP.

In such situation, since reference picture 0 is removed in Pic 267, it is allowed to not signal MSB cycle for LTRP in Pic 268.

Fig. 1 – RPS derivation and marking process with problem

Problem occurs when Pic 267 is discarded so that the reference picture POC 0 will still exist in the DPB when Pic 268 is to be processed. In such situation, according to the current RPS derivation and marking process, the reference picture 0 will be kept in the DPB as LTRP and reference picture 256 will be removed from the DPB.

# Proposed Solution and Discussion

To overcome the abovementioned problem, we identify two possible options:

Option 1: to mandate that delta MSB cycle is always signalled for all LTRP in RPS.

Option 2: to modify marking process such that a LTRP in DPB shall not be marked as ‘unused for reference’ when NAL unit type of current picture is one of the non-referenced NAL unit types and TemporalId of current picture is not 0.

Both options can fix the problem illustrated in Fig. 1 as they will force delta MSB cycle for reference picture 256 is signalled in Pic 268. Fig. 2 illustrates LTRP signalling after either option 1 or option 2 is used to fix the problem.

Fig. 2 – RPS derivation and marking process with fix

Solution in option 1 is a straightforward remedy for the described problem. However, mandating delta MSB cycle for every LTRP may have significant bit-count increase compared to the current LTRP signalling design. On the other hand, solution in option 2 preserves the current design such that delta MSB cycle can be signalled only when it is really needed with the expense that the removal of LTRPs from the DPB may be delayed for a few pictures.

Considering the trade-off of the two options, the authors’ opinion is leaning toward option 2. LTRP may exists in the DPB for a long time so that spending more bits just to signal delta MSB cycle when it is not really necessary may be a high cost. Delay of a few pictures to remove LTRP from the DPB would not be a serious issue. Moreover, option 2 preserves the current concept of signalling delta MSB cycle only when it is really necessary.

# Proposed Modification

## For Option 1

The changed text is marked in bold.

In general, the required modification to accommodate option 1 is as follow:

- Remove syntax element delta_poc_msb_present_flag [i] from general slice segment header.

- Modify derivation of list of POC for LTRPs as follow:

for( i = 0, j = 0, k = 0; i < num_long_term_sps + num_long_term_pics; i++ ) {
	pocLt = PocLsbLt[ i ] + PicOrderCntVal − 	DeltaPocMSBCycleLt[ i ] * MaxPicOrderCntLsb − pic_order_cnt_lsb
	pocLt = PocLsbLt[ i ]
	if( delta_poc_msb_present_flag[ i ] )
		pocLt += PicOrderCntVal − 	DeltaPocMSBCycleLt[ i ] * MaxPicOrderCntLsb − pic_order_cnt_lsb
	if( UsedByCurrPicLt[ i ] ) {
		PocLtCurr[ j ] = pocLt
		CurrDeltaPocMsbPresentFlag[ j++ ] = delta_poc_msb_present_flag[ i ]
	} else {
		PocLtFoll[ k ] = pocLt
		FollDeltaPocMsbPresentFlag[ k++ ] = delta_poc_msb_present_flag[ i ]
	}
}
NumPocLtCurr = j
NumPocLtFoll = k

where PicOrderCntVal is the picture order count of the current picture as specified in subclause Error: Reference source not found.

NOTE 1 – A value of StRpsIdx in the range from 0 to num_short_term_ref_pic_sets − 1, inclusive, indicates that a short-term reference picture set from the active sequence parameter set is being used, where StRpsIdx is the index of the short-term reference picture set to the list of short-term reference picture sets in the order in which they are signalled in the sequence parameter set. StRpsIdx equal to num_short_term_ref_pic_sets indicates that a short-term reference picture set explicitly signalled in the slice header is being used.

For each i in the range of 0 to NumPocLtCurr − 1, inclusive, when CurrDeltaPocMsbPresentFlag[ i ] is equal to 1, it is a requirement of bitstream conformance that the following conditions apply:

- There shall be no j in the range of 0 to NumPocStCurrBefore − 1, inclusive, for which PocLtCurr[ i ] is equal to PocStCurrBefore[ j ].

- There shall be no j in the range of 0 to NumPocStCurrAfter − 1, inclusive, for which PocLtCurr[ i ] is equal to PocStCurrAfter[ j ].

- There shall be no j in the range of 0 to NumPocStFoll − 1, inclusive, for which PocLtCurr[ i ] is equal to PocStFoll[ j ].

- There shall be no j in the range of 0 to NumPocLtCurr − 1, inclusive, where j is not equal to i, for which PocLtCurr[ i ] is equal to PocLtCurr[ j ].

For each i in the range of 0 to NumPocLtFoll − 1, inclusive, when FollDeltaPocMsbPresentFlag[ i ] is equal to 1, it is a requirement of bitstream conformance that the following conditions apply:

- There shall be no j in the range of 0 to NumPocStCurrBefore − 1, inclusive, for which PocLtFoll[ i ] is equal to PocStCurrBefore[ j ].

- There shall be no j in the range of 0 to NumPocStCurrAfter − 1, inclusive, for which PocLtFoll[ i ] is equal to PocStCurrAfter[ j ].

- There shall be no j in the range of 0 to NumPocStFoll − 1, inclusive, for which PocLtFoll[ i ] is equal to PocStFoll[ j ].

- There shall be no j in the range of 0 to NumPocLtFoll − 1, inclusive, where j is not equal to i, for which PocLtFoll[ i ] is equal to PocLtFoll[ j ].

- There shall be no j in the range of 0 to NumPocLtCurr − 1, inclusive, for which PocLtFoll[ i ] is equal to PocLtCurr[ j ].

For each i in the range of 0 to NumPocLtCurr − 1, inclusive, when CurrDeltaPocMsbPresentFlag[ i ] is equal to 0, it is a requirement of bitstream conformance that the following conditions apply:

- There shall be no j in the range of 0 to NumPocStCurrBefore − 1, inclusive, for which PocLtCurr[ i ] is equal to ( PocStCurrBefore[ j ] & ( MaxPicOrderCntLsb − 1 ) ).

- There shall be no j in the range of 0 to NumPocStCurrAfter − 1, inclusive, for which PocLtCurr[ i ] is equal to ( PocStCurrAfter[ j ] & ( MaxPicOrderCntLsb − 1 ) ).

- There shall be no j in the range of 0 to NumPocStFoll − 1, inclusive, for which PocLtCurr[ i ] is equal to ( PocStFoll[ j ] & ( MaxPicOrderCntLsb − 1 ) ).

- There shall be no j in the range of 0 to NumPocLtCurr − 1, inclusive, where j is not equal to i, for which PocLtCurr[ i ] is equal to ( PocLtCurr[ j ] & ( MaxPicOrderCntLsb − 1 ) ).

For each i in the range of 0 to NumPocLtFoll − 1, inclusive, when FollDeltaPocMsbPresentFlag[ i ] is equal to 0, it is a requirement of bitstream conformance that the following conditions apply:

- There shall be no j in the range of 0 to NumPocStCurrBefore − 1, inclusive, for which PocLtFoll[ i ] is equal to ( PocStCurrBefore[ j ] & ( MaxPicOrderCntLsb − 1 ) ).

- There shall be no j in the range of 0 to NumPocStCurrAfter − 1, inclusive, for which PocLtFoll[ i ] is equal to ( PocStCurrAfter[ j ] & ( MaxPicOrderCntLsb − 1 ) ).

- There shall be no j in the range of 0 to NumPocStFoll − 1, inclusive, for which PocLtFoll[ i ] is equal to ( PocStFoll[ j ] & ( MaxPicOrderCntLsb − 1 ) ).

- There shall be no j in the range of 0 to NumPocLtFoll − 1, inclusive, where j is not equal to i, for which PocLtFoll[ i ] is equal to ( PocLtFoll[ j ] & ( MaxPicOrderCntLsb − 1 ) ).

- There shall be no j in the range of 0 to NumPocLtCurr − 1, inclusive, for which PocLtFoll[ i ] is equal to ( PocLtCurr[ j ] & ( MaxPicOrderCntLsb − 1 ) ).

- Modify RPS derivation process as follow:

The derivation process for the reference picture set and picture marking are performed according to the following ordered steps, where DPB refers to the decoded picture buffer as described in Annex Error: Reference source not found:

- The following applies:

- for( i = 0; i < NumPocLtCurr; i++ )
	if( !CurrDeltaPocMsbPresentFlag[ i ] )
		if( there is a long-term reference picture picX in the DPB [Ed. (JB): Should be made more precise. (GJS): Seems roughly OK to me.]
				with pic_order_cnt_lsb equal to PocLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picX
		else if( there is a short-term reference picture picY in the DPB
				with pic_order_cnt_lsb equal to PocLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picY
		else 
			RefPicSetLtCurr[ i ] = "no reference picture"
	else
		if( there is a long-term reference picture picX in the DPB
				with PicOrderCntVal equal to PocLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picX
		else if( there is a short-term reference picture picY in the DPB
				with PicOrderCntVal equal to PocLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picY
		else 
			RefPicSetLtCurr[ i ] = "no reference picture"

## For Option 2

The changed text is marked in bold.

The derivation process for the reference picture set and picture marking are performed according to the following ordered steps, where DPB refers to the decoded picture buffer as described in Annex Error: Reference source not found:

- The following applies:

for( i = 0; i < NumPocLtCurr; i++ )
	if( !CurrDeltaPocMsbPresentFlag[ i ] )
		if( there is a long-term reference picture picX in the DPB [Ed. (JB): Should be made more precise. (GJS): Seems roughly OK to me.]
				with pic_order_cnt_lsb equal to PocLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picX
		else if( there is a short-term reference picture picY in the DPB
				with pic_order_cnt_lsb equal to PocLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picY
		else 
			RefPicSetLtCurr[ i ] = "no reference picture"
	else
		if( there is a long-term reference picture picX in the DPB
				with PicOrderCntVal equal to PocLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picX
		else if( there is a short-term reference picture picY in the DPB
				with PicOrderCntVal equal to PocLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picY
		else 
			RefPicSetLtCurr[ i ] = "no reference picture"
																						(8-1)
for( i = 0; i < NumPocLtFoll; i++ )
	if( !FollDeltaPocMsbPresentFlag[ i ] )
		if( there is a long-term reference picture picX in the DPB
					with pic_order_cnt_lsb equal to PocLtFoll[ i ] )
				RefPicSetLtFoll[ i ] = picX
		else if( there is a short-term reference picture picY in the DPB
					with pic_order_cnt_lsb equal to PocLtFoll[ i ] )
				RefPicSetLtFoll[ i ] = picY
		else 
				RefPicSetLtFoll[ i ] = "no reference picture"
	else
		if( there is a long-term reference picture picX in the DPB
					with PicOrderCntVal to PocLtFoll[ i ] )
				RefPicSetLtFoll[ i ] = picX
			else if( there is a short-term reference picture picY in the DPB
					with PicOrderCntVal equal to PocLtFoll[ i ] )
				RefPicSetLtFoll[ i ] = picY
		else
				RefPicSetLtFoll[ i ] = "no reference picture"

- All reference pictures included in RefPicSetLtCurr and RefPicSetLtFoll are marked as "used for long-term reference".

- The following applies:

for( i = 0; i < NumPocStCurrBefore; i++ )
	if( there is a short-term reference picture picX in the DPB
			with PicOrderCntVal equal to PocStCurrBefore[ i ])
		RefPicSetStCurrBefore[ i ] = picX
	else
		RefPicSetStCurrBefore[ i ] = "no reference picture"

for( i = 0; i < NumPocStCurrAfter; i++ )
	if( there is a short-term reference picture picX in the DPB
			with PicOrderCntVal equal to PocStCurrAfter[ i ])
		RefPicSetStCurrAfter[ i ] = picX
	else
		RefPicSetStCurrAfter[ i ] = "no reference picture"	(8-2)

for( i = 0; i < NumPocStFoll; i++ )
	if( there is a short-term reference picture picX in the DPB
			with PicOrderCntVal equal to PocStFoll[ i ])
		RefPicSetStFoll[ i ] = picX
	else
		RefPicSetStFoll[ i ] = "no reference picture"

- All reference pictures in the decoded picture buffer that are not included in RefPicSetLtCurr, RefPicSetLtFoll, RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetStFoll are marked as "unused for reference".

- If TemporalId of current picture equals 0 and nal_unit_type of current picture is not one of the following, TRAIL_N, TSA_N, STSA_N, RADL_N, RASL_N, all reference pictures in the decoded picture buffer marked as “used for long-term reference” that are not included in RefPicSetLtCurr and RefPicSetLtFoll are marked as "unused for reference".

- All reference pictures in the decoded picture buffer marked as “used for short-term reference” that are not included in RefPicSetLtCurr, RefPicSetLtFoll, RefPicSetStCurrBefore, RefPicSetStCurrAfter or RefPicSetStFoll are marked as "unused for reference".

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).