| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0255 |
| --- | --- |

| Title: | AHG 9: On dependent slice segment signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Byeong Moon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130 Korea | Tel: Email: | - hendry.hendry@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes to further simplify signalling of dependent slice segment by removing the dependent_slice_segments_enabled_flag in PPS and to consider that dependent slice segment is always allowed. Furthermore, dependent_slice_segment_flag in slice segment header can be moved to before pic_parameter_set_id or be removed and a new NAL unit type for dependent slice segment is introduced.

# Introduction

Dependent slice segments are slice segments that have dependencies (i.e., parsing dependency and spatial prediction dependency) to previous coded slice segment. A dependent slice segment is useful (i.e., can be properly processed) only if all previous slice segments up to the nearest previous non-dependent slice segment are available.

Ability to identify dependent and non-dependent slice segments at system level is important and beneficial in some scenarios. For example, the ability may help a smart router to do the operations such as:

- Dropping VCL NAL units containing dependent slice segments that follow a lost / dropped VCL NAL unit as described in contribution JCTVC-K0184 [1].

- Aggregating non-dependent slice segment with all dependent slice segments that follow it.

Considering the need to facilitate easier identification whether or not the slice segment in a NAL unit is a dependent slice segment, in the 11th JCTVC meeting, it was agreed to move the signalling of dependent slice segment to earlier position. In detail, the decision was to move the dependent_slice_segment_flag before the slice_address and after pic_parameter_set_id in slice segment header and to move the dependent_slice_segments_enabled_flag to just before the sign_data_hiding_flag in PPS.

# Problem Statement

The modification of signalling of dependent slice segments by moving dependent_slice_segments_enabled_flag and dependent_slice_segment_flag to earlier position in PPS and slice segment header, respectively, does not help much to alleviate burden for system level application to identify dependent and non-dependent slice segments. The reasons are as the following.

- Memory storage requirement

The use of dependent slice segment is controlled by the enable flag that is signalled in PPS. Since there can be multiple PPS in a sequence, in order to identify whether or not a slice segment in a VCL NAL unit is a dependent slice segment, it is necessary for router implementation to allocate storage to record some information from PPS. In minimum, the implementation has to have an array that associate PPS ID with dependent_slice_segments_enabled_flag.

- Complicated parsing steps

The parsing of dependent_slice_segment_flag in slice segment header is still complicated as there are at least three conditions to be checked in sequential order, they are:

- Whether or not the slice is the first slice segment in a picture

- Whether or not the slice is in a RAP

- Whether or not dependent slice segment is allowed.

Perhaps, out of the three conditions to be checked, the last one may be of burdensome since it needs for access to the right PPS.

# Proposal and Discussion

We propose to further simplify signalling of dependent slice segments by removing the enable flag in PPS and consider that dependent slice segment is always allowed. Our proposal is based on consideration that:

- Removal of the enable flag in PPS will alleviate burden in system level application to identify dependent and non-dependent slice segments.

  - No need to allocate memory for PPS record.

  - Simplify parsing process to identify dependent and non-dependent slice segment.

- The cost of always allowing dependent slice segment is cheap.

  - Implementing dependent slice segment is not very complicated.

  - All decoders that comply with the main profile must implement dependent slice segment anyway. Thus, always enabling dependent slice segment does not add additional burden.

  - Even if encoders do not use dependent slice segment, the signalling cost is only one bit (if option 1 below is more preferred).

After removal of the enable flag in PPS, the signalling of dependent slice segment can be one of the following options:

Option 1: keep dependent_slice_segment_flag and move it before pic_parameter_set_id

Without the enable flag in PPS, dependent_slice_segment_flag can be signalled in earlier position in slice segment header as shown in the following syntax table.

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| if( RapPicFlag ) |  |
| no_output_of_prior_pics_flag | u(1) |
| if( !first_slice_segment_in_pic_flag ) |  |
| dependent_slice_segment_flag | u(1) |
| pic_parameter_set_id | ue(v) |
| if( !first_slice_segment_in_pic_flag ) { |  |
| if(dependent_slice_segments_enabled_flag) |  |
| dependent_slice_segment_flag | u(1) |
| slice_segment_address | u(v) |
| } |  |
| if( !dependent_slice_segment_flag ) { |  |
| … |  |
| // Current syntax element |  |
| … |  |
| } |  |
| … |  |
| } |  |

Option 2: assign a new NAL unit type for dependent slice segment.

Table below give possible assignment of NAL unit type with the additional of this new type for dependent slice segment. It is important to note that this assignment table may not the only possible way of assigning NAL unit type number. The new NAL unit type can be any of currently reserved type for VCL NAL unit.

Proposed NAL unit type table

| nal_unit _type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0, 1 | TRAIL_N, TRAIL_R | Coded non-dependent slice segment of a non-TSA, non-STSA trailing picture slice_segment_layer_rbsp( ) | VCL |
| 2, 3 | TSA_N, TSA_R | Coded non-dependent slice segment of a TSA picture slice_segment_layer_rbsp( ) | VCL |
| 4, 5 | STSA_N, STSA_R | Coded non-dependent slice segment of an STSA picture slice_layer_rbsp( ) | VCL |
| 6, 7 | RADL_N, RADL_R | Coded non-dependent slice segment of an RADL picture slice_layer_rbsp( ) | VCL |
| 8, 9 | RASL_N, RASL_R, | Coded non-dependent slice segment of a RASL picture slice_layer_rbsp( ) | VCL |
| 10, 12 | RSV_VCL_N10 RSV_VCL_N12 | Reserved // reserved non-RAP non-reference VCL NAL unit types | VCL |
| 11, 13 | RSV_VCL_R11 RSV_VCL_R13 | Reserved // reserved non-RAP reference VCL NAL unit types | VCL |
| 14 | RSV_VCL_R14 | Reserved // reserved non-RAP VCL NAL unit type | VCL |
| 15 | DEPENDENT_NUT | Coded dependent slice segment of a picture slice_layer_rbsp( ) | VCL |
| 16, 17, 18 | BLA_W_LP BLA_W_DLP BLA_N_LP | Coded non-dependent slice segment of a BLA picture slice_segment_layer_rbsp( ) | VCL |
| 19, 20 | IDR_W_DLP IDR_N_LP | Coded non-dependent slice segment of an IDR picture slice_segment_layer_rbsp( ) | VCL |
| 21 | CRA_NUT | Coded non-dependent slice segment of a CRA picture slice_segment_layer_rbsp( ) | VCL |
| 22, 23 | RSV_RAP_VCL22.. RSV_RAP_VCL23 | Reserved // reserved RAP VCL NAL unit types | VCL |
| 24..63 |  | For non-VCL or reserved or unspecified |  |

Variable DependentSliceSegmentFlag is defined as follow:

DependentSliceSegmentFlag = (nal_unit_type == DEPENDENT_NUT)

Modification to slice segment header is as shown in the following table:

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| if( RapPicFlag ) |  |
| no_output_of_prior_pics_flag | u(1) |
| pic_parameter_set_id | ue(v) |
| if( !first_slice_segment_in_pic_flag ) { |  |
| if(dependent_slice_segments_enabled_flag) |  |
| dependent_slice_segment_flag | u(1) |
| slice_segment_address | u(v) |
| } |  |
| if( !dependent_slice_segment_flagDependentSliceSegmentFlag) { |  |
| … |  |
| // Current syntax element |  |
| … |  |
| } |  |
| … |  |
| } |  |

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).