| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0256 |
| --- | --- |

| Title: | NonTE 4: Inter-layer Adaptive Filters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mei Guo, PoLin Lai, Shan Liu MediaTek USA 2860 Junction Ave, San Jose, CA 95134, USA Chiayang Tsai, ChingYeh Chen, Yuwen Huang, Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Email: | mei.guo@mediatek.com polin.lai@mediatek.com shan.liu@mediatek.com chiayang.tsai@mediatek.com chingyeh.chen@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents a set of methods for using inter-layer adaptive filters to enhance the performance of inter-layer texture prediction in scalable video coding. The proposed adaptive filtering methods are applied to the reconstructions of base layer to generate the inter-layer texture predictor. Two sets of experimental results are provided for (1) applying inter-layer adaptive filters in the process of upsampling including filtering the reconstructed base layer pixels; and (2) applying inter-layer adaptive filters to the reconstructed base layer pixels only. Compared with SMuC-0.1.1 anchor, average  0.6%, 1.0%, 0.2%, 0.9%, 2.6%, 0.6%, 1.4%, and 3.8% BD-rate gains are reported for scenario (1); average 0.7%, 0.9%, 0.3%, 0.8%, 2.6%, 0.6%, 1.2%, and 3.8% BD-rate gains are reported for scenario (2).

# Introduction

In scalable video coding, the inter-layer intra prediction (i.e., Intra_BL mode in SMuC-0.1.1) is an important tool to reduce the redundant texture information of enhancement layer (EL). The predictor in EL is generated by block-wise upsampling the co-located base layer (BL) reconstruction signal. In Intra_BL mode of SMuC-0.1.1, fixed 8-tap and 4-tap FIR upsampling filters are applied to luma and chroma components, respectively.

In this contribution, we propose two inter-layer adaptive filtering methods to generate the inter-layer texture predictor, as shown in Figure 1. In method 1, there are two types of pixels to be generated, i.e. the pixels corresponding to the reconstructed BL pixels (denoted as PIX_BL) and the interpolated pixels (denoted by PIX_INP), as illustrated in Figure 2. Each PIX_BL pixel is achieved by applying adaptive filter to its BL co-located pixel and its neighbors’ BL co-located pixels. Each PIX_INP pixel is produced by applying adaptive upsampling filter to neighboring PIX_BL pixels. For SNR scalability, there are no PIX_INP pixels in the predictor of EL. In method 2, adaptive filter is only used to filter the reconstructed BL pixels and produce PIX_BL pixels. The inter-layer adaptive filters are proposed to further reduce the redundant texture information of EL in inter-layer texture prediction by improving the predictors of EL. The proposed methods will be introduced in Section 2 and Section 3 will show the simulation results.

Figure 1  Illustration of Inter-layer adaptive filter.

# Algorithm Description

(a)

(b)

Figure 2 PIX_BL pixels (shaded capital letters) and PIX_INP pixels for 2x spatial scalability (a) and for 1.5x spatial scalability (b).

In this contribution, we propose two methods to apply inter-layer adaptive filters to BL reconstructions and generate the predictor of EL. They are denoted as IAF_ALL and IAF_BL and introduced in Section 2.1 and Section 2.2 respectively.

## Method 1: IAF_ALL

### Generation of PIX_BL pixels

To generate PIX_BL pixels (i.e. the pixels denoted by shaded capital letters in Figure 2) a 5x5 diamond shape filter is proposed, as shown in Fig. 3. This filter is 2-D non-separable, represented by seven distinct filter coefficients {hi } i=1, …, 7. For example, pixel “C3” is filtered with Equation (1), where XBL denotes the co-located BL pixel of X.

C3=(A3BL+E3BL)*h1+(B2BL+D4BL)*h2+(B4BL+D2BL)*h3+(B3BL+D3BL)*h4+(C1BL+C5BL)*h5+(C2BL+C4BL)*h6+C3BL*h7    (1)

Figure 3 5x5 diamond shape filter to generate PIX_BL pixels

In order to accommodate diversified characteristics within one picture, we classify the PIX_BL pixels into 16 categories based on their co-located BL pixels. One set of filter coefficients is derived and applied to the pixels in each category. In the current implementation, we utilize the block-based classification method (denoted as BLK) and region-based classification method (denoted as REG) in HM6.1 [1].

## 2.1.2Generation of PIX_INP pixels

To generate PIX_INP pixels, two 2D non-separable filter shapes are proposed for the pixels at different interpolated positions. In 2x spatial scalability, the pixels at positions “a” and “b” are generated with the filter shape in Figure 4(a) while the pixels at position “c” are generated with the filter shape in Figure 4(b). In 1.5x spatial scalability, the pixels at positions “a”, “b”, “c” and “f” are produced with the filter shape in Figure 4(a) while the pixels at positions “d”, “e”, “g” and “h” are produced with the filter shape in Figure 4(b). 18 distinct coefficients {ki } i=1, …, 18 are user for filter shape in Figure 4(a) and 24 distinct coefficients {ki } i=1, …, 24 are used for filter shape in Figure 4(b).

- (b)

Figure 4 2D non-separable filters to generate PIX_INP pixels

## 2.1.3 Derivation of filter coefficients

For spatial scalability (2X and 1.5X), before calculating the filter coefficients at encoder side, a low-pass filter is applied to original EL picture to reduce noises. In the current implementation, we use a 2D separable 7-tap filter with the coefficients [5, -20, 38, 82, 38, -20, 5]. After that, all adaptive filter coefficients are achieved by minimizing the difference between the generated pixel values and the low-pass filtered original pixels at EL. For SNR scalability, the low-pass filter is omitted and the difference between the generated pixels and the original pixels at EL is minimized to derive the filter coefficients.

In 1.5x spatial case, some PIX_BL pixels do not have collocated pixels at EL, such as pixel D5, E4 and E5 in Figure 2(b). Those pixels are not involved in the derivation of filter coefficients, but they are processed by adaptive filtering and the generated pixels are used to produce PIX_INP pixels.

## 2.1.4 Entropy coding of filter coefficients

For each set of filter coefficients, the last coefficient is predicted with the other coefficients within the same set. After the prediction, they are coded with exponential golomb codes. Besides, when the generation of PIX_BL pixels uses classification, the number of categories achieved with merge process and the information about whether each category mergers with the previous one are fixed-length coded.

## 2.1.5 Picture-level filter decisions and signaling

Two filter decisions are made for luma component: the one on whether adaptive filtering is used to generate both PIX_BL and PIX_INP pixels and the other one on whether adaptive filtering is used to only generate PIX_BL pixels. If the second decision is true, PIX_BL pixels are generated with adaptive filters and PIX_INP pixels are generated with PIX_BL pixels using fixed upsampling filters in SMuC-0.1.1. At most one of the two decisions could be true. If both of them are false, BL reconstructed pixels are directly used as the co-located PIX_BL pixels and PIX_INP pixels are generated with PIX_BL pixels using fixed upsampling filters in SMuC-0.1.1. Two flags are delivered to signal the decisions for the luma component of each picture. The chroma component only requires one decision: the one on whether adaptive filtering is used to only generate PIX_BL pixels.

All above decisions are made based on the distortion (calculated with SSD measure) and estimated bit rates for filter coefficients, classification information and decisions.

## Method 2: IAF_BL

The method 2 is similar to method 1 with the difference that method 2 only applies the inter-layer adaptive filter to generate PIX_BL pixels and always uses fixed upsampling filters to generate PIX_INP pixels. Thus, only one decision on whether adaptive filtering is used to generate PIX_BL pixels is made for each component of each picture. For each component of each picture one flag is delivered to signal the decision.

The decisions are also made based on the distortion (calculated with SSD measure) and estimated bit rates for filter coefficients, classification information and decisions.

# Results

The proposed methods are implemented on top of SMuC-0.1.1, which is also used as the anchor of simulation. We tested the proposed two methods: IAF_ALL and IAF_BL. In each method, two classification methods for PIX_BL pixels (i.e., BLK and REG) are both evaluated. The simulation results are given in Table 1~4.

Table 1 Coding performance of IAF_ALL with BLK classification

Table 2 Coding performance of IAF_ALL with REG classification

Table3 Coding performance of IAF_BL with BLK classification

Table 4 Coding performance of IAF_BL with REG classification

# Conclusions

This contribution proposed a set of methods for using inter-layer adaptive filters to enhance the performance of inter-layer texture prediction in scalable video coding. Two sets of experimental results are provided for (1) applying inter-layer adaptive filters in the process of upsampling including filtering the reconstructed base layer pixels; and (2) applying inter-layer adaptive filters to the reconstructed base layer pixels only. Compared with SMuC-0.1.1 anchor, average 0.6%, 1.0%, 0.2%, 0.9%, 2.6%, 0.6%, 1.4%, and 3.8% BD-rate gains are reported for scenario (1); average 0.7%, 0.9%, 0.3%, 0.8%, 2.6%, 0.6%, 1.2%, and 3.8% BD-rate gains are reported for scenario (2).

# References

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 6,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1003, Feb. 2012.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).