| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0258 |
| --- | --- |

| Title: | Inter layer prediction indication flags and pictures management |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Byeong Moon Jeon Do-Kyoung Kwon Madhukar Budagavi Minhua Zhou | Tel: Email: | hendry.hendry@lge.com bm.jeon@lge.com d-kwon@ti.com madhukar@ti.com zhou@ti.com |
| Source: | LG Electronics and Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution proposes two flags so called ilp_flag and ilp_ref_flag to be added in slice header. The ilp_flag indicates whether or not the current picture is needed for inter-layer prediction whereas the ilp_ref_lag indicates whether or not the current picture is needed for inter prediction by any picture whose ilp_flag equals 1. It is claimed that the two flags helps to determine whether a picture should be decoded or not given a target decoding layer which will lighten decoding burden. In addition, even though the pictures whose ilp_flag equal 1 are discarded, the two flags guarantee the reliable decoding when they are used for reference picture list construction process in the non-target layer.

# Introduction

In the 11th JCT-VC meeting two flags for inter-layer prediction were proposed in JCTVC-K0264 [1]. This contribution provides the results of further study of the two flags. The proposed flags are:

- ilp_flag to indicate whether or not the current slice / picture is needed for inter-layer prediction, which has been proposed in [2] as well.

- ilp_ref_flag to indicate whether or not the current slice / picture is needed for inter prediction by other pictures whose ilp_flag equal 1.

The motivation for having the two flags is as follow:

- As a way to trade off coding efficiency and memory bandwidth in multi-loop HEVC scalable extension, not all pictures are used for inter-layer prediction. Thus, given a particular target layer for decoding, coded pictures that are not needed for decoding pictures in the target layer should be clearly indicated so that they can be easily discarded and not to be decoded which saves computing resource.

- Given that a picture may or may not be decoded depending on the given target layer, reference picture list construction may be more complicated since it is necessary to differentiate the reference picture list construction process for a picture / slice when it is in the target layer and when it is not in the target layer (i.e., its layer is less than the target layer). Using different RPS for target and non-target layer does not seem to be a desired solution as it may require more bit count for signalling. The two flags may be utilized so that the same RPS can be used for reference picture list construction pictures in both target and non-target layers.

Figure 1 illustrates an example of a bitstream with three spatial layers (i.e., BL and EL1) with four temporal layers in each spatial layer. The highest layer (i,e., EL1) is coded following random access configuration in the common test condition (CTC) structure whereas the base layer (i.e., BL) are coded with slightly modified configuration to reduce memory / DPB size requirement. Only pictures of temporal layer 0 and 1 in the BL are used for inter-layer prediction so that they DPB requirement is only 3 when the target layer is EL1. When the target layer is EL1, the pictures in BL whose both ilp_flag and ilp_ref_flag equal 0 are not needed so that that they can be removed from the coded bitstream. The pictures to be decoded when EL1 is a target layer are: (00 01) (80 81) (40 41) (21) (11) (31) (61) (51) (71) (160 161) (120 121) (102) (92) (112) (142) (131) (151).

Figure 1 – Example of use of ilp_flag and ilp_ref_flag.

# Proposal

## Syntax and semantic of the flags

The syntax and semantic of the proposed flags are as follow:

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| ilp_flag | u(1) |
| ilp_ref_flag | u(1) |
| … |  |
| } |  |

ilp_flag indicates that current slice may be used for inter-layer prediction. It is requirement of the bitstream that the value of this flag shall be the same in all slice segment headers of a coded picture.

ilp_ref_flag indicates that current slice may be referred / used for inter prediction by other picture(s) that are needed for inter-layer prediction. It is requirement of the bitstream that the value of this flag shall be the same in all slice segment headers of a coded picture.

In the 11th JCTVC meeting, it was agreed that flags for HEVC extension may be added in slice header as part of ‘slice_reserved_undetermined_flag [i]’. However, due to its possible impact to reference picture list construction process as described in Section 2.3, the authors recommend that the flags are not considered as part of the current “slice reserved undetermined flags”.

## Impact to decoding process

The two proposed flags can be used to determine pictures / slices that are needed to be decoded and kept in the DPB. In addition, they can also be utilized for removing reference pictures in the DPB that are not needed.

Let LayerID denotes layer id (e.g., base layer, enhancement layer 1, etc) of a picture and TargetLayer denotes the target layer for decoding / display. The following should be applied to decoding process:

- Given a particular TargetLayer, reconstructed picture of coded picture shall not be kept in memory if all the following are true:

  - LayerID < TargetLayer, and

  - ilp_flag is false, and

  - ilp_ref_flag is false.

- Given a particular TargetLayer, after decoding of a picture with POC poc_curr and LayerID curr_layer_id (i.e., curr_layer_id ≤ TargetLayer), any reference picture in memory shall be removed if all the following is true:

  - POC of the reference picture is the same as poc_curr, and

  - LayerID of the reference picture is less than curr_layer_id, and

  - ilp_ref_flag of the reference picture is false.

## Impact to reference pictures list construction

When pictures that are not needed for inter-layer prediction are removed / not decoded, reference picture list for pictures in the same POC in target layer and non-target layer will be different. Figure 2 illustrates the same example shown in Figure 1 in the decoder side, where the pictures that are not needed for inter-layer prediction are removed in BL. As illustrated, the temporal prediction for picture in temporal layer 0 (i.e., L0) in BL are different from that of in EL1, which implies that we may need to have different RPSs for BL and EL1. However, by utilizing the proposed ilp_flag and ilp_ref_flag, a single RPS may be used for all layers.

Figure 2 – Same example with Figure 1, in which the pictures not needed for inter-layer prediction are removed in the decoder side.

We propose that reference picture lists of a slice are constructed as follow:

- If the ilp_flag of the slice equals 0, the reference picture lists for the slice are constructed based on RPS only. That is, no change to the current reference picture lists construction

- If the ilp_flag of the slice equals 1, the reference picture lists for the slice are constructed by including reference pictures that:

  - Have ilp_ref_flag equals 1, and

  - Are included in the RPS for that slice.

The required modification to the reference picture lists construction can be described as follow (removed text from current specification is indicated by striken red highlighted text and the newly added text are indicated by yellow highlighted):

Let IlpRefList	denotes	list of ilp_ref_flag of pictures and IlpRefList [x] denotes ilp_ref_flag of picture with POC x.

The reference picture lists construction is as the following:

At the beginning of the decoding process for each slice, the reference picture list RefPicList0, and for B slices RefPicList1, are derived as follows.

- The variable NumRpsCurrTempList0 is set equal to Max( num_ref_idx_l0_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp0 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList0 ) {

for( i = 0;  i < NumPocStCurrBefore && rIdx < NumRpsCurrTempList0; rIdx++ , i++ ) 	
		if (ilp_flag == 0 || (ilp_flag == 1 && IlpRefList[RefPicSetStCurrBefore[ i ]]))
			RefPicListTemp0[ rIdx++ ] = RefPicSetStCurrBefore[ i ] 
	for( i = 0;  i < NumPocStCurrAfter && rIdx < NumRpsCurrTempList0; rIdx++ , i++ ) 	
		if (ilp_flag == 0 || (ilp_flag == 1 && IlpRefList[RefPicSetStCurrAfter [ i ]]))
			RefPicListTemp0[ rIdx++ ] = RefPicSetStCurrAfter[ i ]
	for( i = 0; i < NumPocLtCurr && rIdx < NumRpsCurrTempList0; rIdx++ , i++ )
		if (ilp_flag == 0 || (ilp_flag == 1 && IlpRefList[RefPicSetLtCurr [ i ]]))
			RefPicListTemp0[ rIdx++ ] = RefPicSetLtCurr[ i ]
}

- The list RefPicList0 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l0_active_minus1; rIdx++)			
		RefPicList0[ rIdx ] = ref_pic_list_modification_flag_l0 ? RefPicListTemp0[ list_entry_l0[ rIdx ] ] :
																			RefPicListTemp0[ rIdx ]

- When the slice is a B slice, the variable NumRpsCurrTempList1 is set equal to Max( num_ref_idx_l1_active_minus1 + 1, NumPocTotalCurr) and the list RefPicListTemp1 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList1 ) {
	for( i = 0; i < NumPocStCurrAfter && rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		if (ilp_flag == 0 || (ilp_flag == 1 && IlpRefList[RefPicSetStCurrAfter [ i ]]))
			RefPicListTemp1[ rIdx++ ] = RefPicSetStCurrAfter[ i ]
	for( i = 0;  i < NumPocStCurrBefore && rIdx < NumRpsCurrTempList1; rIdx++, i++ ) 	
		if (ilp_flag == 0 || (ilp_flag == 1 && IlpRefList[RefPicSetStCurrBefore [ i ]]))
			RefPicListTemp1[ rIdx++ ] = RefPicSetStCurrBefore[ i ]
	for( i = 0; i < NumPocLtCurr && rIdx < NumRpsCurrTempList1; rIdx++, i++ )
		if (ilp_flag == 0 || (ilp_flag == 1 && IlpRefList[RefPicSetLtCurr [ i ]]))
			RefPicListTemp1[ rIdx++ ] = RefPicSetLtCurr[ i ]
}

- When the slice is a B slice, the list RefPicList1 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l1_active_minus1; rIdx++)			
		RefPicList1[ rIdx ] = ref_pic_list_modification_flag_l1 ? RefPicListTemp1[ list_entry_l1[ rIdx ] ] :
																			RefPicListTemp1[ rIdx ]

# References

- D.-K. Kwon, M. Budagavi and M. Zhou, “Hierarchical inter-layer prediction in multi-loop scalable extension of HEVC,” Document JCTVC-K00264, October 2012.

- J. Xu, “AHG10: Selective inter-layer prediction signaling for HEVC scalable extension,” Document JCTVC-J0239, October 2012.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).