| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0260 |
| --- | --- |

| Title: | NonTE 5.1: Inter-layer Intra Mode Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mei Guo, Shan Liu, Shawmin Lei 2860 Junction Ave, San Jose, CA 95134, USA Joonyoung Park, Jungsun Kim, Byeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea | Email: Email: | mei.guo@mediatek.com shan.liu@mediatek.com shawmin.lei@mediatek.com jy.park@lge.com jungsun.kim@lge.com |
| Source: | MediaTek Inc. and LG Electronics |  |  |

_____________________________

# Abstract

The contribution presents methods to exploit the correlation of intra modes from different layers in scalable video coding. The intra mode of the collocated block at base layer is used to predict the intra mode at enhancement layer so that the bit rate used to deliver intra modes at enhancement layer could be reduced. Three methods to generate the list of most probable modes are introduced in the contribution. It is reported that 0.4% and 0.2% coding gain can be achieved for overall bit-stream (i.e., EL+BL) in AI2x and AI1.5x cases respectively.

# Introduction

- During the intra mode coding of luma component in HM-9[1], three most probable modes are derived for each luma PU from the intra modes of the neighboring PUs (i.e., the PU on the left of the current PU and the one on top of the current PU). If the current intra mode is equal to any of the most probable modes, a flag prev_intra_pred_flag equal to 1 is transmitted followed by the index of that most probable mode in the most probable mode list; otherwise, the remaining mode is transmitted following the flag prev_intra_pred_flag equal to 0. Thus, the selection of most probable modes influences the efficiency of intra mode coding.

In this contribution, we propose to predict the intra mode at enhancement layer with the intra mode of the collocated block at base layer in order to exploit the correlation of intra modes from different layers. The proposed methods will be introduced in Section 2 and Section 3 will show the simulation results.

# Algorithm Description

- In this contribution, three most probable modes (denoted as MPM[ x ] with x=0…2) are used at enhancement layer and they are derived for each luma PU from the intra mode of the corresponding unit at base layer (denoted as modeBL), the intra mode of neighboring PU on the left of the current PU (denoted as modeLeft) and the one of the neighboring PU on top of the current PU  (denoted as modeTop). The intra mode at base layer, which corresponds to the 4x4 unit A (as shown in Figure 1) within the current unit at enhancement layer, is derived as modeBL. When left PU or top PU is coded with Intra_BL mode, modeLeft or modeTop is also derived from their corresponding unit at base layer. Three methods to generate the list of most probable modes are described as follows.

Figure 1.  Illustration of a selected 4x4 unit inside the current unit larger than 4x4

## Method 1

For each of modeBL, modeLeft and modeTop, if it is available and is not DC/Planar, it is put into MPM according to the priority (i.e., modeBL>modeLeft>modeTop) with the repetition removed. When three modes are available in MPM, {modeBL, modeLeft, modeTop} are directly used to predict the current intra mode at enhancement layer; otherwise, MPM is further generated as follows. Denote the number of modes available in MPM as numAvai.

- numAvai = 2, MPM = {MPM[0], MPM[1], DC}

- numAvai = 1, MPM = {MPM[0], MPM[0]-1, MPM[0]+1}

- numAvai = 0, MPM = {DC, VER, HOR}

## Method 2

When modeBL is available and is not DC/Planar, MPM[0] is modeBL and the others are adjacent two directions of modeBL. Otherwise, the same process to derive MPM for base layer is applied. In brief, MPM are decided as follow:

- If modeBL is available and is not DC/Planar, MPM = {modeBL, modeBL+1, modeBL-1}

- Otherwise

  - modeLeft!=modeTop, none of modeLeft and modeTop is Planar, MPM = {modeLeft, modeTop, Planar}

  - modeLeft!=modeTop, one of modeLeft and modeTop is Planar, MPM = {modeLeft, modeTop, DC}

  - modeLeft!=modeTop, all of modeLeft and modeTop are DC/Planar, MPM = {modeLeft, modeTop, VER}

  - modeLeft = modeTop = DC/Planar, MPM = {Planar, DC, VER}

  - modeLeft = modeTop != DC/Planar, MPM = {modeLeft, modeLeft+1, modeLeft-1}

The process from A to E is same as that at base layer.

## Method 3

If modeBL is available, the MPM are derived as {modeBL, modeBL+1, modeBL-1}. Otherwise, if only one of two candidates, modeLeft and modeTop, is available and has certain direction or both are available and have same direction, the MPM are derived as {modeX, modeX+1, modeX-1} where modeX implies the available mode. If both of modeLeft and modeTop are available and have different direction the MPM are derived as { modeLeft, modeTop, DC}. The process in brief is as follow:

- If modeBL is available and is not DC/Planar, MPM = {modeBL, modeBL+1, modeBL-1}

- Otherwise

  - modeLeft!=modeTop, none of modeLeft and modeTop is DC/Planar, MPM = {modeLeft, modeTop, DC}

  - modeLeft = modeTop != DC/Planar, MPM = {modeLeft, modeLeft+1, modeLeft-1}

  - otherwise , MPM = {DC, VER, HOR}

# Results

The proposed methods are implemented on top of SMuC-0.1.1 provided. The detailed results are described in the accompanied excel files. The anchor of the simulation is SHVC 0.1.1.The experimental results for three methods are shown in Table 1, Table 2, and Table 3 respectively.

Table 1 Coding performance of method 1

Table 2 Coding performance of method 2

Table 3 Coding performance of method 3

There exists parsing dependency in the bit-stream of enhancement layer, since the decoded intra mode is required in mode dependent coefficient scan (MDCS). Thus, we further test the proposed methods when MDCS is disabled to remove the parsing dependency. The results are shown in Table 4, Table5, and Table 6 respectively.

Table 4 Coding performance of method 1 when MDCS is disabled

Table 5 Coding performance of method 2 when MDCS is disabled

Table 6 Coding performance of method 3 when MDCS is disabled

# Conclusions

This contribution proposed to use the intra mode of corresponding unit at base layer to predict the intra mode at enhancement layer. The proposed methods achieve 0.4% and 0.2% coding gain overall bit-stream (i.e., EL+BL) in AI2x and AI1.5x cases respectively.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 9,” Document of Joint Collaborative Team on Video Coding, JCTVC-K1003, Oct. 2012.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).