| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0261 |
| --- | --- |

| Title: | Non-TE2: Cross-check report of JCTVC-L0174 Inter-layer reference picture placement |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Liwei Guo 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-845-7532 liweig@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution verified the methods and the BD-rate results of JCTVC-L0174 which is proposed by Dolby and InterDigital.

# Introduction

JCTVC-L0174 describes two methods targeting at reducing the complexity of RefIdx approach based inter-layer. Both of these methods only involve encoder-side change.

# Experimental results

The JCTVC-L0174 software provided by the proponents have been used to generate the results and compared with anchor results generated with using the TE2 tools from InterDigital with setting2 + ZeroMV [2].  The test conditions defined in TE2 test description was used [3]. Encoding and decoding time were obtained from a homogeneous cluster.  It was confirmed that for both Method1 and Method2, the BD-rate results exactly match to the results provided by the proponents.

Table 1 Results of Method 1

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.1% | -0.1% |  |  |  |
| Class B | 0.1% | -0.1% | -0.1% | 0.2% | -0.3% | -0.3% |
| Overall (EL+BL) | 0.1% | -0.1% | -0.1% | 0.2% | -0.3% | -0.3% |
| Overall (EL) | 0.3% | -0.2% | -0.3% | 0.8% | -1.2% | -1.1% |
| Enc Time[%] | 52.7% |  |  | 65.1% |  |  |
| Dec Time[%] | 101.2% |  |  | 100.7% |  |  |

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.4% | -0.5% |  |  |  | 0.0% | -0.3% | -0.5% |
| Class B | -0.1% | -0.7% | -0.9% | 0.0% | -0.6% | -0.8% | 0.0% | -0.8% | -1.1% |
| Overall (EL+BL) | -0.1% | -0.6% | -0.8% | 0.0% | -0.6% | -0.8% | 0.0% | -0.7% | -0.9% |
| Overall (EL) | -0.1% | -1.0% | -1.3% | 0.1% | -1.2% | -1.6% | 0.0% | -1.2% | -1.6% |
| Enc Time[%] | 100.7% |  |  | 102.5% |  |  | 94.7% |  |  |
| Dec Time[%] | 100.5% |  |  | 100.7% |  |  | 101.0% |  |  |

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.3% | -0.3% |  |  |  | 0.0% | -0.2% | -0.2% |
| Class B | -0.1% | -0.3% | -0.4% | -0.1% | -0.3% | -0.2% | -0.1% | -0.3% | -0.3% |
| Overall (EL+BL) | -0.1% | -0.3% | -0.4% | -0.1% | -0.3% | -0.2% | -0.1% | -0.3% | -0.3% |
| Overall (EL) | -0.1% | -0.5% | -0.6% | -0.1% | -0.7% | -0.5% | -0.1% | -0.5% | -0.5% |
| Enc Time[%] | 102.3% |  |  | 103.2% |  |  | 99.9% |  |  |
| Dec Time[%] | 100.2% |  |  | 102.1% |  |  | 100.6% |  |  |

Table 2 Results of Method 2

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% |  |  |  |
| Class B | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Overall (EL+BL) | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |
| Overall (EL) | -0.1% | -0.1% | -0.2% | 0.0% | -0.1% | -0.3% |
| Enc Time[%] | 78.1% |  |  | 90.0% |  |  |
| Dec Time[%] | 102.9% |  |  | 101.0% |  |  |

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.4% | -0.5% |  |  |  | 0.0% | -0.3% | -0.4% |
| Class B | -0.1% | -0.6% | -0.8% | 0.0% | -0.5% | -0.7% | -0.1% | -0.9% | -1.1% |
| Overall (EL+BL) | -0.1% | -0.5% | -0.8% | 0.0% | -0.5% | -0.7% | 0.0% | -0.7% | -0.9% |
| Overall (EL) | -0.2% | -0.9% | -1.2% | -0.1% | -1.0% | -1.5% | -0.1% | -1.2% | -1.4% |
| Enc Time[%] | 100.7% |  |  | 102.5% |  |  | 94.7% |  |  |
| Dec Time[%] | 100.5% |  |  | 100.7% |  |  | 101.0% |  |  |

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.3% |  |  |  | 0.0% | -0.1% | -0.2% |
| Class B | -0.1% | -0.2% | -0.4% | 0.0% | -0.2% | -0.3% | 0.0% | -0.3% | -0.2% |
| Overall (EL+BL) | -0.1% | -0.2% | -0.4% | 0.0% | -0.2% | -0.3% | 0.0% | -0.3% | -0.2% |
| Overall (EL) | -0.1% | -0.4% | -0.6% | -0.1% | -0.6% | -0.7% | -0.1% | -0.4% | -0.4% |
| Enc Time[%] | 103.1% |  |  | 103.4% |  |  | 101.7% |  |  |
| Dec Time[%] | 100.9% |  |  | 101.3% |  |  | 99.8% |  |  |

# References

- P. Yin, T. Lu, T. Chen (Dolby), X.Xiu, Y.Ye (InterDigital), “Non-TE2: Inter-layer reference picture placement”, JCTVC-L0174, Geneva, CH, Jan., 2013.

- X. Xiu, Y. He, Y. He, Y. Ye (InterDigital), “TE2: Results of test 3.2.1 on inter-layer reference picture placement”, JCTVC-L0051, Geneva, CH, Jan., 2013.

- L. Guo, Y. He, D. Kown, J. Zan, H. Lakshman, J. Kang (TE Coordinators), “Description of Tool Experiment A2: Inter-layer Texture Prediction Signaling in SHVC”, JCTVC-K1102, Shanghai, CN, Oct., 2012.