| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0263 |
| --- | --- |

| Title: | AHG 9: On inter-layer dependency signalling in VPS extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Byeong Moon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130 Korea | Tel: Email: | - hendry.hendry@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

It is observed that the agreed signalling of layer dependency in VPS extension described in JCTVC-K1007 is still not tight enough so that it is for a layer to have reference to higher layer. Such referencing may not be desired for application that may extract bitstream based on layer information. This contribution proposes some modifications to syntax and semantic of ref_layer_id [i][j] to ensure that all coded pictures in a layer can only refer to pictures in lower layer.

# Introduction and Problem Statement

Inter-layer dependency information can be of useful for bitstream extraction when given a target layer for processing. Figure 1 illustrates a simple bitstream extractor who takes an input stream with multiple layers and a target layer to produce an output stream by removing layers that are higher than the given target layer.

Figure 1 – Illustration of bitstream extractor.

Operation of bitstream extractor is only possible if the following property is met:

“Any coded picture in layer x, x > 0, can have coding dependency to reference pictures that exists only in layer y that is smaller than x (i.e., 0 ≤ y < x)”

The inter-layer dependency is signaled in the extension of Video Parameter Set (VPS) as tabulate in the table below.

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| // layer dependency |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ]; j++ ) |  |
| ref_layer_id[ i ][ j ] | u(6) |
| } |  |
| } |  |

num_direct_ref_layers [i] specifies number of layers that pictures in layer i depend on.

ref_layer_id [i][j] specifies the identification of the j-th layer that layer i depend on.

vps_max_layers_minus1 is defined is defined in in VPS as tabulate in the following table.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| vps_max_num_layers_minus1 | u(6) |
| … |  |
| vps_extension( ) |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

ref_layer_id [i][j] refers to the ‘nuh_layer_id’ defined in NAL unit header.

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(6) |
| nuh_layer_id | u(6) |
| nuh_temporal_id_plus1 | u(3) |
| } |  |

Current inter-layer dependency concept and signaling do not have any restriction to ensure that the above bitstream property is applied so that any bitstream can be extracted for any given target layer. This contribution proposes some ways to enforce such restriction.

# Proposal

To enforce the restriction such that all coded pictures in a layer can only refer to pictures in lower layer, the following modification options to current inter-layer dependency signalling are proposed.

Option 1: Modify semantic of ref_layer_id [i][j] syntax element

The inter-layer dependency signaled in the extension of Video Parameter Set (VPS) is as the following.

Note that there is no modification to the syntax table but the semantic of ref_layer_id [i][j] is modified.

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| // layer dependency |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ]; j++ ) |  |
| ref_layer_id[ i ][ j ] | u(6) |
| } |  |
| } |  |

num_direct_ref_layers [i] specifies number of layers that pictures in layer i depend on. For any possible value of i, num_direct_ref_layers [i] shall be less than or equals i.

ref_layer_id [i][j] specifies the identification of the j-th layer that layer i depend on. For any possible value of i, ref_layer_id [i][j] shall be smaller than i.

Option 2: Modify syntax of ref_layer_id [i][j]

To enforce the above restrictions, instead of signaling the identification of the reference layer, delta between current layer and reference layer is signaled.

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| // layer dependency |  |
| num_direct_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_direct_ref_layers[ i ]; j++ ) |  |
| delta_ref_layer_id_minus1[ i ][ j ] | u(6) |
| } |  |
| } |  |

num_direct_ref_layers [i] specifies number of layers that pictures in layer i depend on. For any possible value of i, num_direct_ref_layers [i] shall be less than or equals i.

delta_ref_layer_id_minus1 [i][j] plus 1 specifies the difference between current layer identification and reference layer identification.

The variable RefLayerId [i][j] is computed as follow:

RefLayerId [i][j] = i – delta_ref_layer_id [i][j] - 1

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).