| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0265 |
| --- | --- |

| Title: | On interpolation filter for Generalized Residual Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba, Tomoyuki Yamamoto Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 tsukuba.takeshi@sharp.co.jp yamamoto.tomoyuki@sharp.co.jp tomohiro.ikai@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution proposes an interpolation filter for Generalized Residual Prediction to minimize the complexity with improved coding efficiency. In the proposal, 2 or 4 tap filter is used depending on the slice type. The proposed method is implemented on TE3 4.6.3 software. It is reported that the BD-rate (EL+BL) changes compared to SMuC-0.1.1 are -2.1%, -3.4%, -2.1%, -2.4%, -3.6% and -2.4% for RA 2x, RA 1.5x, RA SNR, LP 2x, LP 1.5x and LP SNR cases respectively. It is also reported that the BD-rate (EL+BL) changes compared to TE3 4.6.3, which uses 8 tap filters for luma and 4 tap filters for chroma, are -0.4 %, -0.9 %, -0.5 %, -0.1 %, -0.2 % and 0.5 % for RA 2x, RA 1.5x, RA SNR, LP 2x, LP 1.5x and LP SNR cases respectively.

# Introduction

In the last meeting, variants of Generalized Residual Prediction (GRP) were proposed in the CfP responses, K0036 [1], K0040 [2] and K0041 [3].

The GRP proposed in [2], named as Base-layer enhanced motion compensation prediction (BEMCP), is summarized as follows:

- Generate residual by subtracting base layer MC result with the enhancement layer motion parameters from base layer reconstruction.

- Add the regular prediction and the weighted generated residual (predicted residual) to get final prediction

- Signal the weight (0, 0.5 or 1) on PU level

Specifically, the generating residual process is formulated as:

RLX = UP ( RecBL) – MC ( UP( RefLX,BL), MVLX,EL )		-(eq.1)

where RLX is the generated residual, RecBL is the base layer reconstructed image, RefLX,BL is the base layer reference image, MVLX,EL is the motion parameter in the enhancement layer for PLX, UP( . ) is an up-sampling operator, and MC( . , MV) is a motion compensation operator using a motion parameter MV.

The weighting and adding process to generate uni-prediction PLX’ and bi-prediction PBI’ is formulated as:

PLX’ = PLX + w * RLX					-(eq.2)

PBI’ = ((PL0 + PL1)/2) + w * ( ( RL0 + RL1)/2 )			-(eq.3)

# Motivation

According to TE3 document [4], the original proposal (BEMCP) uses 8 tap for luma and 4 tap for chroma for residual generation in eq.1.

Because the complexity of interpolation process accounts for most of the complexity of generalized residual prediction, it is necessary to simplify the process.

Besides we found out that the simplified interpolation filter could achieve coding efficiency improvement. We also found that the characteristics of the filter should depend on slice type.

# Proposed method

We propose to use simplified interpolation filters instead of 8-tap filter for luma and 4-tap filter for chroma in eq.1. The interpolation filters are selected depending on the slice type as shown Table 1.

Table.1: Interpolation filter selection for each slice type

| Slice type | Interpolation filter |  |
| --- | --- | --- |
|  | Luma | Chroma |
| P-slice | 4-tap DCT-IF | 2-tap (bilinear) |
| B-slice | 2-tap (bilinear) | 2-tap (bilinear) |

Table 2, 3 and 4 show the coefficients for each interpolation filter. Note that 4-tap filter for luma in Table 2 is the same as 4-tap DCT-IF for chroma in HEVC.

Table.2: Interpolation filter for luma in the case of P-slice

| Phase offset | Filter coefficients |
| --- | --- |
| 0/4 | [ 0, 64, 0, 0 ] // 64 |
| 1/4 | [-4, 54, 16, -2 ] // 64 |
| 2/4 | [-4, 36, 36, -4 ] // 64 |
| 3/4 | [-2, 16, 54, -4 ] // 64 |

Table.3: Interpolation filter for luma in the case of B-slice

| Phase offset | Filter coefficients |
| --- | --- |
| 0/4 | [ 64, 0 ] // 64 |
| 1/4 | [48, 16] // 64 |
| 2/4 | [32, 32] // 64 |
| 3/4 | [16, 48] // 64 |

Table.4: Interpolation filter for chroma in the case of P/B-slice

| Phase offset | Filter coefficients |
| --- | --- |
| 0/8 | [ 64, 0 ] // 64 |
| 1/8 | [ 56, 8 ] // 64 |
| 2/8 | [ 48, 16] // 64 |
| 3/8 | [ 40, 24] // 64 |
| 4/8 | [ 32, 32] // 64 |
| 5/8 | [ 24, 40] // 64 |
| 6/8 | [ 16, 48] // 64 |
| 7/8 | [ 8, 56] // 64 |

# Experimental results

The proposed method is implemented on TE3 4.6.3[5] software and evaluation is done based on the test condition described in TE3 document [4]. The experimental results are shown in Table 5 and 6. Table 5 and 6 show the performance of the proposal compared to smuc-0.1.1 and TE3 4.6.3 respectively.

Table 5: Performance of the proposal (ref. SMuC-0.1.1)

Table 6: Performance of the proposal (ref. TE3 4.6.3)

# Conclusion

This contribution proposes an interpolation filter for Generalized Residual Prediction to minimize complexity with improved coding efficiency. In the proposal, 2 or 4 tap filter is used depending on the slice type. The proposed method is implemented on TE3 4.6.3 software. It is reported that the BD-rate (EL+BL) changes compared to SMuC-0.1.1 are -2.1%, -3.4%, -2.1%, -2.4%, -3.6% and -2.4% for RA 2x, RA 1.5x, RA SNR, LP 2x, LP 1.5x and LP SNR cases respectively. It is also reported that the BD-rate (EL+BL) changes compared to TE3 4.6.3, which uses 8 tap filters for luma and 4 tap filters for chroma, are -0.4 %, -0.9 %, -0.5 %, -0.1 %, -0.2 % and 0.5 % for RA 2x, RA 1.5x, RA SNR, LP 2x, LP 1.5x and LP SNR cases respectively. It is proposed to investigate this method in TE/CE as a candidate coding tool for SHVC.

# Reference

- J. Chen, K. Rapaka, X. Li, V. Seregin, L. Guo, M. Karczewicz, G. Van der Auwera, J. Sole, X. Wang, C. J. Tu, Y. Chen, “Description of scalable video coding technology proposal by Qualcomm (configuration 2)”, JCTVC-K0036, Shanghai, China, Oct 2012.

- O. Bici, D. Bugdayci, A. Aminlou, A. Hallapuro, M. Hannuksela, J. Lainema, K. Ugur, “Description of scalable video coding technology proposal by Nokia (encoder configuration 1)”, JCTVC-K0040, Shanghai, China, Oct 2012.

- S. Lasserre, F. Le Léannec, E. Nassor, J. Taquet, N. Ouedraogo, S. Pautet, C. Gisquet, G. Laroche, T. Poirier, Y. Verdavaine, E. François, “Description of the scalable video coding technology proposal by Canon Research Centre France”, JCTVC-K0041, Shanghai, China, Oct 2012.

- X. Li, et.al, “Description of Tool Experiment B3: Combined Prediction in SHVC,” JCTVC-K1103, Shanghai, China, Oct. 2012.

- J. Lainema, K Ugur, “TE3: Results of test 4.6.3 on base enhanced motion compensated prediction,” JCTVC-L0038, Geneva, CH, Jan. 2013.

# Patent rights declaration(s)

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).