| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0266 |
| --- | --- |

| Title: | TE5: Results of Test 5.3.1 on Inter-layer Inferred Prediction Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo Jianle Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-845-7532 liweig@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the results of TE5 test 5.3.1 on inter-layer inferred mode.

# Introduction

In scalable video coding, for corresponding locations in base layer image and enhancement layer image, the motion information may be highly correlated. An inferred prediction mode was introduced [1] to exploit this correlation by copying base layer motion field for the whole CU.

# Proposed method

Inferred prediction mode is introduced for Inter-coded CUs. For a CU with this mode, its motion information (which includes inter prediction direction, reference frame index and motion vectors) is not signaled. Instead, for each basic unit in the CU, its motion information is derived from its co-located base layer block. Scaling is applied to base layer motion vectors if base layer and enhancement layer images have different spatial resolutions. There are cases that the motion information of the co-located base layer block is not available (e.g., the co-located block is intra-predicted). To keep the complexity low, in these cases the basic unit at enhancement layer is predicted using intra-BL method.

The basic unit size can be configured to be 4x4, 8x8 or other block sizes. The motion parameters can be derived from compressed BL motion field which is used for TMVP derivation or the original un-compressed BL motion field. In case of using un-compressed BL motion field, the BL motion compression process is postponed and performed after the encoding of the enhancement layer to be compliant with HEVC specification. Both settings will be evaluated in next section.

This mode is signaled at CU level with a flag and it is only signaled for non-Skip Inter CU.

# Experimental results

The proposed method is implemented on SMuC 0.1.1. Anchor is generated using the same software. Test conditions specified in [2] are used. Encoding time is measured on a homogenous cluster and decoding time is obtained by sequentially decoding all the bitstreams on a single PC.

Three different cases are tested:

A. 4x4 basic unit size with un-compressed BL motion;

B. 8x8 basic unit size with un-compressed BL motion;

C: 8x8 basic unit size with compressed BL motion.

The results are presented in Table 1-3 respectively.

Table 1 Results of Test A:  4x4 Basic Unit Size with un-compressed BL Motion

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -2.66% | -5.44% | -5.26% |  |  |  | -3.08% | -7.78% | -8.03% |
| Class B | -2.02% | -3.56% | -3.63% | -2.28% | -4.05% | -4.28% | -2.43% | -4.42% | -4.63% |
| Overall (EL+BL) | -2.20% | -4.10% | -4.10% | -2.28% | -4.05% | -4.28% | -2.62% | -5.38% | -5.60% |
| Overall (EL) | -4.24% | -7.67% | -7.71% | -6.06% | -10.46% | -10.95% | -5.49% | -10.67% | -11.08% |
| Enc Time[%] | 101.06% |  |  | 94.86% |  |  | 98.06% |  |  |
| Dec Time[%] | 105.49% |  |  | 107.98% |  |  | 109.61% |  |  |

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.72% | -3.46% | -3.33% |  |  |  | -2.81% | -5.38% | -5.41% |
| Class B | -1.18% | -2.03% | -2.16% | -1.29% | -2.33% | -2.43% | -1.90% | -2.96% | -3.07% |
| Overall (EL+BL) | -1.34% | -2.44% | -2.50% | -1.29% | -2.33% | -2.43% | -2.16% | -3.65% | -3.74% |
| Overall (EL) | -2.40% | -4.38% | -4.44% | -2.96% | -5.39% | -5.54% | -3.98% | -6.61% | -6.72% |
| Enc Time[%] | 101.00% |  |  | 97.15% |  |  | 98.21% |  |  |
| Dec Time[%] | 104.19% |  |  | 104.66% |  |  | 107.33% |  |  |

Table 2 Results of Test B:  8x8 Basic Unit Size with un-compressed BL Motion

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -2.66% | -5.44% | -5.26% |  |  |  | -2.68% | -6.47% | -6.73% |
| Class B | -2.02% | -3.56% | -3.63% | -1.96% | -3.38% | -3.56% | -2.25% | -4.02% | -4.20% |
| Overall (EL+BL) | -2.20% | -4.10% | -4.10% | -1.96% | -3.38% | -3.56% | -2.37% | -4.72% | -4.92% |
| Overall (EL) | -4.24% | -7.67% | -7.71% | -5.25% | -8.82% | -9.22% | -4.99% | -9.41% | -9.79% |
| Enc Time[%] | 94.32% |  |  | 98.63% |  |  | 99.21% |  |  |
| Dec Time[%] | 100.60% |  |  | 102.52% |  |  | 103.76% |  |  |

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.72% | -3.46% | -3.33% |  |  |  | -2.26% | -4.08% | -4.12% |
| Class B | -1.18% | -2.03% | -2.16% | -1.01% | -1.79% | -1.95% | -1.67% | -2.56% | -2.62% |
| Overall (EL+BL) | -1.34% | -2.44% | -2.50% | -1.01% | -1.79% | -1.95% | -1.84% | -2.99% | -3.05% |
| Overall (EL) | -2.40% | -4.38% | -4.44% | -2.35% | -4.20% | -4.39% | -3.39% | -5.45% | -5.52% |
| Enc Time[%] | 97.28% |  |  | 102.03% |  |  | 95.79% |  |  |
| Dec Time[%] | 100.82% |  |  | 102.01% |  |  | 103.20% |  |  |

Table 3 Results of Test C:  8x8 Basic Unit Size with un-compressed BL Motion

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.89% | -1.77% | -1.76% |  |  |  | -1.83% | -4.05% | -4.25% |
| Class B | -1.24% | -2.06% | -2.07% | -1.55% | -2.57% | -2.71% | -1.85% | -3.10% | -3.24% |
| Overall (EL+BL) | -1.14% | -1.98% | -1.98% | -1.55% | -2.57% | -2.71% | -1.84% | -3.37% | -3.53% |
| Overall (EL) | -2.15% | -3.75% | -3.79% | -4.17% | -6.80% | -7.12% | -3.88% | -6.80% | -7.09% |
| Enc Time[%] | 100.96% |  |  | 100.57% |  |  | 100.51% |  |  |
| Dec Time[%] | 100.10% |  |  | 101.63% |  |  | 103.11% |  |  |

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.36% | -0.69% | -0.65% |  |  |  | -1.27% | -2.26% | -2.20% |
| Class B | -0.57% | -0.98% | -0.92% | -0.74% | -1.26% | -1.37% | -1.18% | -1.73% | -1.82% |
| Overall (EL+BL) | -0.51% | -0.90% | -0.84% | -0.74% | -1.26% | -1.37% | -1.21% | -1.88% | -1.93% |
| Overall (EL) | -0.87% | -1.59% | -1.48% | -1.71% | -2.93% | -3.16% | -2.23% | -3.43% | -3.49% |
| Enc Time[%] | 103.00% |  |  | 104.58% |  |  | 102.25% |  |  |
| Dec Time[%] | 100.37% |  |  | 101.00% |  |  | 102.44% |  |  |

# Conclusions

This contribution describes TE5 test 5.3.1 on inter-layer inferred prediction mode. Results with different basic unit sizes and handling of base layer motion field compression are reported.

# References

- J. Chen, et al. “Description of scalable video coding technology proposal by Qualcomm (configuration 1)”, JCTVC-K0035, Shanghai, China, Oct. 2012.

- V. Seregin, P. Onno, S. Liu, T. Lee, C. Kim, H. Yang, and H. Laksman, “Description of Tool Experiment C5: Inter-layer syntax prediction using HEVC base layer”, JCTVC-K1105, Shanghai, China, Oct. 2012.

# Patent rights declaration(s)

Qualcomm Incorporated may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).