| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0267 |
| --- | --- |

| Title: | Hybrid Intra and Inter-layer Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomoyuki Yamamoto, Takeshi Tsukuba 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520JAPAN | Tel: Email: | +81-43-299-8526 yamamoto.tomoyuki@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution presents Hybrid Intra and Inter-layer Prediction for SHVC, which creates prediction image by modifying HEVC intra prediction image with average of the collocated base-layer image. It is reported that the BD-rate (EL+BL) changes compared to SMuC-0.1.1 are -0.3%/-0.1% for AI HEVC 2x/1.5x.

# Introduction

Hybrid Intra and Inter-layer Prediction (HIILP) was originally proposed by the authors in a CfP response K0032 [1]. In this contribution, a modified version of HIILP, which is better suited for SMuC, is presented. The basic features of the proposed HIILP are as follows:

(a) HEVC intra prediction is modified by replacing DC value with that of the collocated base layer.

(b) On/off and unit size of the DC replacement depends on prediction block size and intra mode.

(c) No signalling change

In the following section, details of the proposed method as well as experimental results are presented. In addition, some results with modified versions of the proposed methods are presented in appendix.

# Proposed method

When Hybrid Intra and Inter-layer Prediction (HIILP) is enabled in enhancement layer, prediction image derived by HEVC intra modes (c.f. DC, Planar, and Angular modes) is modified based on the average pixel values of the collocated region on (up-sampled) base layer reconstruction. Fig 1 illustrates prediction process in HIILP.

The modification is done with the specific unit (DC unit) determined based on prediction block size and intra modes. Table 1 summarizes sizes of DC units used for each prediction block size and intra mode combination. In Table 1, Angular (H) means IntraPredMode is in the range of 2..17, and Anuglar (V) means IntraPredMode is in the range of 18..34. The modification is not used when CU size is 64x64 or intra mode is DC. Configuration other than the Table 1 have also been tested, which are presented in appendix.

Fig 1: Illustration of prediction process in HIILP

Table 1: DC unit for each prediction block size and IntraPredMode conbination
(Left column: CU size, Top row: IntraPredMode. 
Mark '-' represents DC correction is not applied for the combination)

|  | Planar | DC | Angular(H) | Angular(V) |
| --- | --- | --- | --- | --- |
| 64x64 | - | - | - | - |
| 32x32, 16x16 8x8 | 4x4 | - | 1x4 | 4x1 |

Using vertically long rectangular DC unit is useful for adjusting horizontal prediction because gradient changes along prediction direction could be compensated thanks to the DC adjustment. Prediction image, P(x,y), is derived by the following equation.

P(x,y) = Clip3(0, 255, Po(x,y) - AvgMxN(Po, x,y) + AvgMxN(R, x,y))

Where Po (x,y) is pixel values of prediction image by HEVC intra prediction. The mode used to derive Po (x,y) is determined based on IntraPredMode. AvgMxN (Po, x,y) is the average pixel value on DC unit (MxN region decided based on Table 1) on prediction image Po. Similarly, AvgMxN (R, x, y) is the average pixel value of DC unit on (up-sampled) base layer reconstruction R.

The HIILP modification described above is applied only for Luma component.

# Experimental results

The proposed method is implemented on SMuC-0.1.1bf. Evaluation is done with the test condition in TE3 document [2]. Table 2 shows the results for each configuration. It is observed that constant gains on top of SMuC-0.1.1bf could be achieved with slight increase of decoding time.

Table 2: Results of HIILP (ref. SMuC-0.1.1bf)

# Conclusion

This contribution presents Hybrid Intra and Inter-layer Prediction for SHVC, which create prediction image by modifying HEVC intra prediction image with average of collocated base-layer image. It is reported that the BD-rate (EL+BL) changes compared to SMuC-0.1.1 are -0.3%/-0.1% for AI HEVC 2x/1.5x. It is proposed to investigate this method in TE/CE as a candidate coding tool for SHVC.

# Reference

[1] T. Yamamoto, Y. Yasugi, H. Kumai, and M. Takahashi, "JCTVC-K0032: Description of scalable video coding technology proposal by SHARP (proposal 2)", JCT-VC 11th Meeting: Shanghai, 2012

[2] JCTVC-K1103, X. Li, E. Francois, P. Lai, D. Kwon, and A. Saxena, " Description of Tool Experiment B3: Combined Prediction in SHVC", JCT-VC 11th Meeting: Shanghai, 2012

# Patent rights declaration(s)

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix: Results with different DC unit

This section presents comparison among different DC unit configurations. Table 3 shows DC unit size configurations evaluated in the experiments. Note that "C" corresponds to the proposed method described above. We have not tested DC unit which is longer along angular prediction direction because we found it not efficient in our preliminary experiments.

Table 3: List of DC unit setting

|  | Planar | DC | Angular(H) | Angular(V) |
| --- | --- | --- | --- | --- |
| C (proposed) | 4x4 | - | 1x4 | 4x1 |
| Cr | 4x1 | - | 1x4 | 4x1 |
| Cs | 4x4 | - | 4x4 | 4x4 |
| B | - | - | 1x4 | 4x1 |
| Bs | - | - | 4x4 | 4x4 |

Fig 2 shows BD-rate of 2x AI case for each DC unit configuration. Configuration "C" achieves best Y BD-rate gain among tested combinations. By comparing "C" and "Cs" or "B" and "Bs", it is observed that using 1x4/4x1 DC unit for angular directions is more effective than 4x4 DC unit, especially for chroma BD-rate values.

(a) Luma

(b) Chroma (Cb)

(c) Chroma (Cr)

Fig 2: BD rate comparison of different DC unit setting (anchor: SMuC-0.1.1 bugfix)