| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0268 |
| --- | --- |

| Title: | TE4: Inter-layer adaptive filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomoyuki Yamamoto, Yukinobu Yasugi 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520JAPAN | Tel: Email: | +81-43-299-8526 yamamoto.tomoyuki@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution reports on inter-layer adaptive filter evaluated in context of TE4. The BD-rate (EL+BL) changes compared to SMuC-0.1.1 are -0.4%/-0.7% for AI HEVC 2x/1.5x, -0.0%/-0.3%/-2.2% for RA HEVC 2x/1.5x/SNR, -0.4%/-0.8%/-3.5% for LD-P HEVC 2x/1.5x/SNR.

# Introduction

Inter-Layer Adaptive Filter (ILAF) was proposed in a CfP response K0032 [1], and it has been evaluated in context of TE4 [2] as item 4.4.1. Main properties of the proposed ILAF are summarized as follows:

(a) Quad-tree based on/off signaling

(b) Filter applied for both Luma and Chroma with independent on/off flag and coeff control

(c) 5x5 square shape filter with point symmetric coeff signaling

Details of the proposed ILAF are described in the following section.

# Proposed method

Inter-Layer Adaptive Filter (ILAF) is applied to the up-sampled base-layer reconstruction. In ILAF, 5x5 square shape filter characterized by 13 coefficients is used to each pixel. Figure 1 shows the layout of coefficients in the filtering area. Note that coefficients are signaled with point symmetry. Filtering can be turned on/off in a unit called AF (Adaptive Filter) unit. The AF units are represented by quad-tree structure, of which max depth is equal to 3. In other words, the minimum AF unit size is 1/8 of picture both in width and height.

In ILAF, split flags for AF unit, the coefficients, and filter-enabling flags are signaled in the first slice header of a picture without any inter-frame prediction. Those flags are signaled at each color component independently. Please also refer to syntax table in Table 1.

In the encoding process, the filter coefficients are calculated for each color component from the original enhancement-layer image and the up-sampled base-layer picture. After calculating coefficients, the filter-enable flags are determined on AF unit basis by comparing SAD values before and after applying the filter. Finally, the filter coefficients are re-calculated from the pixels of only enabled AF units.

| c0 | c1 | c2 | c3 | c4 |
| --- | --- | --- | --- | --- |
| c5 | c6 | c7 | c8 | c9 |
| c10 | c11 | c12 | c11 | c10 |
| c9 | c8 | c7 | c6 | c5 |
| c4 | c3 | c2 | c1 | c0 |

Figure 1: Filter shape of inter-layer adaptive filter

Table 1: Syntax and semantics for adaptive filter prior to inter-layer prediction

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| ... |  |
| if (first_slice_in_pic_flag) |  |
| { |  |
| base_layer_filter_flag_qtree(0, 0, 0) |  |
| if (base_layer_filter_split_flag[ 0 ][ 0 ][ 0 ] ¦¦ base_layer_filter_flag[ 0 ][ 0 ]) |  |
| for (cIdx=0; cIdx<=2; cIdx++) |  |
| for ( i=0; i<12; i++) |  |
| base_layer_filter_coeff[ cIdx ][ i ] | se(v) |
| } |  |
| ... |  |
| } |  |

| base_layer_filter_flag_qtree(depth, x, y) { | Descriptor |
| --- | --- |
| MaxDepth = 3 |  |
| if ( depth < MaxDepth ) |  |
| base_layer_filter_split_flag[ depth ][ x ][ y ] | u(1) |
| if ( base_layer_filter_split_flag[ depth ][ x ][ y ] ) { |  |
| subSize = ( 1 << (MaxDepth - (depth+1)) ) |  |
| base_layer_filter_flag_qtree(depth+1, x, y) |  |
| base_layer_filter_flag_qtree(depth+1, x+subSize, y) |  |
| base_layer_filter_flag_qtree(depth+1, x, y+subSize) |  |
| base_layer_filter_flag_qtree(depth+1, x+subSize, y+subSize) |  |
| } |  |
| else |  |
| base_layer_filter_flag[ x ][ y ] | u(1) |
| } |  |

base_layer_filter_split_flag[ depth ][ x ][ y ] equal to 1 specifies a base-layer filter unit is split into base-layer filter units with half horizontal and vertical size. base_layer_filter_split_flag[ depth ][ x ][ y ] equal to 0 specifies a base-layer filter unit is not split. The array indices x, y specify the location ( x, y ) relative to the top-left base-layer filter unit in picture in unit of minimum AF unit. The array index depth specify the depth of the node in quad tree to which a considered base-layer filter unit belongs. When base_layer_filter_split_flag[ depth ][ x ][ y ] is not present, its value is inferred to be equal to 0.

base_layer_filter_flag[ x ][ y ] equal to 1 specifies base-layer filter is applied for a unit identified by the array index x, y. base_layer_filter_flag[ x ][ y ] equal to 0 specifies base-layer filter is not applied for a unit identified by x, y. When base_layer_filter_flag[ x ][ y ] is not present, its value is inferred to be equal to 0.

base_layer_filter_coeff[ cIdx ][ i ] specifies the base-layer filter coefficients for cIdx color component. The association between the array index i and coefficient position are shown in Figure 2.

# Experimental results

The proposed method is implemented on SMuC-0.1.1. Evaluation is done with the test condition in TE4 document [2]. Table 2 shows the results for each configuration.

The proposed ILAF is especially effective for SNR scalability. It achieves 2.2% and 3.5% EL+BL BD-rate gain for RA/SNR and LD-P/SNR.

Observed decoding time increases mainly stems from the fact that ILAF is applied for every parts of base layer irrespective of whether I_BL mode is selected.

Table 2: Results of inter-layer adaptive filter (ref. SMuC-0.1.1)

# Conclusion

This contribution reports on inter-layer adaptive filter (ILAF) evaluated in context of TE4. The BD-rate (EL+BL) changes compared to SMuC-0.1.1 are -0.4%/-0.7% for AI HEVC 2x/1.5x, -0.0%/-0.3%/-2.2% for RA HEVC 2x/1.5x/SNR, -0.4%/-0.8%/-3.5% for LD-P HEVC 2x/1.5x/SNR. It is proposed to adopt the proposed ILAF to SMuC or reference software.

# Reference

[1] T. Yamamoto, Y. Yasugi, H. Kumai, and M. Takahashi, "JCTVC-K0032: Description of scalable video coding technology proposal by SHARP (proposal 2)", JCT-VC 11th Meeting: Shanghai, 2012

[2] JCTVC-K1104, J. Chen, A. Segall. E. Alshina, S. Liu, J. Dong. J.-Y. Park "Description of Tool Experiment B4: Inter-layer filtering", JCT-VC 11th Meeting: Shanghai, 2012

# Patent rights declaration(s)

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).