| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0270 |
| --- | --- |

| Title: | TE3: Cross check for test 4.1.3 on filling unavailable reference samples (JCTVC-L0099) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomoyuki Yamamoto 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520JAPAN | Tel: Email: | +81-43-299-8526 yamamoto.tomoyuki@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This document reports cross-check result of TE3 test 4.1.3 on filling unavailable reference samples in intra prediction at enhancement layer. Code and algorithm are studied. Simulation results match with those provided by the proponent.

# Algorithm and implementation

JCTVC-L0099 introduces a method to fill unavailable reference samples with corresponding up-sampled base-layer reconstruction.

Compared to SMuC-0.1.1, only TComPattern.cpp/h and TypeDef.h are modified. Mainly, reference sample padding process is modified so that up-sampled base-layer reconsted pixel is used when it is available. The availability of base-layer reconstructed pixel is determined based on whether target pixel is in base-layer picture or not. Any base-layer syntax are not used in the process.

# Experimental results

Executables are made from source code provided by the proponent using gcc under Linux. Simulation was carried out based on the test condition defined in TE3. AI HEVC 2x and 1.5x cases are tested. Linux cluster was used to run simulation.

Table 1 shows the performance of the proposed method compared to SMuC-0.1.1. Bitrate and SNR values exactly match with the results provided by the proponent. Similar encoding and decoding time are observed.

Table 1: Results of the proposed method (ref: SMuC-0.1.1 anchor)