| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0273 |
| --- | --- |

| Title: | Cross-check results of Hybrid Intra and Inter-layer Prediction (JCTVC-L0267) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Zhan Ma Felix Fernandes 1301 E. Lookout Drive, Richardson, TX 75082, U.S.A | Tel: Email: | +1-972-761-7134 zhan.ma@sta.samsung.com felix.f@sta.samsung.com |
| Source: | Samsung Electronics |  |  |

_____________________________

# Abstract

This contribution reports the cross-check results of Hybrid Intra and Inter-layer Prediction (JCTVC-L0267) proposed by Sharp. Proponents provide the source code, simulation results as well as the contribution. All the BD-rate results matched exactly those provided by Sharp.

# Introduction

Proposal is implemented on top of the SMuC0.11bf with the following features:

- Replacing intra DC prediction using the collocated base layer.

- DC unit is determined based on the prediction block size and intra modes

# Cross-check results

All the BD-rate results exactly matched the results provided by proponent. Table 2 shows the cross-check results. More detailed results can be found in the Excel attachment. YUV files were written when the decoding times were measured. Both Encoding and Decoding were done on a Linux cluster with uniform computing nodes.

Table 2 Cross-check results for SMuC0.1.1bf anchor

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.4% | -0.5% |  |  |  |
| Class B | -0.3% | -0.7% | -0.8% | -0.1% | -0.3% | -0.3% |
| Overall (EL+BL) | -0.3% | -0.6% | -0.7% | -0.1% | -0.3% | -0.3% |
| Overall (EL) | -0.5% | -1.1% | -1.2% | -0.3% | -0.5% | -0.6% |
| Enc Time[%] | 109.4% |  |  | 107.8% |  |  |
| Dec Time[%] | 100.9% |  |  | 100.4% |  |  |
| Enc Mem[%] | 100.1% |  |  | 99.9% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# Conclusion

The results presented by Sharp (JCTVC-L0267) are verified by Samsung Electronics.

# Reference

[1] T. Yamamoto (Sharp), “Hybrid Intra and Inter-layer Prediction”, JCTVC-L0267, Geneva, CH, Jan. 2013