| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0276r2 |
| --- | --- |

| Title: | On hrd_parameters() |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazushi Sato Sony City Osaki 2-10-1 Osaki Shinagawa Tokyo 141-8610 Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

In HEVC HRD parameters can be defined either as whole-sequence level or sub-stream level. However in the current syntax specification it is not clear that sub-stream level definition is whether just for the current layer or for the sum from lower layers. This is also true for temporal layers.

This contribution proposes to change hrd_parameters() syntax so that the HRD parameters are either for a single-decoder model or multi-decoder model or for both.

# Introduction / Problem Statement / …

In the left example of Fig 1.1, the bitstream contains 2 temporal layers. It is possible that a whole bitstream be decoded with one decoder (scenario 1) or each temporal layer be decoded with separate decoders (scenario 2).

This is also true in the right example of Fig 1.1. In this case there are 2 scalable layers: BL and EL. It is possible that whole layer be decoded with one decoder (scenario 1) or each layer be decoded with separate decoders (scenario 2).

Fig 1.1

With 2-decoder model case, each layers/sublayers are decoded as shown in Fig 1.2.

Fig 1.2

HRD behaves differently with scenario 1 and scenario 2 both in temporal-scalability and SHVC cases. However in the current syntax specification it is not clear that sub-stream level definition is whether just for the current layer/sub-layer or for the sum from lower layers/sub-layers.

# Proposal

To solve the problem stated in the previous section it is proposed to change the syntax of “hrd_parameters()” as shown in Table 2.1. “Type_1” means that parameters are defined just for the current layer/sublayer, and “Type_2” means that they are defined as the sum from the lower layers/sublayers.

For the base layer/sub-layer, or if a stream contains just one layer/sub-layer HRD parameters can be defined either as “Type_1” or “Type_2”.

Table 2.1

| hrd_parameters( commonInfPresentFlag, maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| if( commonInfPresentFlag ) { |  |
| timing_info_present_flag | u(1) |
| if( timing_info_present_flag ) { |  |
| num_units_in_tick | u(32) |
| time_scale | u(32) |
| } |  |
| nal_hrd_parameters_present_flag | u(1) |
| vcl_hrd_parameters_present_flag | u(1) |
| if( nal_hrd_parameters_present_flag ¦ ¦ vcl_hrd_parameters_present_flag ){ |  |
| type1_layer_hrd_parameters_present_flag | u(1) |
| type2_layer_hrd_parameters_present_flag | u(1) |
| if( type1_layer_hrd_parameters_present_flag) { |  |
| type1_sub_pic_cpb_params_present_flag | u(1) |
| if( type1_sub_pic_cpb_params_present_flag ) { |  |
| type1_tick_divisor_minus2 | u(8) |
| type1_du_cpb_removal_delay_length_minus1 | u(5) |
| type1_sub_pic_cpb_params_in_pic_timing_sei_flag | u(1) |
| } |  |
| type1_bit_rate_scale | u(4) |
| type1_cpb_size_scale | u(4) |
| if( type1_sub_pic_cpb_params_present_flag ) |  |
| type1_cpb_size_du_scale | u(4) |
| type1_initial_cpb_removal_delay_length_minus1 | u(5) |
| type1_au_cpb_removal_delay_length_minus1 | u(5) |
| type1_dpb_output_delay_length_minus1 | u(5) |
| } |  |
| if( type2_layer_hrd_parameters_present_flag) { |  |
| type2_sub_pic_cpb_params_present_flag | u(1) |
| if( type2_sub_pic_cpb_params_present_flag ) { |  |
| type2_tick_divisor_minus2 | u(8) |
| type2_du_cpb_removal_delay_length_minus1 | u(5) |
| type2_sub_pic_cpb_params_in_pic_timing_sei_flag | u(1) |
| } |  |
| type2_bit_rate_scale | u(4) |
| type2_cpb_size_scale | u(4) |
| if( type2_sub_pic_cpb_params_present_flag ) |  |
| type2_cpb_size_du_scale | u(4) |
| type2_initial_cpb_removal_delay_length_minus1 | u(5) |
| type2_au_cpb_removal_delay_length_minus1 | u(5) |
| type2_dpb_output_delay_length_minus1 | u(5) |
| } |  |
| } |  |
| } |  |
| for( i = 0; i <= maxNumSubLayersMinus1; i++ ) { |  |
| fixed_pic_rate_general_flag[ i ] | u(1) |
| if( !fixed_pic_rate_general_flag[ i ] ) |  |
| fixed_pic_rate_within_cvs_flag[ i ] | u(1) |
| if( fixed_pic_rate_within_cvs_flag[ i ] ) |  |
| elemental_duration_in_tc_minus1[ i ] | ue(v) |
| type1_sub_layer_hrd_parameters_present_flag[i] | u(1) |
| type2_sub_layer_hrd_parameters_present_flag[i] | u(1) |
| if( type1_sub_layer_hrd_parameters_present_flag[i]) { |  |
| type1_low_delay_hrd_flag[ i ] | u(1) |
| type1_cpb_cnt_minus1[ i ] | ue(v) |
| if( nal_hrd_parameters_present_flag ) |  |
| sub_layer_hrd_parameters( i ) |  |
| if( vcl_hrd_parameters_present_flag ) |  |
| sub_layer_hrd_parameters( i ) |  |
| } |  |
| if( type2_sub_layer_hrd_parameters_present_flag[i]) { |  |
| type2_low_delay_hrd_flag[ i ] | u(1) |
| type2_cpb_cnt_minus1[ i ] | ue(v) |
| if( nal_hrd_parameters_present_flag ) |  |
| sub_layer_hrd_parameters( i ) |  |
| if( vcl_hrd_parameters_present_flag ) |  |
| sub_layer_hrd_parameters( i ) |  |
| } |  |
| } |  |
| } |  |

# Conclusion

In the current HEVC specification it is not clear that sub-stream level definition of HRD parameters is whether just for the current layer or for the sum from lower layers. This is also true for temporal layers. This contribution proposes to change hrd_parameters() syntax so that the HRD parameters are either for a single-decoder model or multi-decoder model or for both.

It is recommended that this modification be adopted into HEVC specification.

# References

[1] “High Efficiency Video Coding (HEVC) text specification draft 9 (SoDIS)”, JCTVC-K1003, Oct 2012

# Patent rights declaration(s)

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).