| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0277r1 |
| --- | --- |

| Title: | Suggested Modifications for Still Picture Profile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazushi Sato Sony City Osaki 2-10-1 Osaki Shinagawa Tokyo 141-8610 Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

At the 11th JCTVC meeting, it is decided that Still Picture Profile be included in HEVC standard. If an image is coded as HEVC with Still Picture Profile, transmission of information on HRD, temporal layer, or inter-coding tools is redundant.

This document proposes restrictions on semantics to remove such redundancies. It is recommended that the proposed restrictions be adopted into HEVC specification.

# Introduction / Problem Statement / …

If an image is coded with Still Picture Profile, transmission of syntax elements related to HRD, temporal layer or inter-coding tools is redundant. However, change for Still Picture Profile should be minimized at this stage of the standard.

# Proposal

To solve the problem stated above, this contribution proposes to introduce constraints to semantics as follow if an image is coded as Still Picture Profile:

## Video Parameter Set

- vps_max_sub_layers_minus1 shall be 0.

- vps_num_hrd_parameters shall be 0.

Table 2.1.1

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| vps_reserved_three_2bits | u(2) |
| vps_reserved_zero_6bits | u(6) |
| vps_max_sub_layers_minus1 | u(3) |
| vps_temporal_id_nesting_flag | u(1) |
| vps_reserved_0xffff_16bits | u(16) |
| profile_tier_level( 1, vps_max_sub_layers_minus1 ) |  |
| bit_rate_pic_rate_info( 0, vps_max_sub_layers_minus1 ) |  |
| vps_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( vps_sub_layer_ordering_info_present_flag ? 0 : vps_max_sub_layers_minus1 ); i <= vps_max_sub_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_max_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| vps_max_nuh_reserved_zero_layer_id | u(6) |
| vps_num_op_sets_minus1 | ue(v) |
| for( i = 1; i <= vps_num_op_sets_minus1; i++ ) |  |
| operation_point_set( i ) |  |
| vps_num_hrd_parameters | ue(v) |
| for( i = 0; i < vps_num_hrd_parameters; i++ ) { |  |
| hrd_op_set_idx[ i ] | ue(v) |
| if( i > 0 ) |  |
| cprms_present_flag[ i ] | u(1) |
| hrd_parameters( cprms_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

## Sequence Parameter Set

- sps_max_sub_layers_minus1 shall be 0.

- num_short_term_ref_pic_sets shall be 0.

- long_term_ref_pics_present_flag shall be 0.

- sps_temporal_mvp_enable_flag shall be 0.

Table 2.2.1

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_temporal_id_nesting_flag | u(1) |
| profile_tier_level( 1, sps_max_sub_layers_minus1 ) |  |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| sps_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( sps_sub_layer_ordering_info_present_flag ? 0 : sps_max_sub_layers_minus1 ); i <= sps_max_sub_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering[ i ] | ue(v) |
| sps_max_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase[ i ] | ue(v) |
| } |  |
| log2_min_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_luma_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enable_flag | u(1) |
| if( scaling_list_enable_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_data( ) |  |
| } |  |
| amp_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| log2_min_pcm_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| pcm_loop_filter_disable_flag | u(1) |
| } |  |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ) { |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_temporal_mvp_enable_flag | u(1) |
| strong_intra_smoothing_enable_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

## Picture Parameter Set

- num_ref_idx_l0_default_active_minus1 shall be 0.

- num_ref_idx_l1_default_active_minus1 shall be 0.

- weighted_pred_flag shall be 0.

- weighted_bipred_flag shall be 0.

-	lists_modification_present_flag shall be 0.

-	log2_parallel_merge_level_minus2 shall be 0.

Table 2.3.1

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| dependent_slice_segments_enabled_flag | u(1) |
| sign_data_hiding_flag | u(1) |
| cabac_init_present_flag | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| init_qp_minus26 | se(v) |
| constrained_intra_pred_flag | u(1) |
| transform_skip_enabled_flag | u(1) |
| cu_qp_delta_enabled_flag | u(1) |
| if ( cu_qp_delta_enabled_flag ) |  |
| diff_cu_qp_delta_depth | ue(v) |
| pps_cb_qp_offset | se(v) |
| pps_cr_qp_offset | se(v) |
| pps_slice_chroma_qp_offsets_present_flag | u(1) |
| weighted_pred_flag | u(1) |
| weighted_bipred_flag | u(1) |
| output_flag_present_flag | u(1) |
| transquant_bypass_enable_flag | u(1) |
| tiles_enabled_flag | u(1) |
| entropy_coding_sync_enabled_flag | u(1) |
| if( tiles_enabled_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height_minus1[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| loop_filter_across_slices_enabled_flag | u(1) |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_disable_deblocking_filter_flag | u(1) |
| if( !pps_disable_deblocking_filter_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| pps_scaling_list_data_present_flag | u(1) |
| if( pps_scaling_list_data_present_flag ) |  |
| scaling_list_data( ) |  |
| lists_modification_present_flag | u(1) |
| log2_parallel_merge_level_minus2 | ue(v) |
| num_extra_slice_header_bits | u(3) |
| slice_segment_header_extension_present_flag | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

## Scaling List

Semantics of “sizeId” and “matrixId” in Table 2.4.1 are shown in Table 2.4.2 and Table 2.4.3.

If sizeId is 0, 1 or 2 and matrixId is 3, 4 or 5, or if sizeId is 3 and matrixId is 1, transmitting scaling_list_data is redundant. Therefore in this case

- scaling_list_pred_mode_flag[ sizeId ][ matrixId ] shall be 0.

- scaling_list_pred_matrix_id_delta[ sizeId ][ matrixId ] shall be 0.

Table 2.4.1

| scaling_list_data( ) { | Descriptor |
| --- | --- |
| for( sizeId = 0; sizeId < 4; sizeId++ ) |  |
| for( matrixId = 0; matrixId < ( ( sizeId = = 3 ) ? 2 : 6 ); matrixId++ ) { |  |
| scaling_list_pred_mode_flag[ sizeId ][ matrixId ] | u(1) |
| if( !scaling_list_pred_mode_flag[ sizeId ][ matrixId ] ) |  |
| scaling_list_pred_matrix_id_delta[ sizeId ][ matrixId ] | ue(v) |
| else { |  |
| nextCoef = 8 |  |
| coefNum = Min( 64, ( 1 << ( 4 + ( sizeId << 1) ) ) ) |  |
| if( sizeId > 1 ) { |  |
| scaling_list_dc_coef_minus8[ sizeId − 2 ][ matrixId ] | se(v) |
| nextCoef = scaling_list_dc_coef_minus8[ sizeId − 2 ][ matrixId ] + 8 |  |
| } |  |
| for( i = 0; i < coefNum; i++) { |  |
| scaling_list_delta_coef | se(v) |
| nextCoef = ( nextCoef + scaling_list_delta_coef + 256 ) % 256 |  |
| ScalingList[ sizeId ][ matrixId ][ i ] = nextCoef |  |
| } |  |
| } |  |
| } |  |
| } |  |

Table 2.4.2

| - Size of quantization matrix | - sizeId |
| --- | --- |
| - 4x4 | - 0 |
| - 8x8 | - 1 |
| - 16x16 | - 2 |
| - 32x32 | - 3 |

Table 2.4.3

| - sizeId | - CuPredMode | - cIdx (colour component) | - matrixId |
| --- | --- | --- | --- |
| - 0, 1, 2 | - MODE_INTRA | - 0 (Y) | - 0 |
| - 0, 1, 2 | - MODE_INTRA | - 1 (Cb) | - 1 |
| - 0, 1, 2 | - MODE_INTRA | - 2 (Cr) | - 2 |
| - 0, 1, 2 | - MODE_INTER | - 0 (Y) | - 3 |
| - 0, 1, 2 | - MODE_INTER | - 1 (Cb) | - 4 |
| - 0, 1, 2 | - MODE_INTER | - 2 (Cr) | - 5 |
| - 3 | - MODE_INTRA | - 0 (Y) | - 0 |
| - 3 | - MODE_INTER | - 0 (Y) | - 1 |

## Slice Header

- slice_type shall be I_slice.

IdrPicFlag = ( nal_unit_type  = =  IDR_W_LP  | |  nal_unit_type  = =  IDR_N_LP ) and it should be “1” for Still Picture Profile. Nal_unit_types are shown in Table 2.5.2. Therefore

- nal_unit_type of the slices shall be IDR_W_LP or IDR_N_LP.

Table 2.5.1

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| if( RapPicFlag ) |  |
| no_output_of_prior_pics_flag | u(1) |
| slice_pic_parameter_set_id | ue(v) |
| if( !first_slice_segment_in_pic_flag ) { |  |
| if(dependent_slice_segments_enabled_flag ) |  |
| dependent_slice_segment_flag | u(1) |
| slice_segment_address | u(v) |
| } |  |
| if( !dependent_slice_segment_flag ) { |  |
| for ( i = 0; i < num_extra_slice_header_bits; i++ ) |  |
| slice_reserved_undetermined_flag[ i ] | u(1) |
| slice_type | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( !IdrPicFlag ) { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| if( num_long_term_ref_pics_sps > 0 ) |  |
| num_long_term_sps | ue(v) |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ ) { |  |
| if( i < num_long_term_sps ) |  |
| lt_idx_sps[ i ] | u(v) |
| else { |  |
| poc_lsb_lt[ i ] | u(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| } |  |
| } |  |
| if( sps_temporal_mvp_enable_flag ) |  |
| slice_temporal_mvp_enable_flag | u(1) |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| if( lists_modification_present_flag && NumPocTotalCurr > 1 ) |  |
| ref_pic_lists_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( slice_temporal_mvp_enable_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) [Ed. (GJS): Does the logic of this condition check make sense when the slice is not a B slice?] |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| } |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_disable_deblocking_filter_flag | u(1) |
| if( !slice_disable_deblocking_filter_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| if( loop_filter_across_slices_enabled_flag && ( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ¦ ¦ !slice_disable_deblocking_filter_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( tiles_enabled_flag ¦ ¦ entropy_coding_sync_enabled_flag ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset[ i ] | u(v) |
| } |  |
| } |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| slice_segment_header_extension_length | ue(v) |
| for( i = 0; i < slice_segment_header_extension_length; i++) |  |
| slice_segment_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

Table 2.5.2

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0, 1 | TRAIL_N, TRAIL_R | Coded slice segment of a non-TSA, non-STSA trailing picture slice_segment_layer_rbsp( ) | VCL |
| 2, 3 | TSA_N, TSA_R | Coded slice segment of a TSA picture slice_segment_layer_rbsp( ) | VCL |
| 4, 5 | STSA_N, STSA_R | Coded slice segment of an STSA picture slice_layer_rbsp( ) | VCL |
| 6, 7 | RADL_N, RADL_R | Coded slice segment of a RADL picture slice_layer_rbsp( ) | VCL |
| 8, 9 | RASL_N, RASL_R, | Coded slice segment of a RASL picture slice_layer_rbsp( ) | VCL |
| 10, 12, 14 | RSV_VCL_N10 RSV_VCL_N12 RSV_VCL_N14 | Reserved // reserved non-RAP non-reference VCL NAL unit types | VCL |
| 11, 13, 15 | RSV_VCL_R11 RSV_VCL_R13 RSV_VCL_R15 | Reserved // reserved non-RAP reference VCL NAL unit types | VCL |
| 16, 17, 18 | BLA_W_LP BLA_W_DLP BLA_N_LP | Coded slice segment of a BLA picture slice_segment_layer_rbsp( ) [Ed. (YK): BLA_W_DLP -> BLA_W_RADL?] | VCL |
| 19, 20 | IDR_W_DLP IDR_N_LP | Coded slice segment of an IDR picture slice_segment_layer_rbsp( ) [Ed. (YK: IDR_W_DLP -> IDR_W_RADL?] | VCL |
| 21 | CRA_NUT | Coded slice segment of a CRA picture slice_segment_layer_rbsp( ) | VCL |
| 22, 23 | RSV_RAP_VCL22.. RSV_RAP_VCL23 | Reserved // reserved RAP VCL NAL unit types [Ed.(YK): Some RAP related restrictions need to be specified for these 2 NUTs] | VCL |
| 24..31 | RSV_VCL24.. RSV_VCL31 | Reserved // reserved non-RAP VCL NAL unit types [Ed.(YK): Some non-RAP related restrictions may need to be specified for these NUTs] | VCL |
| 32 | VPS_NUT | Video parameter set video_parameter_set_rbsp( ) | non-VCL |
| 33 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 34 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 35 | AUD_NUT | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 36 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 37 | EOB_NUT | End of bitsteam end_of_bitstream_rbsp( ) | non-VCL |
| 38 | FD_NUT | Filler data filler_data_rbsp( ) | non-VCL |
| 39, 40 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information (SEI) sei_rbsp( ) | non-VCL |
| 41..47 | RSV_NVCL41.. RSV_NVCL47 | Reserved | non-VCL |
| 48..63 | UNSPEC48.. UNSPEC63 | Unspecified | non-VCL |

# Conclusion

If an image is coded as HEVC with Still Picture Profile, transmission of information on HRD, temporal layer, or inter-coding tools is redundant. However, change for Still Picture Profile should be minimized at this stage of the standard. Therefore this contribution proposes to introduce constraints to semantics as follow if an image is coded as Still Picture Profile.

It is recommended that the proposed method be adopted into HEVC standard.

# References

[1] “High Efficiency Video Coding (HEVC) text specification draft 9 (SoDIS)”, JCTVC-K1003, Oct 2012

# Patent rights declaration(s)

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).