| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0278r2 |
| --- | --- |

| Title: | SHVC: On Inter-layer Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazushi Sato Sony City Osaki 2-10-1 Osaki Shinagawa Tokyo 141-8610 Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

SMuC 0.1.1 contains IntraBL, which use up-sampled base-layer as predicted samples. This topic is being investigated further under TE2 of SHVC as inter-layer pixel prediction. Another topic, inter-layer syntax prediction is being investigated with elements like motion or intra-mode information. The former requires more memory access, and this document claims study on the trade-off between coding efficiency and implementation cost with these tools, including enabling/disabling functionalities in HLS.

# Introduction / Problem Statement / ...

For SHVC dependencies between layers are specified in vps_extension at sequence level [3]. “Inter-layer prediction” can be categorized into 2 areas: inter-layer pixel prediction [4] and inter-layer syntax prediction. The former requires more memory access than the latter, and trade-off between coding efficiency and implementation cost should be studied.

# Proposed Algorithm

If time distance between the current and the reference frames is far, more PUs are coded as Intra and in this case IntraBL will be helpful to improve coding efficiency. At the last JCTVC meeting there has been similar discussions in JCTVC-K0175 [1] and JCTVC-K0264 [2]. This is not true if time distance between the current and the reference frames is close. Even in this case there is co-relation in syntax between the base and the enhancement layer.

Therefore it is proposed to enable/disable inter-layer prediction, for example, as shown in Fig 2.1:

-	Inter-layer pixel and syntax prediction be able to be set independently.

-	Inter-layer pixel prediction be set in terms of temporal hierarchy.

JCTVC-L0071 [8] contains essentially same proposal as the first proposal.

As an example of the second proposal JCTVC-K0175 proposed an additional syntax in vps_extension. It would be better that on/off can be override at slice header, similarly as RPS.

Fig 2.1

# Simulation

## Simulation Condition

In this contribution macro “SVC_MVP” is used as an example of inter-layer syntax prediction. 4 kinds of tests have been conducted as follow:

Test A:

Anchor: Simulcast; Tested: SMuC 0.1.1

Test B:

Anchor: SMuC 0.1.1; Tested: SMuC0.1.1 (IntraBL disabled at highest temp. hierarchy)

Test C:

Anchor: SMuC 0.1.1; Tested: SMuC0.1.1 (IntraBL disabled at highest temp. hierarchy) + SVC_MVP

Test D:

Anchor: SMuC 0.1.1; Tested: SMuC0.1.1 + SVC_MVP

Class A and B sequences are tested with RA_{2x,1.5x} conditions.

## Simulation Result

Summary of the results with Test A, B, C and D are shown in Table 3.2.1, 3.2.2, 3.2.3 and 3.2.4 respectively. Complete results are shown in JCTVC-L0278_a.xls, JCTVC-L0278_b.xls, JCTVC-L0278_c.xls and JCTVC-L0278_d.xls, respectively.

The author would like to thank Sharp and JVC for crosschecking [6][7].

As can be seen gains with IntraBL are -16.4% and -26.1% with RA_2x and RA_1.5x respectively. Just by disabling it at the highest temporal hierarchy loss by 1.0% and 2.1% are observed with RA_2x and RA_1.5x respectively. This loss can be compensated by inter-layer syntax prediction: by enabling SVC_MVP gain by -0.4% and -0.4% can be obtained in comparison with SMuC 0.1.1, even if IntraBL is disabled at the highest temporal hierarchy.

As reference, gain purely obtained by the option “SVC_MVP” is -1.2% and -1.8% with RA_2x and RA_1.5x respectively.

Table 3.2.1

Table 3.2.2

Table 3.2.3

Table 3.2.4

# Conclusion

This document proposes the followings for inter-layer prediction:

-	Inter-layer pixel and syntax prediction be able to be set independently.

-	Inter-layer pixel prediction be set in terms of temporal hierarchy.

It is recommended that this topic be investigated under CE or AHG, including HLS to support these functionalities.

# References

[1] “On inter-layer prediction enabling/disabling for HEVC scalable extensions”, JCTVC-K0175, Oct 2012

[2] “Hierarchical inter-layer prediction in multi-loop scalable extension of HEVC”, JCTVC-K0264, Oct 2012

[3] “NAL unit header and parameter set designs for HEVC extensions”, JCTVC-K1007, Oct 2012

[4] “Description of Tool Experiment A2: Inter-layer Texture Prediction Signaling in SHVC”, JCTVC-K1102, Oct 2012

[5] “Description of Tool Experiment C5: Inter-layer syntax prediction using HEVC base layer”, JCTVC-K1105, Oct 2012

[6] “Cross check for restriction on Inter-layer prediction (JCTVC-L0278)”, JCTVC-L0326, Jan 2013

[7] “SHVC: Cross-verification of restriction on inter-layer texture prediction at high temporal hierarchy (JCTVC-L0278)”, JCTVC-L0337, Jan 2013

[8] “AHG9: Inter-layer prediction flag and inter-layer syntax prediction flag”, JCTVC-L0071, Jan 2013

# Patent rights declaration(s)

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).