| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0279r1 |
| --- | --- |

| Title: | SHVC: On Motion Data Compression |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazushi Sato, Shuo Lu Sony City Osaki 2-10-1 Osaki Shinagawa Tokyo 141-8610 Japan Jun Xu 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com Shuo.Lu@jp.sony.com +1-408-352-4086 jun.xu@am.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

HEVC employs TMVP for motion vector prediction both in AMVP and Merge scheme. The smallest size of inter PU is either 4x8 or 8x4 so the motion information should originally be stored in the unit of 4x4. To reduce buffer size for motion data storage HEVC allows to storing motion data with decimation like in the unit of 16x16.

In SMuC 0.1.1 this motion storage compression is executed right after the encoding/decoding of the base-layer. However most proposals in TE 5.2.x postpone this compression after the encoding/decoding of the enhancement layer. The former requires less buffer size, while the latter provides better coding efficiency.

In this proposal intermediate solution between the 2 methods is investigated. The proposed method provides a trade-off between coding efficiency and buffer size.

It is recommended that the proposed method be investigated under CE or AHG.

# Introduction / Problem Statement / …

HEVC employs TMVP for motion vector prediction both in AMVP and Merge scheme. The smallest size of inter PU is either 4x8 or 8x4 so the motion information should originally be stored in the unit of 4x4. To reduce buffer size for motion data storage HEVC allows to storing motion data with decimation like in the unit of 16x16.

In SMuC 0.1.1 this motion storage compression is executed right after the encoding/decoding of the base-layer as shown in the left part of Fig 1.1. Most proposals in TE 5.2.x postpone this compression after the encoding/decoding of the existing layer as shown in the right part of Fig 1.1.

The latter provides better coding efficiency, but buffer size in the unit of 4x4 with the base-layer resolution is needed for motion data storage. On the contrary less buffer size is needed for the former method, but it causes loss in coding efficiency. Especially, for example, in the case of 2x scalability, the former method means that motion data is stored in the unit of 32x32 unit in the enhancement-layer resolution.

Fig 1.1

# Proposed Algorithm

In this proposal intermediate solution between the methods described in the previous section is investigated as shown in Fig 2.1: that is, motion data compression by 2:1 is executed after encoding/decoding of base-layer, and other 2:1 compression is executed after encoding/decoding of enhancement-layer.

In the case of 2x scalability, motion data is stored in the unit of 16x16 in the enhancement-layer resolution, same as single-layer HEVC case.

Fig 2.1

# Simulation

## Simulation Condition

In this document, 3 kinds of tests are shown as follows:

Test A: ETRI_Merge (MOTION_COMP_MODIFICATION=1)

Test B: Tested: ETRI_Merge (MOTION_COMP_MODIFICATION=0)

Test C: Tested: ETRI_Merge (With the proposed method as shown in Fig 2.1)

SMuC 0.1.1 is used as anchor. Class A and B sequences are tested with {RA, LP}_{2x,1.5x} conditions.

## Simulation Result

Summary of the results with Test A, B and C are shown in Table 3.2.1, 3.2.2 and 3.2.3 respectively.

Complete result with Test A is included in [2]. The ones with Test B and C are shown in JCTVC-L0279_0.xls and JCTVC-L0279_0.5.xls, respectively.

The author would like to thank ETRI for crosschecking [3].

As can be seen the proposed method provides a trade-off between coding efficiency and complexity.

Gain with Test A is -2.2%, -2.3%, -1.2% and -1.2% for RA_2x, RA_1.5x, LD-P_2x and LD-P_1.5x, respectively. Gain with Test B is -1.1%, -1.8%, -0.3% and -0.8% for RA_2x, RA_1.5x, LD-P_2x and LD-P_1.5x, respectively. Loss of Test B over A is larger with the 2x-case than with the 1.5x-case, as the resolution of motion vector storage with enhancement-layer resolution is coarser with the 2x-case.

Gain with Test C is -1.9%, -2.2%, -0.9% and -1.1% for RA_2x, RA_1.5x, LD-P_2x and LD-P_1.5x, respectively. Result with Test C is closer to Test A than Test B. The proposed method provides good trade-off between coding efficiency and complexity.

Table 3.2.1

Table 3.2.2

Table 3.2.3

# Conclusion

There are 2 types of motion data compression schemes in SMuC 0.1.1 and proposals in TE 5.2.x. In SMuC 0.1.1 this motion storage compression is executed right after the encoding/decoding of the base-layer. Most proposals in TE 5.2.x postpone this compression after the encoding/decoding of the existing layer. The former requires less buffer size, while the latter provides better coding efficiency.

In this proposal intermediate solution between the 2 methods is investigated. The proposed method provides a trade-off between coding efficiency and buffer size.

It is recommended that the proposed method be investigated under CE or AHG.

# References

[1] “Description of Tool Experiment C5: Inter-layer syntax prediction using HEVC base layer”, JCTVC-K1105, Oct 2012

[2] “SHVC: Crosscheck of TE5.2.3”, JCTVC-L0274, Jan 2013

[3] “Cross-check of JCTVC-L0279”, JCTVC-L0368, Jan 2013

[4] “TE5: Results of test 5.2.3 on inter-layer motion vector prediction”, JCTVC-L0065, Jan 2013

# Patent rights declaration(s)

Sony Corp. / Sony Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).