| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 12th Meeting: Geneva, CH, 14-23 Jan., 2013 | Document: JCTVC-L0283 |
| --- | --- |

| Title: | Non-TE4: Inter-layer SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhenzhong Chen, Shan Liu, Ximin Zhang MediaTek USA 2860 Junction Ave, San Jose, CA 95134, USA Shih-Ta Hsiang, Chih-Ming Fu, Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | +1-408-5261899 {zhenzhong.chen, shan.liu, ximin.zhang, shih-ta.hsiang, chihming.fu, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution describes MediaTek’s work on inter-layer sample adaptive offset (SAO) with iterations. Two methods are proposed. The first proposed method adopts the inter-layer sample adaptive offset (SAO) for the up-sampled picture of reconstructed base layer and allows the inter-layer sample adaptive offset (SAO) to be applied on the processed picture iteratively. Experimental results show that based on the first proposed method, luma BD-rate savings of 0.5% for AI-2x, 0.1 for AI-1.5x, 0.6% for RA-2x, 0.1% for RA-1.5x, 0.5% for RA-SNR, 0.7% for LD-P-2x, 0.3% for LD-P-1.5x, and 0.9% for LD-P-SNR (BD-rate calculated using both enhancement layer and base layer rates). The encoding time increase is 0.6% on average and the decoding time increase is 6.9% on average. The second method allows all the SAO types to be applied. Experimental results show that based on the second proposed method, luma BD-rate savings of 0.5% for AI-2x, 0.1 for AI-1.5x, 0.7% for RA-2x, 0.1% for RA-1.5x, 0.5% for RA-SNR, 0.9% for LD-P-2x, 0.3% for LD-P-1.5x, 1.0% for LD-P-SNR (BD-rate calculated using both enhancement layer and base layer rates). The encoding time increase is negligible on average and the decoding time increase is 8.8% on average.

- Introduction

Sample adaptive offset (SAO) is a useful tool in HEVC to improve the coding performance. Similarly, inter-layer SAO provides coding gain for SHVC [1]. Inter-layer SAO is to remove the texture redundancy between layers. After the up-sampled picture of reconstructed base layer is processed by inter-layer SAO, additional inter-layer SAO can further reduce the difference between the original EL picture and processed picture from BL for inter-layer texture prediction.

# Proposed methods

In HEVC based scalable extension (SHVC), when a coding unit (CU) in the EL is coded as inter-layer texture prediction, the corresponding texture in the BL is up-sampled and used as texture predictors for the CU in the EL. In this contribution we propose to perform the inter-layer sample adaptive offset (SAO) for the up-sampled picture of reconstructed base layer and allow the inter-layer sample adaptive offset (SAO) to be applied on the processed picture iteratively, as shown in Figure 1. The reconstructed base layer (BL) samples are up-sampled to generate the predictor for co-located enhancement layer (EL) samples, as shown in Figure 1. The up-sampling procedure of generating the inter-layer texture prediction samples is based on the adopted up-sampling filter and phase shift.

We propose two methods. In the first method, we propose to calculate the offset by using the inter-layer SAO method iteratively for the processed up-sampled reconstructed base layer picture in the last round of iteration. The encoder provides the offset information in the enhancement layer bit stream and adds the offsets to the prediction. The corresponding sets of offset and band positions are sent in slice header.

Figure 1. Scalable Video Coding diagram with inter-layer SAO.

In the second method, we propose to calculate the offsets for each SAO type, i.e., four EO types and BO. The encoder provides the offset information of each SAO type in the enhancement layer bit stream and adds the offsets to the prediction. For each SAO type, on/off flag is to indicate whether or not the corresponding SAO type is used. The corresponding sets of offset and band positions are sent in slice header.

# Simulation results and discussion

The common test conditions defined in the CfP are used. Anchor is generated using the SMuC0.1.1 software.

In Table 1, experimental results of the first proposed method for inter-layer sample adaptive offset (SAO) with three iterations are provided. As shown in Table 1, the first proposed method provides luma BD-rate savings of 0.5% for AI-2x, 0.1 for AI-1.5x, 0.6% for RA-2x, 0.1% for RA-1.5x, 0.5% for RA-SNR, 0.7% for LD-P-2x, 0.3% for LD-P-1.5x, and 0.9% for LD-P-SNR (BD-rate calculated using both enhancement layer and base layer rates). When BD-rate is calculated using EL only, the luma BD-rate savings are 1.1% for AI-2x, 0.2 for AI-1.5x, 1.2% for RA-2x, 0.3% for RA-1.5x, 1.1% for RA-SNR, 1.5% for LD-P-2x, 0.7% for LD-P-1.5x, and 1.8% for LD-P-SNR. The encoding time increase is 0.6% on average and the decoding time increase is 6.9% on average.

In Table 2, experimental results of the second proposed method for inter-layer sample adaptive offset (SAO) Experimental results show that based on the second proposed method, luma BD-rate savings of 0.5% for AI-2x, 0.1 for AI-1.5x, 0.7% for RA-2x, 0.1% for RA-1.5x, 0.5% for RA-SNR, 0.9% for LD-P-2x, 0.3% for LD-P-1.5x, 1.0% for LD-P-SNR (BD-rate calculated using both enhancement layer and base layer rates). When BD-rate is calculated using EL only, the luma BD-rate savings are 1.1% for AI-2x, 0.2 for AI-1.5x, 1.4% for RA-2x, 0.2% for RA-1.5x, 1.0% for RA-SNR, 1.8% for LD-P-2x, 0.7% for LD-P-1.5x, 1.9% for LD-P-SNR.

Table 1.  BD rate comparison results of the first proposed method with three iterations.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -1.3% | -2.3% | -2.5% |  |  |  |  |  |  |
| Class B | -0.1% | -0.5% | -0.6% | -0.1% | -0.3% | -0.3% |  |  |  |
| Overall (EL+BL) | -0.5% | -1.0% | -1.1% | -0.1% | -0.3% | -0.3% |  |  |  |
| Overall (EL) | -1.1% | -2.2% | -2.4% | -0.2% | -0.6% | -0.7% |  |  |  |
| Enc Time[%] | 101.5% |  |  | 101.7% |  |  |  |  |  |
| Dec Time[%] | 107.7% |  |  | 103.4% |  |  |  |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.4% | -1.1% | -1.5% |  |  |  | -1.0% | -1.4% | -2.1% |
| Class B | -0.2% | -0.3% | -0.3% | -0.1% | -0.2% | -0.2% | -0.3% | -0.6% | -0.4% |
| Overall (EL+BL) | -0.6% | -0.5% | -0.6% | -0.1% | -0.2% | -0.2% | -0.5% | -0.8% | -0.9% |
| Overall (EL) | -1.2% | -1.1% | -1.3% | -0.3% | -0.5% | -0.3% | -1.1% | -2.0% | -2.1% |
| Enc Time[%] | 100.5% |  |  | 100.1% |  |  | 99.9% |  |  |
| Dec Time[%] | 107.4% |  |  | 101.9% |  |  | 104.3% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.4% | -1.6% | -2.0% |  |  |  | -1.3% | -2.2% | -3.0% |
| Class B | -0.4% | -1.1% | -0.9% | -0.3% | -1.0% | -0.9% | -0.8% | -2.0% | -1.7% |
| Overall (EL+BL) | -0.7% | -1.2% | -1.2% | -0.3% | -1.0% | -0.9% | -0.9% | -2.0% | -2.0% |
| Overall (EL) | -1.5% | -2.4% | -2.4% | -0.7% | -2.2% | -1.7% | -1.8% | -4.3% | -4.3% |
| Enc Time[%] | 100.3% |  |  | 100.2% |  |  | 100.3% |  |  |
| Dec Time[%] | 113.4% |  |  | 105.9% |  |  | 110.8% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 2.  BD rate comparison results of the second proposed method.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -1.3% | -2.2% | -2.4% |  |  |  |  |  |  |
| Class B | -0.1% | -0.5% | -0.6% | -0.1% | -0.2% | -0.3% |  |  |  |
| Overall (EL+BL) | -0.5% | -1.0% | -1.1% | -0.1% | -0.2% | -0.3% |  |  |  |
| Overall (EL) | -1.1% | -2.1% | -2.3% | -0.2% | -0.5% | -0.5% |  |  |  |
| Enc Time[%] | 99.6% |  |  | 99.6% |  |  |  |  |  |
| Dec Time[%] | 109.4% |  |  | 106.0% |  |  |  |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.7% | -1.0% | -1.5% |  |  |  | -1.1% | -1.3% | -1.8% |
| Class B | -0.2% | -0.3% | -0.2% | -0.1% | -0.2% | -0.1% | -0.3% | -0.5% | -0.3% |
| Overall (EL+BL) | -0.7% | -0.5% | -0.6% | -0.1% | -0.2% | -0.1% | -0.5% | -0.7% | -0.7% |
| Overall (EL) | -1.4% | -1.0% | -1.2% | -0.2% | -0.4% | -0.3% | -1.0% | -1.7% | -1.8% |
| Enc Time[%] | 99.9% |  |  | 99.5% |  |  | 99.7% |  |  |
| Dec Time[%] | 111.0% |  |  | 103.8% |  |  | 106.5% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.9% | -1.6% | -2.1% |  |  |  | -1.4% | -2.1% | -2.8% |
| Class B | -0.5% | -1.0% | -0.8% | -0.3% | -0.8% | -0.7% | -0.8% | -1.8% | -1.5% |
| Overall (EL+BL) | -0.9% | -1.2% | -1.1% | -0.3% | -0.8% | -0.7% | -1.0% | -1.9% | -1.9% |
| Overall (EL) | -1.8% | -2.3% | -2.2% | -0.7% | -1.8% | -1.2% | -1.9% | -4.0% | -3.9% |
| Enc Time[%] | 99.8% |  |  | 99.3% |  |  | 99.6% |  |  |
| Dec Time[%] | 116.2% |  |  | 105.3% |  |  | 112.3% |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# Conclusions

This contribution proposed two methods to perform inter-layer SAO. The first proposed method allows the inter-layer sample adaptive offset (SAO) to be applied on the processed picture iteratively. Simulation results of the first method show luma BD-rate savings of 0.5% for AI-2x, 0.1 for AI-1.5x, 0.6% for RA-2x, 0.1% for RA-1.5x, 0.5% for RA-SNR, 0.7% for LD-P-2x, 0.3% for LD-P-1.5x, and 0.9% for LD-P-SNR (BD-rate calculated using both enhancement layer and base layer rates). The encoding time increase is 0.6% on average and the decoding time increase is 6.9% on average. The second method allows all the SAO types to be applied. Experimental results of the second method show luma BD-rate savings of 0.5% for AI-2x, 0.1 for AI-1.5x, 0.7% for RA-2x, 0.1% for RA-1.5x, 0.5% for RA-SNR, 0.9% for LD-P-2x, 0.3% for LD-P-1.5x, 1.0% for LD-P-SNR (BD-rate calculated using both enhancement layer and base layer rates). The encoding time increase is negligible on average and the decoding time increase is 8.8% on average.

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Ximin Zhang, Shan Liu, Chih-Ming Fu, and Shawmin Lei, “TE4.3.1 Inter-layer SAO,” JCT-VC Document, JCTVC-L0212, Jan. 2013.