| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0284 |
| --- | --- |

| Title: | TE2: Results of Test 3.1.1 on Inter-layer Texture Prediction Signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo Vadim Seregin Jianle Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-845-7532 liweig@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the results of TE2 test 3.1.1 on CU level intra-BL mode coding. The test includes intra-BL Skip mode and the corresponding de-blocking BS setting for the new modes.

# Introduction

Inter-layer texture prediction is an important tool for scalable video coding where base layer reconstructed pixels are used to predict enhancement layer pixels. Inter-layer texture prediction is implemented using Intra-BL mode in HEVC scalable extension base software. This contribution describes Intra-BL Skip mode which was presented in [1]. As Intra-BL and Intra-BL Skip are newly introduced modes in scalable extension, the corresponding de-blocking boundary strength (BS) setting behavior [1] is also defined.

# Proposed methods

Besides Intra-BL mode, a new mode Intra-BL Skip mode is introduced. Both the signaling of Intra-BL and Intra-BL Skip is at CU level. A flag intra_bl_skip is first transmitted to indicate whether the CU mode is Intra-BL Skip. If intra_bl_skip is 1, no further information needs to be transmitted. The CU is reconstructed by copying the reconstructed base layer pixel values (may need up-sampling).  If intra_bl_skip is 0, syntax intra_bl_flag is transmitted to indicate if the CU mode is Intra-BL mode.

The de-blocking BS setting for Intra-BL and Intra-BL Skip modes is defined as follows. In case both blocks are coded with intra-BL skip mode, the edge is not processed; otherwise, if at least one of the blocks is coded as normal intra mode, the BS is set to 2; otherwise, if at least one of the blocks is coded in intra-BL mode, then the BS for the luma edge is set to 1, and the BS is set to 2 for the chroma edge.

# Experimental results

The proposed methods are implemented on HEVC base software version 0.1.1. Anchor is generated using HM 8.1 with simucast settings. Test conditions specified in [2] is used. Running time is measured on a homogenous cluster.

For 4x4 TU in Intra-BL mode, two different transform schemes are tested: A. both forward and backward transforms are DCT; B. both forward and backward transforms are DST. The results of using DCT and DST are presented in Table 1 and Table 2 respectively.

Table 1 Results of Test A

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -27.14% | -27.37% | -28.34% |  |  |  |
| Class B | -21.43% | -21.42% | -21.27% | -32.92% | -32.99% | -33.17% |
| Overall (EL+BL) | -23.06% | -23.12% | -23.29% | -32.92% | -32.99% | -33.17% |
| Overall (EL) | -35.20% | -35.46% | -35.77% | -58.66% | -58.19% | -58.56% |
| Enc Time[%] | 116.98% |  |  | 113.42% |  |  |
| Dec Time[%] | 102.65% |  |  | 99.53% |  |  |

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -18.47% | -7.89% | -9.46% |  |  |  | -21.55% | -12.06% | -11.79% |
| Class B | -15.72% | -9.19% | -7.82% | -26.31% | -20.03% | -18.65% | -19.59% | -10.84% | -8.03% |
| Overall (EL+BL) | -16.50% | -8.82% | -8.29% | -26.31% | -20.03% | -18.65% | -20.15% | -11.19% | -9.10% |
| Overall (EL) | -25.45% | -13.58% | -12.97% | -47.56% | -37.51% | -35.10% | -31.85% | -18.69% | -15.67% |
| Enc Time[%] | 105.76% |  |  | 108.80% |  |  | 104.97% |  |  |
| Dec Time[%] | 126.07% |  |  | 128.39% |  |  | 99.82% |  |  |

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -13.54% | -1.93% | -3.03% |  |  |  | -13.98% | -7.31% | -6.31% |
| Class B | -11.86% | -6.74% | -4.86% | -22.02% | -16.28% | -14.62% | -12.85% | -5.18% | -1.76% |
| Overall (EL+BL) | -12.34% | -5.36% | -4.34% | -22.02% | -16.28% | -14.62% | -13.17% | -5.79% | -3.06% |
| Overall (EL) | -19.70% | -8.33% | -6.97% | -40.86% | -30.84% | -27.82% | -21.18% | -10.99% | -7.08% |
| Enc Time[%] | 106.67% |  |  | 113.98% |  |  | 104.03% |  |  |
| Dec Time[%] | 134.54% |  |  | 134.11% |  |  | 104.01% |  |  |

Table 2 Results of Test B

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -27.16% | -27.38% | -28.33% |  |  |  |
| Class B | -21.48% | -21.44% | -21.29% | -32.96% | -32.96% | -33.15% |
| Overall (EL+BL) | -23.10% | -23.13% | -23.30% | -32.96% | -32.96% | -33.15% |
| Overall (EL) | -35.25% | -35.47% | -35.78% | -58.71% | -58.12% | -58.51% |
| Enc Time[%] | 118.45% |  |  | 114.31% |  |  |
| Dec Time[%] | 101.81% |  |  | 98.50% |  |  |

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -18.48% | -7.92% | -9.46% |  |  |  | -21.57% | -12.13% | -11.79% |
| Class B | -15.75% | -9.17% | -7.83% | -26.34% | -19.98% | -18.61% | -19.61% | -10.80% | -7.94% |
| Overall (EL+BL) | -16.53% | -8.81% | -8.30% | -26.34% | -19.98% | -18.61% | -20.17% | -11.18% | -9.04% |
| Overall (EL) | -25.48% | -13.57% | -12.99% | -47.59% | -37.39% | -35.01% | -31.89% | -18.68% | -15.57% |
| Enc Time[%] | 108.32% |  |  | 111.62% |  |  | 109.61% |  |  |
| Dec Time[%] | 126.28% |  |  | 126.81% |  |  | 99.20% |  |  |

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -13.56% | -1.98% | -3.15% |  |  |  | -14.01% | -7.30% | -6.35% |
| Class B | -11.88% | -6.72% | -4.92% | -22.02% | -16.29% | -14.58% | -12.86% | -5.18% | -1.68% |
| Overall (EL+BL) | -12.36% | -5.37% | -4.41% | -22.02% | -16.29% | -14.58% | -13.19% | -5.79% | -3.01% |
| Overall (EL) | -19.73% | -8.34% | -7.07% | -40.84% | -30.86% | -27.76% | -21.21% | -10.97% | -6.99% |
| Enc Time[%] | 108.14% |  |  | 115.48% |  |  | 106.77% |  |  |
| Dec Time[%] | 134.31% |  |  | 133.15% |  |  | 101.52% |  |  |

# Syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| CurrCbAddrTS = MinCbAddrZS[ x0 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] |  |
| intra_bl_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( !intra_bl_skip_flag[ x0 ][ y0 ] ) { | ae(v) |
| if( slice_type != I ) { |  |
| skip_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| intra_bl_flag | ae(v) |
| …… |  |
| } |  |

# References

- J. Chen, et al. “Description of scalable video coding technology proposal by Qualcomm (configuration 1)”, JCTVC-K0035, Shanghai, China, Oct. 2012.

- L. Guo, Y. He, D. -K. Kwon, J. Zan, H. Lakshman, and J.-W. Kang, “Description of Tool Experiment A2: Inter-layer Texture Prediction Signaling in SHVC”, JCTVC-K1102, Shanghai, China, Oct. 2012.

# Patent rights declaration(s)

Qualcomm Incorporated may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).