| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0285 |
| --- | --- |

| Title: | TE A2: Signalling of Inter-layer Skip and Direct |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenhao Zhang Yi-Jen Chiu Lidong Xu Yu Han Zhipin Deng Xiaoxia Cai | Email: | wenhao.zhang@intel.com yi-jen.chiu@intel.com lidong.xu@intel.com yu.han@intel.com zhipin.deng@intel.com xiaoxia.cai@intel.com |
| Source: | Intel Corporation |  |  |

_____________________________

# Abstract

This contribution presents a CU level inter-layer texture prediction scheme for SHVC. It utilizes the reconstructed sample pixels of collocated base layer block to produce the predictive signal to encode the enhancement layer CU. To signal this coding tool, two CU level syntax flags are proposed: 1) inter_layer_skip_flag indicates if the inter-layer texture prediction is applied to the current CU or not; 2) inter_layer_direct_flag indicates if current CU has residual signal when inter_layer_skip_flag is equal to 1. If residual exists, it will be transformed, quantized and entropy coded as an intra-coded CU. Compared to the simulcast anchor, the proposed method achieves 22.8%, 32.6%, 16.4%, 26.1%, 20.0%, 12.3%, 21.9% and 13.1% EL+BL BD-bitrate reduction under the configurations of AI_2x, AI_1.5x, RA_2x, RA_1.5x, RA_SNR, LP_2x, LP_1.5x and LP_SNR.

# Introduction

This contribution describes an inter-layer texture prediction method for SHVC. Inter-layer texture prediction utilizes the reconstructed sample pixels of base layer (BL) to produce the predictive signal to encode the collocated pixels in enhancement layer (EL). The proposed scheme applies at CU level by a signal flag, denoted as inter_layer_skip_flag, to be added to CU syntax table to indicate if the current CU to apply inter-layer pixel prediction or not. When inter_layer_skip_flag is equal to 1, there is a flag, denoted as inter_layer_direct_flag, to indicate if residual signal exists or not for current CU. With these two flags, two new coding modes are proposed for CU coding of EL video.

Table 1 - Two inter-layer texture predicted coding modes

|  | inter_layer_skip_flag | inter_layer_direct_flag |  |
| --- | --- | --- | --- |
| Inter-layer skip mode | 1 | 1 | Without residual transmitted |
| Inter-layer direct mode | 1 | 0 | With residual |

In the proposed EL CU level syntax table, the inter_layer_skip_flag is inserted before the traditional skip flag. The inter_layer_direct_flag is only signaled when inter_layer_skip_flag is equal to 1. The resulting CU decoding flowchart is shown in Figure 1.

Figure 1 - Flowchart of CU decoding process with inter-layer texture prediction

# Experiment results

The proposed inter-layer texture prediction is implemented in the SMuC0.1.1 by replacing the existing IntraBL tool of SMuC0.1.1. As specified in the TE description [1], sequences of Class A and Class B are tested by using the configuration of AI_2x, AI_1.5x, RA_2x, RA_1.5x, RA_SNR, LP_2x, LP_1.5x and LP_SNR. Simulcast coding results are used as anchor to evaluate the proposed method. The average BD-bitrate reduction and coding time are shown in Table 2 to Table 4. The per-sequence bit rate and PSNR results are provided in the accompanied spreadsheet.

Table 2 - Average results of all intra scenarios (simulcast anchor)

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -27.0% | -27.2% | -28.1% |  |  |  |
| Class B | -21.1% | -21.2% | -21.0% | -32.7% | -32.9% | -33.0% |
| Overall (EL+BL) | -22.8% | -23.0% | -23.0% | -32.7% | -32.9% | -33.0% |
| Overall (EL) | -34.8% | -35.2% | -35.4% | -58.4% | -58.1% | -58.4% |
| Enc Time[%] | 115.1% |  |  | 113.3% |  |  |
| Dec Time[%] | 94.0% |  |  | 90.8% |  |  |

Table 3 - Average results of random access scenarios (simulcast anchor)

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -18.5% | -7.8% | -9.3% |  |  |  | -21.5% | -11.9% | -11.5% |
| Class B | -15.6% | -9.1% | -7.7% | -26.3% | -20.1% | -18.6% | -19.4% | -10.9% | -7.9% |
| Overall (EL+BL) | -16.4% | -8.7% | -8.2% | -26.3% | -20.1% | -18.6% | -20.0% | -11.2% | -8.9% |
| Overall (EL) | -25.4% | -13.4% | -12.8% | -47.8% | -37.6% | -35.2% | -31.7% | -18.7% | -15.4% |
| Enc Time[%] | 113.0% |  |  | 110.6% |  |  | 105.0% |  |  |
| Dec Time[%] | 117.8% |  |  | 119.2% |  |  | 107.4% |  |  |

Table 4 - Average results of low delay P scenarios (simulcast anchor)

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -13.7% | -1.6% | -2.7% |  |  |  | -14.0% | -6.9% | -6.0% |
| Class B | -11.9% | -6.4% | -4.6% | -22.2% | -16.4% | -14.7% | -12.9% | -5.1% | -1.5% |
| Overall (EL+BL) | -12.4% | -5.1% | -4.1% | -22.2% | -16.4% | -14.7% | -13.2% | -5.6% | -2.8% |
| Overall (EL) | -19.9% | -7.9% | -6.5% | -41.5% | -31.2% | -28.0% | -21.3% | -10.7% | -6.7% |
| Enc Time[%] | 112.3% |  |  | 111.0% |  |  | 104.1% |  |  |
| Dec Time[%] | 118.8% |  |  | 121.0% |  |  | 104.3% |  |  |

Table 5 to Table 7 provide additional results for informative purpose, in which the existing IntraBL in SMuC0.1.1 is used as anchor.

Table 5 - Average results of all intra scenarios (SMuC0.1.1 anchor)

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | -0.1% | 0.1% | 0.0% | -0.1% | 0.3% | 0.2% |
| Overall (EL+BL) | 0.0% | 0.1% | 0.0% | -0.1% | 0.3% | 0.2% |
| Overall (EL) | -0.2% | 0.2% | 0.1% | -0.8% | 2.1% | 1.6% |
| Enc Time[%] | 101.1% |  |  | 101.1% |  |  |
| Dec Time[%] | 101.6% |  |  | 102.7% |  |  |

Table 6 - Average results of random access scenarios (SMuC0.1.1 anchor)

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.2% | 0.1% |  |  |  | -0.1% | 0.3% | 0.3% |
| Class B | -0.1% | 0.3% | 0.3% | -0.3% | 0.5% | 0.3% | -0.1% | 0.4% | 0.4% |
| Overall (EL+BL) | -0.1% | 0.3% | 0.2% | -0.3% | 0.5% | 0.3% | -0.1% | 0.4% | 0.4% |
| Overall (EL) | -0.4% | 0.6% | 0.4% | -1.3% | 1.5% | 0.7% | -0.3% | 0.9% | 0.9% |
| Enc Time[%] | 100.1% |  |  | 99.9% |  |  | 100.2% |  |  |
| Dec Time[%] | 100.8% |  |  | 97.7% |  |  | 105.1% |  |  |

Table 7 - Average results of low delay P scenarios (SMuC0.1.1 anchor)

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.7% | 0.6% |  |  |  | -0.1% | 0.5% | 0.5% |
| Class B | -0.2% | 0.7% | 0.7% | -0.4% | 1.0% | 0.8% | -0.2% | 0.7% | 0.7% |
| Overall (EL+BL) | -0.2% | 0.7% | 0.7% | -0.4% | 1.0% | 0.8% | -0.1% | 0.7% | 0.7% |
| Overall (EL) | -0.5% | 1.4% | 1.3% | -1.9% | 3.1% | 2.3% | -0.4% | 1.2% | 1.2% |
| Enc Time[%] | 99.8% |  |  | 99.5% |  |  | 97.3% |  |  |
| Dec Time[%] | 99.6% |  |  | 101.3% |  |  | 108.9% |  |  |

# References

- Liwei Guo, Yong He, Do-Kyoung Kwon, Jinwen Zan, Haricharan Lakshman, Jung Won Kang, “Description of Tool Experiment A2: Inter-layer Texture Prediction Signaling in SHVC”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 11th Meeting, JCTVC-K1102, Shanghai, CN, 10–19 Oct. 2012.

# Patent rights declaration(s)

Intel Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).