| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0287 |
| --- | --- |

| Title: | TE B4: Inter-layer adaptive refine filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenhao Zhang Lidong Xu Yu Han Zhipin Deng Xiaoxia Cai Yi-Jen Chiu | Email: | wenhao.zhang@intel.com lidong.xu@intel.com yu.han@intel.com zhipin.deng@intel.com xiaoxia.cai@intel.com yi-jen.chiu@intel.com |
| Source: | Intel Corporation |  |  |

_____________________________

# Abstract

This contribution presents an inter-layer adaptive refining filter scheme for SHVC. 2D 13-tap filter with diamond shape is proposed to process the inter-layer texture predicted pixel samples. It can improve the accuracy of inter-layer texture prediction to achieve better compression performance. For each picture, two filter sets are applied to Luma component and chroma component, respectively. The filter coefficients are coded and transmitted in the slice header. Furthermore, a CU level syntax flag is used to indicate if the proposed refining filter is applied to the current CU or not. Compared to the anchor results of SMuC0.1.1, the proposed method achieves 0.2%, 0.4%, 0.0%, 0.1%, 1.7%, 0.2%, 0.4% and 2.9% EL+BL BD-bitrate reduction under the configurations of AI_2x, AI_1.5x, RA_2x, RA_1.5x, RA_SNR, LP_2x, LP_1.5x and LP_SNR.

# Introduction

This contribution describes an inter-layer adaptive filtering scheme for SHVC. With the inter-layer texture prediction to produce the predicted pixel samples based on the reconstructed pixels from BL, the predicted pixel samples can be further processed by an inter-layer refining filter. This refining filter can improve the accuracy of inter-layer prediction to achieve better compression performance for video in EL. This contribution applies a 13-tap 2D filter on each predicted pixel sample value. The refining filter has the diamond shape as shown in the following figureFigure ,

.

Figure 3 Filter window of inter-layer refining filter

P0 to P12 indicate the predicted pixel samples before filtering. Assume P0’ is the refined value of the center pixel P0, the filtering process can be expressed as the following equation,

P0’ = C0* P0 + C1* P1 + … + C12* P12 	(1)

where C0 to C12 are the filter coefficients. The filter coefficients are produced by encoder, and are adaptively refreshed to decoder according to the characteristics of the input video. This contribution uses picture-level refreshing of the filter by coding the coefficients in the first slice header of each picture in EL video.

In order to reduce the overhead of transmitting the coefficients, the refining filter is implemented in symmetry. Specifically, the filter taps whose positions are with respect to center with symmetry share the same coefficients, as described in following equations,

C0

C1 = C2;

C3 = C4;

Filter coefficients include:	C5 = C6;	(2)

C7 = C8;

C9 = C10;

C11 = C12.

With symmetry, the number of filter coefficients is reduced from 13 to 7. In this contribution, two sets of refining filter are used by Luma component and Chroma components, respectively. The total number of coefficients for each picture of EL video is equal to 14.

A CU level flag is proposed to indicate if the current CU uses refining filter or not. This flag is only coded when the CU is using inter-layer texture prediction.

# Experiment results

The proposed inter-layer texture prediction is implemented in the SMuC0.1.1. As specified in the TE description [1], sequences of Class A and Class B are tested by using the configuration of AI_2x, AI_1.5x, RA_2x, RA_1.5x, RA_SNR, LP_2x, LP_1.5x and LP_SNR. Coding results of SMuC0.1.1are used as anchor to evaluate the proposed method. The average BD-bitrate reduction and coding time are shown in Table 1 to Table 3. The per-sequence bit rate and PSNR results are provided in the accompanied spreadsheet.

Table 1 - Average results of all intra scenarios

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.6% | -0.7% |  |  |  |
| Class B | -0.3% | -0.4% | -0.4% | -0.4% | -0.9% | -1.0% |
| Overall (EL+BL) | -0.2% | -0.5% | -0.5% | -0.4% | -0.9% | -1.0% |
| Overall (EL) | -0.3% | -1.0% | -1.1% | -1.1% | -3.1% | -3.2% |
| Enc Time[%] | 116.7% |  |  | 113.7% |  |  |
| Dec Time[%] | 99.5% |  |  | 100.8% |  |  |

Table 2 - Average results of random access scenarios

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.2% | -0.3% |  |  |  |
| Class B | 0.0% | -0.1% | 0.0% | -0.1% | -0.4% | -0.3% |
| Overall (EL+BL) | 0.0% | -0.1% | -0.1% | -0.1% | -0.4% | -0.3% |
| Overall (EL) | 0.1% | -0.3% | -0.2% | -0.2% | -1.2% | -0.4% |
| Enc Time[%] | 104.9% |  |  | 104.0% |  |  |
| Dec Time[%] | 97.9% |  |  | 95.4% |  |  |
|  | RA HEVC SNR |  |  |  |  |  |
|  | Y | U | V |  |  |  |
| Class A | -3.9% | -1.2% | -1.0% |  |  |  |
| Class B | -0.8% | -1.0% | -1.0% |  |  |  |
| Overall (EL+BL) | -1.7% | -1.0% | -1.0% |  |  |  |
| Overall (EL) | -3.5% | -2.0% | -1.9% |  |  |  |
| Enc Time[%] | 104.1% |  |  |  |  |  |
| Dec Time[%] | 100.8% |  |  |  |  |  |

Table 3 - Average results of low delay P scenarios

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.3% | -0.4% |  |  |  |
| Class B | -0.3% | -0.6% | -0.2% | -0.4% | -0.8% | -0.3% |
| Overall (EL+BL) | -0.2% | -0.5% | -0.3% | -0.4% | -0.8% | -0.3% |
| Overall (EL) | -0.4% | -1.1% | -0.6% | -1.1% | -2.6% | -1.1% |
| Enc Time[%] | 105.2% |  |  | 104.7% |  |  |
| Dec Time[%] | 96.3% |  |  | 97.7% |  |  |
|  | LD-P HEVC SNR |  |  |  |  |  |
|  | Y | U | V |  |  |  |
| Class A | -5.5% | -2.3% | -2.2% |  |  |  |
| Class B | -1.8% | -1.4% | -0.6% |  |  |  |
| Overall (EL+BL) | -2.9% | -1.7% | -1.0% |  |  |  |
| Overall (EL) | -5.3% | -3.5% | -2.5% |  |  |  |
| Enc Time[%] | 104.4% |  |  |  |  |  |
| Dec Time[%] | 102.6% |  |  |  |  |  |

# References

- Jianle Chen, Andrew Segall, Elena Alshina, Shan Liu, Jie Dong, Joonyoung Park, “Description of Tool Experiment A2: Inter-layer Texture Prediction Signaling in SHVC”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 11th Meeting, JCTVC-K1104, Shanghai, CN, 10–19 Oct. 2012.

# Patent rights declaration(s)

Intel Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).