| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0288 |
| --- | --- |

| Title: | TE C5: Inter-layer motion prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenhao Zhang Lidong Xu Yu Han Zhipin Deng Xiaoxia Cai Yi-Jen Chiu | Email: | wenhao.zhang@intel.com lidong.xu@intel.com yu.han@intel.com zhipin.deng@intel.com xiaoxia.cai@intel.com yi-jen.chiu@intel.com |
| Source: | Intel Corporation |  |  |

_____________________________

# Abstract

This contribution presents an inter-layer motion parameter prediction scheme for SHVC. For Merge mode and AMVP mode, the motion information of BL, which aligns to the right-bottom corner of current PU in EL, is extracted and is placed as the first entry in the candidate list. With this motion information from the co-located BL added to the set of the motion information from EL, the maximum number of motion candidates is increased from 5 to 6 for Merge PUs and is increased from 2 to 3 for AMVP PUs. Compared to the SMuC0.1.1 anchor, the proposed method achieves 1.2%, 1.9%, 2.1%, 0.4%, 0.9% and 1.5% EL+BL BD-bitrate reduction under the configurations of RA_2x, RA_1.5x, RA_SNR, LP_2x, LP_1.5x and LP_SNR.

# Algorithm

It’s observed that there is a correlation for the motion patterns of the video object between BL video and EL video. The motion information of BL can be used as the prediction signal to encode for the video in EL. The motion parameters of BL such as the inter prediction direction, reference index and motion vectors, can be used as the additional candidate information to encoder for the inter coding for EL video. For Merge mode and AMVP mode, the motion information of BL, which aligns to the right-bottom corner of current PU in EL, is extracted and is placed as the first entry in the candidate list. With this motion information from the co-located BL added to the set of the motion information from EL, the maximum number of motion candidates is increased from 5 to 6 for Merge PUs and is increased from 2 to 3 for AMVP PUs. Figure 1 and Figure 2 pictorially illustrate the motion information candidates to use for the coding of video in EL.

Figure 1 - Merge candidates in coding video in EL

As shown in Figure 1, Merge candidate list is set up with the following four steps,

Step 1: Check motion parameter of A from the collocated block in BL;

Step 2: Check motion parameters of spatial neighbors B, C, D and E. If one of them is unavailable, check spatial neighbor F;

Step 3: Check motion parameter of temporal neighbor G;

Step 4: Process the candidates in steps 1-3 by pruning the duplicates to limit the list size equal to 6.

Figure 2 - AMVP candidates in coding video in EL

As shown in Figure 2, AMVP candidate list is set up with the following five steps,

Step 1: Check motion parameter of A from the collocated block in BL;

Step 2: Check motion parameter of the first left spatial neighbors in the order of B0, B1;

Step 3: Check motion parameter of the first top spatial neighbors in the order of C0, C1 and C2;

Step 4: Check motion parameter of temporal neighbor D;

Step 5: Process the candidates in steps 1-4 by pruning the duplicates to limit the list size equal to 3.

# Experiment results

The proposed inter-layer residual refinement is implemented in the SMuC0.1.1. As specified in the TE description [1], sequences of Class A and Class B are tested by using the configuration of RA_2x, RA_1.5x, RA_SNR, LP_2x, LP_1.5x and LP_SNR. The average BD-bitrate reduction and coding time are shown in Table 1 and Table 2. The per-sequence bit rate and PSNR results are provided in the accompanied spreadsheet.

Table 1 - Average results of random access scenarios

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.9% | -2.1% | -2.0% |  |  |  | -2.2% | -4.5% | -4.8% |
| Class B | -1.3% | -2.4% | -2.4% | -1.9% | -3.0% | -3.2% | -2.1% | -3.5% | -3.6% |
| Overall (EL+BL) | -1.2% | -2.3% | -2.3% | -1.9% | -3.0% | -3.2% | -2.1% | -3.8% | -4.0% |
| Overall (EL) | -2.4% | -4.3% | -4.4% | -5.2% | -7.5% | -8.1% | -4.4% | -7.3% | -7.7% |
| Enc Time[%] | 98.1% |  |  | 95.9% |  |  | 99.0% |  |  |
| Dec Time[%] | 93.1% |  |  | 94.5% |  |  | 95.8% |  |  |

Table 2 - Average results of low delay P scenarios

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.8% | -0.7% |  |  |  | -1.6% | -2.6% | -2.5% |
| Class B | -0.5% | -1.3% | -1.3% | -0.9% | -1.5% | -1.6% | -1.4% | -2.1% | -2.2% |
| Overall (EL+BL) | -0.4% | -1.1% | -1.1% | -0.9% | -1.5% | -1.6% | -1.5% | -2.2% | -2.3% |
| Overall (EL) | -0.7% | -2.0% | -1.9% | -1.9% | -3.2% | -3.5% | -2.6% | -3.9% | -4.0% |
| Enc Time[%] | 99.2% |  |  | 97.3% |  |  | 99.8% |  |  |
| Dec Time[%] | 94.4% |  |  | 94.5% |  |  | 96.4% |  |  |

# References

- Vadim Seregin, Patrice Onno, Shan Liu, Tammy Lee, Chulkeun Kim, Haitao Yang, Haricharan Laksman, “Description of Tool Experiment C5: Inter-layer syntax prediction using HEVC base layer”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 11th Meeting, JCTVC-K1103, Shanghai, CN, 10–19 Oct. 2012.

# Patent rights declaration(s)

Intel Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).