| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0289 |
| --- | --- |

| Title: | TE C5: Inter-layer pattern/mode prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenhao Zhang Lidong Xu Yu Han Zhipin Deng Xiaoxia Cai Yi-Jen Chiu | Email: | wenhao.zhang@intel.com lidong.xu@intel.com yu.han@intel.com zhipin.deng@intel.com xiaoxia.cai@intel.com yi-jen.chiu@intel.com |
| Source: | Intel Corporation |  |  |

_____________________________

# Abstract

This contribution presents an inter-layer pattern/mode prediction scheme for SHVC. To encode a CU in EL, the information of sub-CU quadtree structure, PU partitions and intra/inter prediction modes can be derived from co-located blocks in BL when all of the co-located BL blocks belong to a single CU. A CU level control flag is added in CU syntax table to indicate if current CU uses inter-layer pattern/mode prediction or not. The proposed method can be used for 2x spatial scalability and SNR scalability. Compared to the SMuC0.1.1 anchor, the proposed method achieves 0.0%, 0.5%, 1.4%, 0.2% and 0.8% EL+BL BD-bitrate reduction under the configurations of AI_2x, RA_2x, RA_SNR, LP_2x and LP_SNR.

# Algorithm

Inter-layer pattern/mode prediction utilizes the splitting pattern and prediction information of BL to encode video in EL. To encode a CU in EL, the information of sub-CU quadtree structure, PU partitions and intra/inter prediction modes can be derived from collocated blocks in BL when all of the collocated BL blocks belong to a single CU. With the quadtree pattern projection from BL to EL, the partition sizes and prediction modes of each node CU in BL can be copied to the collocated CU in EL. Inter-layer pattern prediction transmits only the transform parameter and residual for the target CU in EL. Inter-layer pattern/mode prediction is an additional CU coding mode to code video in EL. A CU level control flag, denoted as inter_layer_pattern_mode_pred_flag, is added in CU syntax table to indicate if current CU uses inter-layer pattern/mode prediction or not. With this flag, a new coding mode called ILPMPred mode, is proposed for CU coding of EL video. Figure 1 illustrates an example of inter-layer pattern/mode prediction.

Figure 1 - An example of inter-layer pattern/mode prediction

For spatial scalability, it’s required to up-scale the pattern/mode parameters of collocated BL block based on the spatial ratio between EL and BL.

# Experiment results

The proposed inter-layer pattern/mode prediction is implemented in the SMuC0.1.1. As specified in the TE description [1], sequences of Class A and Class B are tested by using the configuration of RA_2x , RA_2x, RA_SNR, LP_2x and LP_SNR. The average BD-bitrate reduction and coding time are shown in Table 1 to Table 3. The per-sequence bit rate and PSNR results are provided in the accompanied spreadsheet.

Table 1 - Average results of all intra scenarios

|  | AI HEVC 2x |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% |
| Overall (EL+BL) | 0.0% | 0.0% | 0.0% |
| Overall (EL) | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 104.2% |  |  |
| Dec Time[%] | 96.4% |  |  |

Table 2 - Average results of random access scenarios

|  | RA HEVC 2x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.6% | -3.3% | -3.1% | -1.7% | -3.9% | -4.1% |
| Class B | 0.0% | 0.0% | 0.0% | -1.3% | -2.5% | -2.4% |
| Overall (EL+BL) | -0.5% | -0.9% | -0.9% | -1.4% | -2.9% | -2.9% |
| Overall (EL) | -1.0% | -1.9% | -1.9% | -3.3% | -6.2% | -6.2% |
| Enc Time[%] | 100.6% |  |  | 101.7% |  |  |
| Dec Time[%] | 95.2% |  |  | 95.3% |  |  |

Table 3 - Average results of low delay P scenarios

|  | LD-P HEVC 2x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.8% | -1.8% | -1.6% | -1.2% | -2.6% | -2.5% |
| Class B | 0.0% | 0.0% | 0.0% | -0.6% | -1.5% | -1.5% |
| Overall (EL+BL) | -0.2% | -0.5% | -0.5% | -0.8% | -1.8% | -1.8% |
| Overall (EL) | -0.5% | -1.0% | -0.9% | -1.7% | -3.7% | -3.5% |
| Enc Time[%] | 100.8% |  |  | 101.8% |  |  |
| Dec Time[%] | 94.3% |  |  | 97.5% |  |  |

# References

- Vadim Seregin, Patrice Onno, Shan Liu, Tammy Lee, Chulkeun Kim, Haitao Yang, Haricharan Laksman, “Description of Tool Experiment C5: Inter-layer syntax prediction using HEVC base layer”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 11th Meeting, JCTVC-K1103, Shanghai, CN, 10–19 Oct. 2012.

# Patent rights declaration(s)

Intel Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).