| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0291 |
| --- | --- |

| Title: | Non-TE C5: Inter-layer motion data inheritance |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng, Wenhao Zhang, Lidong Xu, Yu Han, Xiaoxia Cai, and Yi-Jen Chiu | Tel: Email: | 408-765-9494 (Yi-Jen Chiu) {zhipin.deng, wenhao.zhang, lidong.xu, xiaoxia.cai, yu.han, yi-jen.chiu }@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

In this contribution, an inter-layer motion data inheritance (MDI) mode is studied as an additional coding mode to improve the coding efficiency of SHVC. The MDI mode applies the motion data of the collocated base layer block to predict for the enhancement layer block coding. This MDI mode is only applied to inter 2Nx2N PU and the motion data are copied with NxN granularity. The memory buffer access required for the motion data is reduced when compared to the inter-layer inferred prediction mode with finer granularity such as 4x4 or 8x8 granularity, and the data process parallelism can be improved marginally. The experimental results show that the proposed method achieves average 2.3% and 1.7% BD-rate savings for RA and LD-P cases compared to SMuC0.1.1. (The results here were obtained by 1/10 test clips, we will update the results with full clips soon.)

# Introduction

In our previous proposal in [1], an inter-layer pattern/mode prediction was proposed using the splitting pattern and prediction information of base layer (BL) blocks to predict enhancement layer (EL) blocks. Extra buffers are required to store the quardtree structure information and the memory accesses for the finer granular motion data are not to be neglected. In this contribution, we propose an inter-layer MDI method to directly copy the motion vector, reference index and inter direction of the collocated BL block, as shown in Figure 1. This method is only applied to inter 2Nx2N PU and the motion data are copied with NxN granularity. Thus the memory buffer for quardtree structure of the base layer picture is totally removed and the memory accesses for motion data are also reduced as compared to the inter-layer inferred prediction mode with 4x4 or 8x8 granular motion data reusing.

Figure 1: The block diagram of the proposed inter-layer motion data inheritance

# Proposed algorithm

In this contribution, an inter_layer_MDI_flag is applied to each inter 2Nx2N PU to signal if the inter-layer MDI is used or not. If the inter_layer_MDI_flag is true, the motion data of the collocated BL block are directly inherited for EL block and then perform the motion compensation. As shown in Figure 2, an EL block is divided into average four NxN sub-blocks, and the center point of each NxN sub-block is used to find the collocated BL block. Since the inherited motion granularity is NxN, each inter 2Nx2N PU has 4 inherited motion vectors. The motion vectors are scaled in the case of spatial 2x and 1.5x scalabilities. Besides, if motion data of the collocated NxN BL block is not available, the collocated NxN block in the BL reconstructed picture will be copied as the prediction of the current EL NxN block.

16

16

16

16

BL picture

EL picture

Current 2Nx2N PU

(2N = 16)

Find collocated BL block

Figure 2: The derivation of collocated BL blocks in the case of 2x scalability

# Simulation results

The proposed method is integrated into the SMuC software 0.1.1 [2] and the tests are pursued under the common test conditions [3].

Table 1 and Table 2 give the results of Test 1 which applies the proposed algorithm to all sizes of inter 2Nx2N PU, which means the proposed algorithm is applied to inter 8x8, inter 16x16, inter 32x32, and inter 64x64 PUs. It is reported that as compared to SMuC 0.1.1, the proposed method of Test 1 achieves 2.0%, 2.0%, and 2.3% of EL+BL BD bitrate reduction for RA_2x, RA_1.5x, and RA_SNR cases, respectively. And for LD-P_2x, LD-P _1.5x, and LD-P_SNR cases, the BD bitrate reductions of Test 1 are 1.0%, 1.0%, and 1.7%, respectively. Compared to the inter-layer inferred prediction mode with finer granularity such as 4x4 or 8x8 granularity, NxN motion inheritance granularity reduces the memory buffer access and improves the data process parallelism (one SIMD can process more data sine all pixels in an NxN block have the same motion). (The results here were obtained by 1/10 test clips, we will update the results with full clips soon.)

In order to further reduce the memory accesses, we also run Test 2 in which the inter 8x8 PU only copies one motion vector, while other PU sizes such as inter 16x16, inter 32x32, and inter 64x64 PUs still copy 4 motion vectors as Test 1. As compared to SMuC 0.1.1, the results in Table 3 and Table 4 show that the proposed method of Test 1 achieves 2.0%, 1.9%, and 2.1% of EL+BL BD bitrate reduction for RA_2x, RA_1.5x, and RA_SNR cases, respectively. And for LD-P_2x, LD-P _1.5x, and LD-P_SNR cases, the BD bitrate reductions of Test 1 are 1.0%, 0.9%, and 1.5%, respectively. The detail data can be found in the spreadsheet of JCTVC-LXXXX.xls. (The results here were obtained by 1/10 test clips, we will update the results with full clips soon.)

Table 1: simulation results of RA configuration of Test 1 (To be updated)

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -2.0% | -4.5% | -4.5% |  |  |  | -2.3% | -6.5% | -6.8% |
| Class B | -2.0% | -3.7% | -3.9% | -2.0% | -4.0% | -4.2% | -2.2% | -4.2% | -4.2% |
| Overall (EL+BL) | -2.0% | -3.9% | -4.1% | -2.0% | -4.0% | -4.2% | -2.3% | -4.8% | -4.9% |
| Overall (EL) | -3.9% | -7.7% | -7.9% | -5.5% | -10.4% | -10.4% | -4.9% | -9.9% | -10.1% |
| Enc Time[%] |  |  |  |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |  |  |  |
| Enc Mem[%] |  |  |  |  |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 2: simulation results of LD-P configuration of Test 1 (To be updated)

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.1% | -2.6% | -2.4% |  |  |  | -1.9% | -4.7% | -4.9% |
| Class B | -1.0% | -1.9% | -1.9% | -1.0% | -1.8% | -2.0% | -1.6% | -2.5% | -2.7% |
| Overall (EL+BL) | -1.0% | -2.1% | -2.0% | -1.0% | -1.8% | -2.0% | -1.7% | -3.1% | -3.3% |
| Overall (EL) | -1.9% | -4.1% | -3.9% | -2.4% | -4.3% | -4.7% | -3.2% | -6.1% | -6.4% |
| Enc Time[%] |  |  |  |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |  |  |  |
| Enc Mem[%] |  |  |  |  |  |  |  |  |  |
| BL Match | Matched |  |  | Not matched |  |  | Matched |  |  |

Table 3: simulation results of RA configuration of Test 2 (To be updated)

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -2.0% | -4.5% | -4.5% |  |  |  | -2.1% | -5.6% | -6.0% |
| Class B | -2.0% | -3.7% | -3.9% | -1.9% | -3.7% | -3.9% | -2.1% | -4.0% | -4.0% |
| Overall (EL+BL) | -2.0% | -3.9% | -4.1% | -1.9% | -3.7% | -3.9% | -2.1% | -4.5% | -4.6% |
| Overall (EL) | -3.9% | -7.7% | -7.9% | -5.3% | -9.7% | -9.9% | -4.6% | -9.3% | -9.5% |
| Enc Time[%] |  |  |  |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |  |  |  |
| Enc Mem[%] |  |  |  |  |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 4: simulation results of LD-P configuration of Test 2 (To be updated)

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.1% | -2.6% | -2.4% |  |  |  | -1.5% | -3.8% | -3.9% |
| Class B | -1.0% | -1.9% | -1.9% | -0.9% | -1.7% | -2.0% | -1.4% | -2.3% | -2.4% |
| Overall (EL+BL) | -1.0% | -2.1% | -2.0% | -0.9% | -1.7% | -2.0% | -1.5% | -2.7% | -2.9% |
| Overall (EL) | -1.9% | -4.1% | -3.9% | -2.3% | -4.1% | -4.5% | -2.9% | -5.3% | -5.6% |
| Enc Time[%] |  |  |  |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |  |  |  |
| Enc Mem[%] |  |  |  |  |  |  |  |  |  |
| BL Match | Matched |  |  | Not matched |  |  | Matched |  |  |

# Conclusions

In this contribution, an inter-layer motion data inheritance (MDI) mode is studied as an additional coding mode to improve the coding efficiency of SHVC. The MDI mode applies the motion data of the collocated base layer block to predict for the enhancement layer block coding. This MDI mode is only applied to inter 2Nx2N PU and the motion data are copied with NxN granularity. The memory buffer access required for the motion data is reduced when compared to the inter-layer inferred prediction mode with finer granularity such as 4x4 or 8x8 granularity, and the data process parallelism can be improved marginally (one SIMD can process more data sine all pixels in an NxN block have the same motion). The experimental results show that the proposed method achieves average 2.3% and 1.7% BD-rate savings for RA and LD-P cases compared to SMuC0.1.1. (The results here were obtained by 1/10 test clips, we will update the results with full clips soon.)

# References

- J. Chen, V. Seregin, M. Zhou, B. Jeon, S. Lei, E. Nassor, K. Ugur, A. Segall, A. Tabatabai, H. Yu, J.W. Kang, Y. Ye, W. Zhang, P. Bordes, “A proposal for Scalable HEVC Test Model”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 11th Meeting, JCTVC-K0348, Shanghai, CN, 10-19 Oct., 2012.

- SMuC Software version 0.1.1: https://hevc.hhi.fraunhofer.de/svn/svn_SMuCSoftware/tags/0.1.1/

- Description of Tool Experiment C5: Inter-layer syntax prediction using HEVC base layer. version 15, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-K1105, Shanghai, China, October 2012.

# Patent rights declaration(s)

Intel Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).