| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0291 |
| --- | --- |

| Title: | Non-TE5: Inter-layer motion data inheritance by Intel |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng, Wenhao Zhang, Lidong Xu, Yu Han, Xiaoxia Cai, and Yi-Jen Chiu | Tel: Email: | 408-765-9494 (Yi-Jen Chiu) {zhipin.deng, wenhao.zhang, lidong.xu, xiaoxia.cai, yu.han, yi-jen.chiu }@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

In this contribution, an inter-layer motion data inheritance (MDI) technique is studied to improve the coding efficiency of SHVC. The proposed MDI is only applied to inter 2Nx2N PU of EL. It partitions a 2Nx2N PU into four NxN sub-PUs, and then the copy of collocated BL MV and the motion compensation are all performed at NxN granularity. Compared to the 4x4/8x8 granularity based inter-layer motion data prediction, the proposed MDI has better memory access efficiency and parallel performance of motion compensation for large PUs. In addition, the proposed MDI limits the 8x8 PU only copy one MV from BL, so that it doesn’t worsen EL’s worst case of memory access during motion compensation. The experimental results show that the proposed method can achieve 2.1%/2.1%/2.3% BD-rate savings for RA_2x/RA_1.5x/RA_SNR cases, and 1.2%/1.1%1.8% BD-rate savings for LP_2x/LP_1.5x/LP_SNR cases, compared to SMuC0.1.1.

# Introduction

In our previous proposal in [1], an inter-layer pattern/mode prediction was proposed using the splitting pattern and prediction information of base layer (BL) blocks to predict enhancement layer (EL) blocks. Extra buffers are required to store the quardtree structure information and the inter-layer memory accesses for the fine granular motion data are not to be ignored. In this contribution, we propose an inter-layer MDI method to directly copy the motion vector, reference index and inter direction of the collocated BL block, as shown in Figure 1. This method is only applied to inter 2Nx2N PU and the motion data are copied with NxN granularity. Thus the memory buffer for quardtree structure of the base layer picture is totally removed and the inter-layer memory access complexity is also reduced as compared to the inter-layer inferred prediction mode with 4x4 or 8x8 granular motion data reusing.

Figure 1: The block diagram of the proposed inter-layer motion data inheritance

# Proposed algorithm

In this contribution, an inter_layer_MDI_flag is applied to each inter 2Nx2N PU to signal if the inter-layer MDI is used or not. If the inter_layer_MDI_flag is true, the motion data of the collocated BL block are directly inherited for the motion compensation of the EL block. As shown in Figure 2, a 2Nx2N EL block is divided into four NxN sub-blocks, and the center point of each NxN sub-block is used to find the collocated BL block. Since the inherited motion granularity is NxN, each inter 2Nx2N PU has 4 inherited motion vectors. The motion vectors are scaled in the case of spatial 2x and 1.5x scalabilities. Besides, if motion data of the collocated NxN BL block is not available, the collocated NxN block in the up-sampled BL reconstructed picture will be copied as the prediction of the current EL NxN block.

16

16

16

16

BL picture

EL picture

Current 2Nx2N PU

(2N = 16)

Find collocated BL block

Figure 2: The derivation of collocated BL blocks in the case of 2x scalability

# Simulation results

The proposed method is integrated into the SMuC software 0.1.1 [2] and tested under the common test conditions [3]. Two cases were tested as follows.

Case 1: Derive the motion data at NxN granularity for all sizes of 2Nx2N PUs.

Case 2: If a 2Nx2N PU is in 8x8 size, derive the motion data for it at 2Nx2N granularity; otherwise, derive the motion data at NxN granularity.

Table 1 and Table 2 give the results of case 1, i.e. the proposed MDI algorithm is applied to inter 8x8, inter 16x16, inter 32x32, and inter 64x64 PUs. Compared to SMuC 0.1.1, case 1 achieves 2.1%, 2.2%, and 2.5%  “EL+BL” BD bitrate reduction for RA_2x, RA_1.5x, and RA_SNR cases, and 1.2%, 1.2%, and 2.1% “EL+BL” BD bitrate reduction for LD-P_2x, LD-P _1.5x, and LD-P_SNR cases. Compared to the inter-layer inferred prediction mode with finer granularity such as 4x4 or 8x8 granularity, NxN motion inheritance granularity reduces the inter-layer memory buffer access and improves the data process parallelism (one SIMD can process more data since all pixels in an NxN block have the same motion).

Table 1: Simulation results of RA configuration of Case 1

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -2.5% | -5.4% | -5.2% |  |  |  | -2.9% | -7.4% | -7.7% |
| Class B | -2.0% | -3.5% | -3.6% | -2.2% | -3.9% | -4.1% | -2.3% | -4.3% | -4.5% |
| Overall (EL+BL) | -2.1% | -4.0% | -4.1% | -2.2% | -3.9% | -4.1% | -2.5% | -5.2% | -5.4% |
| Overall (EL) | -4.1% | -7.6% | -7.6% | -5.8% | -9.8% | -10.4% | -5.1% | -10.2% | -10.6% |
| Enc Time[%] | 98.0% |  |  | 96.4% |  |  | 99.1% |  |  |
| Dec Time[%] | 91.3% |  |  | 93.3% |  |  | 95.2% |  |  |

Table 2: Simulation results of LD-P configuration of Case 1

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.6% | -3.2% | -3.1% |  |  |  | -2.7% | -5.0% | -5.1% |
| Class B | -1.1% | -2.0% | -2.0% | -1.2% | -2.1% | -2.2% | -1.8% | -2.8% | -2.9% |
| Overall (EL+BL) | -1.2% | -2.4% | -2.3% | -1.2% | -2.1% | -2.2% | -2.1% | -3.4% | -3.5% |
| Overall (EL) | -2.2% | -4.3% | -4.2% | -2.7% | -4.7% | -4.8% | -3.8% | -6.1% | -6.3% |
| Enc Time[%] | 99.6% |  |  | 98.4% |  |  | 100.5% |  |  |
| Dec Time[%] | 93.1% |  |  | 94.5% |  |  | 96.2% |  |  |

Table 3 and Table 4 show the test results of case 2 which achieves 2.1%, 2.1%, and 2.3% “EL+BL” BD bitrate reduction for RA_2x, RA_1.5x, and RA_SNR cases, and 1.2%, 1.1%, and 1.8% “EL+BL” BD bitrate reduction for LD-P_2x, LD-P _1.5x, and LD-P_SNR cases. The detail data can be found in the spreadsheet of JCTVC-L0291.xls.

Table 3: simulation results of RA configuration of Case 2

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -2.5% | -5.4% | -5.2% |  |  |  | -2.6% | -6.5% | -6.8% |
| Class B | -2.0% | -3.5% | -3.6% | -2.1% | -3.6% | -3.9% | -2.2% | -4.0% | -4.2% |
| Overall (EL+BL) | -2.1% | -4.0% | -4.1% | -2.1% | -3.6% | -3.9% | -2.3% | -4.7% | -4.9% |
| Overall (EL) | -4.1% | -7.6% | -7.6% | -5.5% | -9.4% | -9.9% | -4.8% | -9.3% | -9.7% |
| Enc Time[%] | 98.1% |  |  | 96.5% |  |  | 99.3% |  |  |
| Dec Time[%] | 93.7% |  |  | 93.8% |  |  | 96.2% |  |  |

Table 4: simulation results of LD-P configuration of Case 2

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.6% | -3.2% | -3.1% |  |  |  | -2.2% | -4.1% | -4.2% |
| Class B | -1.1% | -2.0% | -2.0% | -1.1% | -1.9% | -2.0% | -1.6% | -2.5% | -2.6% |
| Overall (EL+BL) | -1.2% | -2.4% | -2.3% | -1.1% | -1.9% | -2.0% | -1.8% | -2.9% | -3.1% |
| Overall (EL) | -2.2% | -4.3% | -4.2% | -2.5% | -4.2% | -4.4% | -3.3% | -5.3% | -5.5% |
| Enc Time[%] | 99.7% |  |  | 98.5% |  |  | 100.5% |  |  |
| Dec Time[%] | 94.0% |  |  | 94.8% |  |  | 96.4% |  |  |

The simulation results show that the gain loss of Case 2 is less than 0.1% compared to the results of Case 1 for 2x and 1.5x cases,  and less than 0.3% for SNR cases. Since Case 2 doesn’t introduce 4x4 motion compensation, it doesn’t worsen the worst case of EL memory access. Based on these considerations, the Case 2 is proposed.

# Conclusions

In this contribution, an inter-layer motion data inheritance (MDI) technique is studied to improve the coding efficiency of SHVC. The proposed MDI is only applied to inter 2Nx2N PU of EL. It partitions a 2Nx2N PU into four NxN sub-PUs, and then the copy of collocated BL MV and the motion compensation are all performed at NxN granularity. Compared to the 4x4/8x8 granularity based inter-layer motion data prediction, the proposed MDI has better memory access efficiency and parallel performance of motion compensation for large PUs. In addition, the proposed MDI limits the 8x8 PU only copy one MV from BL, so that it doesn’t worsen EL’s worst case of memory access during motion compensation. The experimental results show that the proposed method can achieve 2.1%/2.1%/2.3% BD-rate savings for RA_2x/RA_1.5x/RA_SNR cases, and 1.2%/1.1%1.8% BD-rate savings for LP_2x/LP_1.5x/LP_SNR cases, compared to SMuC0.1.1.

# References

- J. Chen, V. Seregin, M. Zhou, B. Jeon, S. Lei, E. Nassor, K. Ugur, A. Segall, A. Tabatabai, H. Yu, J.W. Kang, Y. Ye, W. Zhang, P. Bordes, “A proposal for Scalable HEVC Test Model”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 11th Meeting, JCTVC-K0348, Shanghai, CN, 10-19 Oct., 2012.

- SMuC Software version 0.1.1: https://hevc.hhi.fraunhofer.de/svn/svn_SMuCSoftware/tags/0.1.1/

- Description of Tool Experiment C5: Inter-layer syntax prediction using HEVC base layer. version 15, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-K1105, Shanghai, China, October 2012.

# Patent rights declaration(s)

Intel Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).