| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0292 |
| --- | --- |

| Title: | AHG9: On LCU bit size limit |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenhao Zhang Lidong Xu Yi-Jen Chiu Lingzhi Liu | Email: | wenhao.zhang@intel.com lidong.xu@intel.com yi-jen.chiu@intel.com lingzhi.liu@intel.com |
| Source: | Intel Corporation |  |  |

_____________________________

# Abstract

This report proposes the maximum expansion ratio of LCU bit size to be increased to 3/2 to avoid the violation found with the test scenario of disabling transform skip. In HEVC SoDIS, the maximum expansion ratio of LCU bit size is limited to be equal to 4/3. This proposal investigates the impact of this constraint by studying the synthesized SandStorms testing sequence with Gaussian white noise. Simulation results show that a number of violations of the expansion ratio of LCU bit size of 4/3 are observed for the test scenario of QP equal to 4 when transform skip is turned off, and the expansion ratio of LCU bit size of 3/2 works well for both scenarios of transform skip on/off.

# Introduction

The maximum expansion ratio of LCU bit size is specified to be equal to 4/3 in HEVC SoDIS [1]. This constraint plays a role in balancing the implementation burden between the encoder and decoder. A large limitation value will lower the violation possibility so that decreases the encoder complexity, but on the other hand it will increase the decoder cost because large memory bandwidth is required. This contribution investigates the impact of current LCU bit size limit by using the synthesized SandStorms testing sequence. The statistics of violation counts and LCU data expansion ratio, under the condition of turning on/off the transform skip, are reported. Based on the simulation results, a LCU bit size limit of 3/2 expansion is proposed.

# Simulation

A synthesized Sandstorms testing sequence with Gaussian white noise is generated by using the software provided in [2] and Figure 1 shows it.

Figure 1: Synthesized Sandstorms testing sequence with Gauss white noise (640x240@30Hz 30frames)

The synthesized sequence is coded by HM9.1 under the testing configurations of AI_main, LD_main, LP_main and RA_main specified in [3]. QP value is tested from 4 to 9 and the LCU size is set to 64x64 and 32x32. During the simulation, the QPoffset are all set to 0. Two sets of simulation are performed, in which the transform skip is turned on and off, respectively. The statistics information is collected by a modified source code patch provided in [2].

All the modified configuration files and HM9.1 patch file are accompanied in the attachments.

# Results

## Average data expansion ratio

The statistics of LCU data expansion ratio are show in Table 1. A ratio value greater than 1 indicates that the bit size of the codec LCU exceeds its uncompressed raw data size. The value listed in Table 1 is the average expansion ratio values among all the coded LCUs.

Table 1: Average data expansion ration for 64x64 LCU

| LCU size = 64x64 | Transform skip on |  |  |  |
| --- | --- | --- | --- | --- |
| Total LCU number = 1500 | AI_main | LD_main | LP_main | RA_main |
| QP = 4 | 1.25 | 1.23 | 1.23 | 1.24 |
| QP = 5 | 1.22 | 1.21 | 1.21 | 1.21 |
| QP = 6 | 1.20 | 1.19 | 1.18 | 1.19 |
| QP = 7 | 1.15 | 1.13 | 1.13 | 1.14 |
| QP = 8 | 1.14 | 1.11 | 1.11 | 1.12 |
| QP = 9 | 1.08 | 1.07 | 1.06 | 1.07 |
| LCU size = 64x64 | Transform skip off |  |  |  |
| Total LCU number = 1500 | AI_main | LD_main | LP_main | RA_main |
| QP = 4 | 1.41 | 1.40 | 1.40 | 1.40 |
| QP = 5 | 1.36 | 1.35 | 1.35 | 1.35 |
| QP = 6 | 1.32 | 1.31 | 1.31 | 1.31 |
| QP = 7 | 1.27 | 1.26 | 1.26 | 1.26 |
| QP = 8 | 1.23 | 1.22 | 1.22 | 1.22 |
| QP = 9 | 1.19 | 1.18 | 1.18 | 1.18 |

It’s observed that the expansion ratio values are all less than 1.3 in the simulation with transform skip on. However, for the simulation with transform skip off, larger expansion ratio values are observed and the largest value is 1.41 for QP equal to 4. These simulation results in Table 1 indicate that current LCU bit size limitation with 4/3 expansion provides enough margin when transform skip is on, but is too tight when transform skip is off.

## Violation counts

Table 2 shows the accumulated counts of the 64x64 LCUs which violate the bit size limit of 4/3 expansion.

Table 2: Violation counts with LCU bit size limit of 4/3 expansion

| LCU size = 64x64 | Transform skip on |  |  |  |
| --- | --- | --- | --- | --- |
| Total LCU number = 1500 | AI_main | LD_main | LP_main | RA_main |
| QP = 4 | 0 | 0 | 0 | 0 |
| QP = 5 | 0 | 0 | 0 | 0 |
| QP = 6 | 0 | 0 | 0 | 0 |
| QP = 7 | 0 | 0 | 0 | 0 |
| QP = 8 | 0 | 0 | 0 | 0 |
| QP = 9 | 0 | 0 | 0 | 0 |
| LCU size = 64x64 | Transform skip off |  |  |  |
| Total LCU number = 1500 | AI_main | LD_main | LP_main | RA_main |
| QP = 4 | 1500 | 1500 | 1500 | 1500 |
| QP = 5 | 1500 | 1500 | 1500 | 1499 |
| QP = 6 | 13 | 2 | 4 | 5 |
| QP = 7 | 0 | 0 | 0 | 0 |
| QP = 8 | 0 | 0 | 0 | 0 |
| QP = 9 | 0 | 0 | 0 | 0 |

The results in Table 2 confirm the conclusion of section 3.1. There’s no violation when transform skip is on and a number of violations are observed if transform skip is off. Especially for QP equal to 4 and 5, all the coded LCUs exceed the bit size constraint with 4/3 expansion.

# Discussion

According to the simulation results in section 3, current LCU bit size limitation in HEVC SoDIS cannot achieves a balanced trade-off between the encoder and decoder implications, when transform skip is turned off. In HEVC SoDIS , the syntax “transform_skip_enabled_flag” is defined in the PPS header, which allows the transform skip to be turned off. But the conformance of the LCU bit size limitation will be violated if we don’t use transform skip while using current 4/3 expansion. In another word, there is a conflict of LCU size limitation and transform_skip_enabled_flag in current SoDIS, because in order to conform to the 4/3 expansion limitation, transform_skip_enabled_flag can only be set to 1 which is not the intention of the design. On the other hand, transform skip is a coding tool dedicating for compression of screen content video, so the encoder implementation for traditional natural video content may not apply it to lower cost. Forcing to implement transform skip can be challenging for some consumer encoders. Based on the above considerations, the testing case without transform skip should be regarded as the worst case to decide the LCU bit size limitation.

The simulation in section 3.2 is re-tested by using LCU bit size limit with 3/2 expansion, which is larger than the existing one. The results are show in Table 3.

Table 3: Violation counts with LCU bit size limit of 3/2 expansion

| LCU size = 64x64 | Transform skip on |  |  |  |
| --- | --- | --- | --- | --- |
| Total LCU number = 1500 | AI_main | LD_main | LP_main | RA_main |
| QP = 4 | 0 | 0 | 0 | 0 |
| QP = 5 | 0 | 0 | 0 | 0 |
| QP = 6 | 0 | 0 | 0 | 0 |
| QP = 7 | 0 | 0 | 0 | 0 |
| QP = 8 | 0 | 0 | 0 | 0 |
| QP = 9 | 0 | 0 | 0 | 0 |
| LCU size = 64x64 | Transform skip off |  |  |  |
| Total LCU number = 1500 | AI_main | LD_main | LP_main | RA_main |
| QP = 4 | 0 | 0 | 0 | 0 |
| QP = 5 | 0 | 0 | 0 | 0 |
| QP = 6 | 0 | 0 | 0 | 0 |
| QP = 7 | 0 | 0 | 0 | 0 |
| QP = 8 | 0 | 0 | 0 | 0 |
| QP = 9 | 0 | 0 | 0 | 0 |

No violation is observed in Table 3. Besides, based on the results in section 3.1 (the largest data expansion ratio is 1.41 for QP equal to 4), the 3/2 expansion provides enough margin for encoder to confirm regardless of transform skip on or off.

# Conclusion

This contribution proposes to increase current LCU bit size limit to 3/2 expansion to avoid the violation found with the test scenario of disabling transform skip. Simulations are performed on the synthesized SandStorms testing sequence with Gaussian white noise. Results show that a number of violations of the expansion ratio of LCU bit size of 4/3 are observed for the test scenario of QP equal to 4 when transform skip is turned off, and the expansion ratio of LCU bit size of 3/2 works well for both scenarios of transform skip on/off.

# Reference

- Benjamin Bross, Woo-Jin Han, Jens-Rainer Ohm, Gary J. Sullivan, Thomas Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 9”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 11th Meeting, JCTVC-K1003, Shanghai, CN, 10–19 Oct. 2012.

- Keiichi Chono, “AHG4: Suggested constraint on number of bits/bins per LCU”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 10th Meeting, JCTVC-J0059, Stockholm, SE, 11–20 July 2012. 2012.

- Frank Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 11th Meeting, JCTVC-K1100, Shanghai, CN, 10–19 Oct. 2012.

# Patent rights declaration(s)

Intel Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix: Text changes

    - Main profile

Bitstreams conforming to the Main profile shall obey the following constraints:

- Sequence parameter sets shall have chroma_format_idc equal to 1 only.

- Sequence parameter sets shall have bit_depth_luma_minus8 equal to 0 only.

- Sequence parameter sets shall have bit_depth_chroma_minus8 equal to 0 only.

- Log2CtbSizeY shall be in the range from 4 to 6, inclusive.

- When a picture parameter set has tiles_enabled_flag is equal to 1, it shall have entropy_coding_sync_enabled_flag equal to 0.

- When a picture parameter set has tiles_enabled_flag is equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 256 for all values of i in the range of 0 to num_tile_columns_minus1, inclusive, and RowHeightInLumaSamples[ j ] shall be greater than or equal to 64 for all values of j in the range of 0 to num_tile_rows_minus1, inclusive.

- The number of times read_bits( 1 ) is called in subclauses Error: Reference source not found and Error: Reference source not found when parsing coding_tree_unit( ) data for any coding tree unit shall be less than or equal to 4 * RawCtuBits / 3.

- The number of times read_bits( 1 ) is called in subclauses Error: Reference source not found and Error: Reference source not found when parsing coding_tree_unit( ) data for any coding tree unit shall be less than or equal to 3 * RawCtuBits / 2.

- The level constraints specified for the Main profile in subclause Error: Reference source not found shall be fulfilled.

Conformance of a bitstream to the Main profile is indicated by general_profile_idc being equal to 1 or general_profile_compatibility_flag[ 1 ] being equal to 1.

NOTE – When general_profile_compatibility_flag[ 1 ] is equal to 1, general_profile_compatibility_flag[ 2 ] should also be equal to 1.

Decoders conforming to the Main profile at a specific level (identified by a specific value of general_level_idc) shall be capable of decoding all bitstreams for which the all of following conditions apply:

- general_profile_compatibility_flag[ 1 ] is equal to 1.

- general_level_idc represents a level lower than or equal to the specified level.

- general_tier_flag represents a tier lower than or equal to the specified tier.

    - Main 10 profile

Bitstreams conforming to the Main 10 profile shall obey the following constraints:

- Sequence parameter sets shall have chroma_format_idc equal to 1 only.

- Sequence parameter sets shall have bit_depth_luma_minus8 in the range of 0 to 2, inclusive.

- Sequence parameter sets shall have bit_depth_chroma_minus8 in the range of 0 to 2, inclusive.

- Log2CtbSizeY shall be in the range from 4 to 6, inclusive.

- When a picture parameter set has tiles_enabled_flag is equal to 1, it shall have entropy_coding_sync_enabled_flag equal to 0.

- When a picture parameter set has tiles_enabled_flag is equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 256 for all values of i in the range of 0 to num_tile_columns_minus1, inclusive, and RowHeightInLumaSamples[ j ] shall be greater than or equal to 64 for all values of j in the range of 0 to num_tile_rows_minus1, inclusive.

- The number of times read_bits( 1 ) is called in subclauses Error: Reference source not found and Error: Reference source not found when parsing coding_tree_unit( ) data for any coding tree unit shall be less than or equal to 4 * RawCtuBits / 3.

- The number of times read_bits( 1 ) is called in subclauses Error: Reference source not found and Error: Reference source not found when parsing coding_tree_unit( ) data for any coding tree unit shall be less than or equal to 3 * RawCtuBits / 2.

- The level constraints specified for the Main 10 profile in subclause Error: Reference source not found shall be fulfilled.

Conformance of a bitstream to the Main 10 profile is indicated by general_profile_idc being equal to 2 or general_profile_compatibility_flag[ 2 ] being equal to 1.

Decoders conforming to the Main 10 profile at a specific level (identified by a specific value of general_level_idc) shall be capable of decoding all bitstreams for which the all of following conditions apply:

- general_profile_compatibility_flag[ 1 ] is equal to 1 or general_profile_compatibility_flag[ 2 ] is equal to 1.

- general_level_idc represents a level lower than or equal to the specified level.

- general_tier_flag represents a tier lower than or equal to the specified tier.

    - Main Still Picture profile

Bitstreams conforming to the Main Still Picture profile shall obey the following constraints:

- The bitstream shall contain only one picture.

- Sequence parameter sets shall have chroma_format_idc equal to 1 only.

- Sequence parameter sets shall have bit_depth_luma_minus8 equal to 0 only.

- Sequence parameter sets shall have bit_depth_chroma_minus8 equal to 0 only.

- Sequence parameter sets shall have sps_max_dec_pic_buffering[ sps_max_sub_layers_minus1 ] equal to 0 only.

- Log2CtbSizeY shall be in the range from 4 to 6, inclusive.

- When a picture parameter set has tiles_enabled_flag is equal to 1, it shall have entropy_coding_sync_enabled_flag equal to 0.

- When a picture parameter set has tiles_enabled_flag is equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 256 for all values of i in the range of 0 to num_tile_columns_minus1, inclusive, and RowHeightInLumaSamples[ j ] shall be greater than or equal to 64 for all values of j in the range of 0 to num_tile_rows_minus1, inclusive.

- The number of times read_bits( 1 ) is called in subclauses Error: Reference source not found and Error: Reference source not found when parsing coding_tree_unit( ) data for any coding tree unit shall be less than or equal to 4 * RawCtuBits / 3.

- The number of times read_bits( 1 ) is called in subclauses Error: Reference source not found and Error: Reference source not found when parsing coding_tree_unit( ) data for any coding tree unit shall be less than or equal to 3 * RawCtuBits / 2.

- The level constraints specified for the Main Still Picture profile in subclause Error: Reference source not found shall be fulfilled.

Conformance of a bitstream to the Main Still Picture profile is indicated by general_profile_idc being equal to 3 or general_profile_compatibility_flag[ 3 ] being equal to 1.

Decoders conforming to the Main Still Picture profile at a specific level (identified by a specific value of general_level_idc) shall be capable of decoding all bitstreams for which the all of following conditions apply:

- general_profile_compatibility_flag[ 3 ] is equal to 1.

- general_level_idc represents a level lower than or equal to the specified level.

- general_tier_flag represents a tier lower than or equal to the specified tier.