| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0292 |
| --- | --- |

| Title: | AHG9: On LCU bit size limit |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenhao Zhang Lidong Xu Yi-Jen Chiu Lingzhi Liu | Email: | wenhao.zhang@intel.com lidong.xu@intel.com yi-jen.chiu@intel.com lingzhi.liu@intel.com |
| Source: | Intel Corporation |  |  |

_____________________________

# Abstract

This report proposes the maximum expansion ratio of LCU bit size to be increased to 3/2 for Main Profile and Main Still Picture Profile and 5/3 for Main 10 Profile, to avoid the violation found in the simulation. In HEVC SoDIS, the maximum expansion ratio of LCU bit size is limited to be equal to 4/3. This proposal investigates the impact of this constraint by studying the synthesized SandStorms testing sequence with Gaussian white noise. Simulation results show that a number of violations exceeding the LCU bit size limit with expansion ratio of 4/3 are observed. The violation problem is even worse when transform is turned off. This contribution regards the case without transform skip as the worst cast to increase the LCU bit size limit for encoder to confirm. Further simulation results show that the proposed LCU bit size limit works well for both scenarios of transform skip on/off.

# Introduction

The maximum expansion ratio of LCU bit size is specified to be equal to 4/3 in HEVC SoDIS [1]. This constraint plays a role in balancing the implementation burden between the encoder and decoder. A large limitation value will lower the violation possibility so that decreases the encoder complexity, but on the other hand it will increase the decoder cost because large memory bandwidth is required. This contribution investigates the impact of current LCU bit size limit by using the synthesized SandStorms testing sequence. The statistics of violation counts and LCU data expansion ratio, under the condition of turning on/off the transform skip, are reported. Based on the simulation results, the expansion ratio of a LCU bit size limit is proposed to be increased to 3/2 for Main Profile and Main Still Picture and 5/3 for Main 10 Profile.

# Simulation

A synthesized Sandstorms testing sequence with Gaussian white noise is generated by using the software provided in [2] and Figure 1 shows it.

Figure 1: Synthesized Sandstorms testing sequence with Gauss white noise (640x240@30Hz 30frames)

The synthesized sequence is coded by HM9.1 under the testing configurations of AI_Main/AI_He10, LD_Main/LD_He10, LP_Main/LP_He10 and RA_Main/RA_He10 specified in [3]. The LCU size is set to 64x64 and 32x32. QP value is tested from 4 to 9 for Main configurations, and from -8 to -3 for He10 configurations. QPoffset are all set to 0. Two sets of simulation are performed, in which the transform skip is turned on and off, respectively. The statistics information is collected by a modified source code patch provided in [2].

All the modified configuration files and HM9.1 patch file are accompanied in the attachments.

# Simulation and Results

## Average data expansion ratio

In this simulation, 1500 64x64 LCU are coded to collect the statistics of average LCU data expansion ratio, are show in Table 1. An expansion ratio value greater than 1 indicates that the bit size of the coded LCU exceeds its raw data size before compression.

Table 1: Average data expansion ration for 64x64 LCU (1500 LCUs)

| QP | Transform skip on |  |  |  | Transform skip off |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI_main | LD_main | LP_main | RA_main | AI_main | LD_main | LP_main | RA_main |
| 4 | 1.25 | 1.23 | 1.23 | 1.24 | 1.41 | 1.40 | 1.40 | 1.40 |
| 5 | 1.22 | 1.21 | 1.21 | 1.21 | 1.36 | 1.35 | 1.35 | 1.35 |
| 6 | 1.20 | 1.19 | 1.18 | 1.19 | 1.32 | 1.31 | 1.31 | 1.31 |
| 7 | 1.15 | 1.13 | 1.13 | 1.14 | 1.27 | 1.26 | 1.26 | 1.26 |
| 8 | 1.14 | 1.11 | 1.11 | 1.12 | 1.23 | 1.22 | 1.22 | 1.22 |
| 9 | 1.08 | 1.07 | 1.06 | 1.07 | 1.19 | 1.18 | 1.18 | 1.18 |
|  | AI_he10 | LD_he10 | LP_he10 | RA_he10 | AI_he10 | LD_he10 | LP_he10 | RA_he10 |
| -8 | 1.57 | 1.35 | 1.34 | 1.36 | 1.57 | 1.57 | 1.57 | 1.57 |
| -7 | 1.43 | 1.32 | 1.31 | 1.35 | 1.54 | 1.53 | 1.54 | 1.53 |
| -6 | 1.49 | 1.30 | 1.29 | 1.31 | 1.50 | 1.50 | 1.50 | 1.49 |
| -5 | 1.34 | 1.26 | 1.26 | 1.29 | 1.46 | 1.45 | 1.46 | 1.45 |
| -4 | 1.38 | 1.23 | 1.23 | 1.25 | 1.42 | 1.41 | 1.41 | 1.41 |
| -3 | 1.22 | 1.16 | 1.16 | 1.18 | 1.38 | 1.37 | 1.37 | 1.37 |

In Table 1, all the expansion ratio values greater than the current maximum expansion limit of 4/3 are marked by red color. It’s observed that current LCU bit size limitation doesn’t provide enough margin, because larger expansion ratio values are observed. In the simulations under the Main testing configurations with transform skip off, expansion ratio values for QP equal to 4 and 5 are all exceeds the constrained expiation ratio of 4/3, and the maximum expansion ratio is as large as 1.41. For the simulations under the He10 testing configurations, the situation is even worse. A maximum expansion ratio of 1.57 is observed and all the tested expiation ration exceeds constrained expiation ratio of 4/3 when transform skip is off.

The simulation results also show that the data expansion ratio with transform skip off is always larger than the one with transform skip on, and the data expansion ration of He10 tests is always larger than the one of Main tests under the similar conditions.

## Violation counts

In this simulation, the number of LCUs which violate the bit size limit of 4/3 expansion is collected. Table 2 shows the statistics of violation, among the tested 1500 64x64 LCUs.

Table 2: Violation counts with LCU bit size limit of 4/3 expansion

| QP | Transform skip on |  |  |  | Transform skip off |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI_main | LD_main | LP_main | RA_main | AI_main | LD_main | LP_main | RA_main |
| 4 | 0 | 0 | 0 | 0 | 1500 | 1500 | 1500 | 1500 |
| 5 | 0 | 0 | 0 | 0 | 1500 | 1500 | 1500 | 1499 |
| 6 | 0 | 0 | 0 | 0 | 13 | 2 | 4 | 5 |
| 7 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 9 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | AI_he10 | LD_he10 | LP_he10 | RA_he10 | AI_he10 | LD_he10 | LP_he10 | RA_he10 |
| -8 | 1500 | 1012 | 492 | 1288 | 1500 | 1500 | 1500 | 1500 |
| -7 | 1500 | 264 | 153 | 981 | 1500 | 1500 | 1500 | 1500 |
| -6 | 1500 | 81 | 55 | 214 | 1500 | 1500 | 1500 | 1500 |
| -5 | 1083 | 50 | 50 | 162 | 1500 | 1500 | 1500 | 1500 |
| -4 | 1500 | 1 | 1 | 50 | 1500 | 1500 | 1500 | 1500 |
| -3 | 0 | 0 | 0 | 0 | 1500 | 1500 | 1500 | 1500 |

The LCU violation numbers in Table 2 confirm the conclusion of section 3.1. A number of violations are observed, especially when transform skip is off. This result indicates current LCU bit size limit with 4/3 expansion

# Discussions

According to the simulation results in section 3, current LCU bit size limit with 4/3 expansion in HEVC SoDIS is too tight to provide enough margin for encoder to confirm.

- Tests without transform skip should be regarded as the worst case.

The simulation results show that the data expansion ratio with transform skip off is always larger than the one with transform skip on. In HEVC SoDIS, the syntax “transform_skip_enabled_flag” is defined in the PPS header, which allows the transform skip to be turned off. But the conformance of the LCU bit size limitation will be violated if we don’t use transform skip while using current 4/3 expansion. In another word, there is a conflict between the LCU bit size limitation and transfom skip in current SoDIS, because in order to conform to the 4/3 expansion limitation, transform_skip_enabled_flag can only be set to 1 which is not the intention of the design. On the other hand, transform skip is a coding tool dedicating for compression of screen content video, so the encoder implementation for traditional natural video content may not apply it to lower cost. Forcing to implement transform skip can be challenging for some consumer encoders. Thus, the testing case without transform skip should be regarded as the worst case to decide the LCU bit size limitation.

- 8-bit and 10-bit should be separately considered.

The simulation results show that the data expansion ratio of Main configuration is always larger than the one of He10 configuration under the similar condition. In HEVC SoDIS, Main Profile and Main Still Picture Profile are for 8-bit scenario and Main 10 Profile is for 10-bit scenario. Thus different LCU bit size limit should be used by different profile according to its support of bit depth.

Based on the above considerations, the following LCU bit size limits is proposed.

- For Main Profile and Main Still Picture Profile, it’s proposed to increase the maximum data expansion ratio from current 4/3 to 3/2;

- For Main 10 Profile, it’s proposed to increase the maximum data expansion ratio from current 4/3 to 5/3.

To evaluate the impact of the proposed LCU bit size limits, the simulation of LCU violation in section 3.2 is re-tested by using 3/2 for Main test and 5/3 for He10 test. The results are show in Table 3.

Table 3: Violation counts with proposed LCU bit size limit

| QP | Transform skip on |  |  |  | Transform skip off |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI_main | LD_main | LP_main | RA_main | AI_main | LD_main | LP_main | RA_main |
| 4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 6 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 7 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 9 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | AI_he10 | LD_he10 | LP_he10 | RA_he10 | AI_he10 | LD_he10 | LP_he10 | RA_he10 |
| -8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| -7 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| -6 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| -5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| -4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| -3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

No violation is observed in Table 3. It indicates the proposed LCU bit size limit provides enough margin for encoder to confirm regardless of transform skip on or off.

# Conclusion

This contribution proposes to increase current LCU bit size limit to 3/2 expansion for Main Profile and Main Still Picture Profile and 5/3 expansion for Main 10 Profile, to avoid the violations. Simulations are performed on the synthesized SandStorms testing sequence with Gaussian white noise. Results show that a number of violations of the expansion ratio of LCU bit size of 4/3 are observed. Further simulations prove that the proposed LCU bit size limit provides enough margin for both scenarios of transform skip on/off.

# Reference

- Benjamin Bross, Woo-Jin Han, Jens-Rainer Ohm, Gary J. Sullivan, Thomas Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 9”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 11th Meeting, JCTVC-K1003, Shanghai, CN, 10–19 Oct. 2012.

- Keiichi Chono, “AHG4: Suggested constraint on number of bits/bins per LCU”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 10th Meeting, JCTVC-J0059, Stockholm, SE, 11–20 July 2012. 2012.

- Frank Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 11th Meeting, JCTVC-K1100, Shanghai, CN, 10–19 Oct. 2012.

# Patent rights declaration(s)

Intel Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix: Text changes

    - Main profile

Bitstreams conforming to the Main profile shall obey the following constraints:

- Sequence parameter sets shall have chroma_format_idc equal to 1 only.

- Sequence parameter sets shall have bit_depth_luma_minus8 equal to 0 only.

- Sequence parameter sets shall have bit_depth_chroma_minus8 equal to 0 only.

- Log2CtbSizeY shall be in the range from 4 to 6, inclusive.

- When a picture parameter set has tiles_enabled_flag is equal to 1, it shall have entropy_coding_sync_enabled_flag equal to 0.

- When a picture parameter set has tiles_enabled_flag is equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 256 for all values of i in the range of 0 to num_tile_columns_minus1, inclusive, and RowHeightInLumaSamples[ j ] shall be greater than or equal to 64 for all values of j in the range of 0 to num_tile_rows_minus1, inclusive.

- The number of times read_bits( 1 ) is called in subclauses Error: Reference source not found and Error: Reference source not found when parsing coding_tree_unit( ) data for any coding tree unit shall be less than or equal to 4 * RawCtuBits / 3.

- The number of times read_bits( 1 ) is called in subclauses Error: Reference source not found and Error: Reference source not found when parsing coding_tree_unit( ) data for any coding tree unit shall be less than or equal to 3 * RawCtuBits / 2.

- The level constraints specified for the Main profile in subclause Error: Reference source not found shall be fulfilled.

Conformance of a bitstream to the Main profile is indicated by general_profile_idc being equal to 1 or general_profile_compatibility_flag[ 1 ] being equal to 1.

NOTE – When general_profile_compatibility_flag[ 1 ] is equal to 1, general_profile_compatibility_flag[ 2 ] should also be equal to 1.

Decoders conforming to the Main profile at a specific level (identified by a specific value of general_level_idc) shall be capable of decoding all bitstreams for which the all of following conditions apply:

- general_profile_compatibility_flag[ 1 ] is equal to 1.

- general_level_idc represents a level lower than or equal to the specified level.

- general_tier_flag represents a tier lower than or equal to the specified tier.

    - Main 10 profile

Bitstreams conforming to the Main 10 profile shall obey the following constraints:

- Sequence parameter sets shall have chroma_format_idc equal to 1 only.

- Sequence parameter sets shall have bit_depth_luma_minus8 in the range of 0 to 2, inclusive.

- Sequence parameter sets shall have bit_depth_chroma_minus8 in the range of 0 to 2, inclusive.

- Log2CtbSizeY shall be in the range from 4 to 6, inclusive.

- When a picture parameter set has tiles_enabled_flag is equal to 1, it shall have entropy_coding_sync_enabled_flag equal to 0.

- When a picture parameter set has tiles_enabled_flag is equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 256 for all values of i in the range of 0 to num_tile_columns_minus1, inclusive, and RowHeightInLumaSamples[ j ] shall be greater than or equal to 64 for all values of j in the range of 0 to num_tile_rows_minus1, inclusive.

- The number of times read_bits( 1 ) is called in subclauses Error: Reference source not found and Error: Reference source not found when parsing coding_tree_unit( ) data for any coding tree unit shall be less than or equal to 4 * RawCtuBits / 3.

- The number of times read_bits( 1 ) is called in subclauses Error: Reference source not found and Error: Reference source not found when parsing coding_tree_unit( ) data for any coding tree unit shall be less than or equal to 5 * RawCtuBits / 3.

- The level constraints specified for the Main 10 profile in subclause Error: Reference source not found shall be fulfilled.

Conformance of a bitstream to the Main 10 profile is indicated by general_profile_idc being equal to 2 or general_profile_compatibility_flag[ 2 ] being equal to 1.

Decoders conforming to the Main 10 profile at a specific level (identified by a specific value of general_level_idc) shall be capable of decoding all bitstreams for which the all of following conditions apply:

- general_profile_compatibility_flag[ 1 ] is equal to 1 or general_profile_compatibility_flag[ 2 ] is equal to 1.

- general_level_idc represents a level lower than or equal to the specified level.

- general_tier_flag represents a tier lower than or equal to the specified tier.

    - Main Still Picture profile

Bitstreams conforming to the Main Still Picture profile shall obey the following constraints:

- The bitstream shall contain only one picture.

- Sequence parameter sets shall have chroma_format_idc equal to 1 only.

- Sequence parameter sets shall have bit_depth_luma_minus8 equal to 0 only.

- Sequence parameter sets shall have bit_depth_chroma_minus8 equal to 0 only.

- Sequence parameter sets shall have sps_max_dec_pic_buffering[ sps_max_sub_layers_minus1 ] equal to 0 only.

- Log2CtbSizeY shall be in the range from 4 to 6, inclusive.

- When a picture parameter set has tiles_enabled_flag is equal to 1, it shall have entropy_coding_sync_enabled_flag equal to 0.

- When a picture parameter set has tiles_enabled_flag is equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 256 for all values of i in the range of 0 to num_tile_columns_minus1, inclusive, and RowHeightInLumaSamples[ j ] shall be greater than or equal to 64 for all values of j in the range of 0 to num_tile_rows_minus1, inclusive.

- The number of times read_bits( 1 ) is called in subclauses Error: Reference source not found and Error: Reference source not found when parsing coding_tree_unit( ) data for any coding tree unit shall be less than or equal to 4 * RawCtuBits / 3.

- The number of times read_bits( 1 ) is called in subclauses Error: Reference source not found and Error: Reference source not found when parsing coding_tree_unit( ) data for any coding tree unit shall be less than or equal to 3 * RawCtuBits / 2.

- The level constraints specified for the Main Still Picture profile in subclause Error: Reference source not found shall be fulfilled.

Conformance of a bitstream to the Main Still Picture profile is indicated by general_profile_idc being equal to 3 or general_profile_compatibility_flag[ 3 ] being equal to 1.

Decoders conforming to the Main Still Picture profile at a specific level (identified by a specific value of general_level_idc) shall be capable of decoding all bitstreams for which the all of following conditions apply:

- general_profile_compatibility_flag[ 3 ] is equal to 1.

- general_level_idc represents a level lower than or equal to the specified level.

- general_tier_flag represents a tier lower than or equal to the specified tier.