| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 |  |  | Document: JCTVC-L0293 |
| --- | --- | --- | --- |
| Title: | On frame packing arrangement SEI message |  |  |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Chanyul Kim Jeonghoon Park | Email: | b.d.choi@samsung.com dionism@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

A minor change of frame packing arrangement (FPA) SEI message is proposed. By adding a flag, which indicates whether the current video sequence is the primary half resolution 3D data or the complementary (secondary) data, the full resolution 3D video as well as the half resolution 3D video can be delivered in a scalable manner, with the proposed FPA SEI message.

# Introduction

The frame packing arrangement (FPA) SEI message was adopted in HEVC specification text. It is necessary to provide stereoscopic 3D video service supporting the compatibility with 2D video system, before the professional 3D CODEC is feasible. The H.264/AVC based frame packing 3D systems have been successfully used in Cable TV systems. HEVC-based frame packing 3D can be also used intermediately, before the HEVC-based 3D CODEC (MV-HEVC or 3D-HEVC) is fully feasible. The limitation of the frame packing 3D like side-by-side is the visual quality of the decoded frames. By sub-sampling the left and right images into a single frame, the half resolution 3D is reconstructed at decoder, and up-scaled for display, which may provide unsatisfactory visual quality in high-end 3D display.

In this proposal, we propose a minor change of the adopted FPA SEI message to enable the 3D video delivery in full resolution 3D video as well as half resolution. By delivering two simulcast bitstreams, which are frame-packed with the primary half resolution 3D video or the complementary (secondary) half resolution 3D video, respectively. If both the primary bitstreams and the secondary bitstreams are received, the full resolution 3D video can be reconstructed. Otherwise, if only the primary bitstreams is received, the half resolution 3D video can be reconstructed. This mechanism enables to deliver 3D video data in a scalable manner, where the base layer bitstream is identical to the conventional FPA, and the enhancement layer bitstream is the complementary data of the base layer.

# Proposal

The figure 1 illustrates conceptually how to arrange two video sequences to deliver full resolution 3D video with FPS SEI message. When the primary half resolution 3D data is frame-packed and coded into a bitstream, the complementary (secondary) half resolution 3D data can be frame-packed and coded into another bitstream. Two bitstreams can be transmitted in simulcast, or conveyed in an elementary stream. While the half resolution 3D can be decoded and displayed when only the primary bitstream is delivered, if both primary and secondary frame-packed bitstreams are delivered, the full resolution 3D video can be reconstructed by rearranging two decoded frames, as shown in Figure 1.

Figure 1. Full resolution 3D delivery mechanism with FPS SEI messages.

To enable the above mechanism, we propose a minor change of the adopted FPA SEI message. The proposed complementary_data_flag specifies whether the current coded video sequence is the complementary video data of another video sequence or the primary half resolution 3D video data of the full resolution 3D video. When both the coded video sequences with complementary_data_flag equal to 0 and the coded video sequences with complementary_data_flag equal to 1 are received, the decoder can decode full resolution 3D video. Otherwise, if only the coded video sequences with complementary_data_flag equal to 0 is received, the half resolution 3D video can be decoded.

# Proposed text

| frame_packing_arrangement( payloadSize ) { | Descriptor |
| --- | --- |
| frame_packing_arrangement_id | ue(v) |
| frame_packing_arrangement_cancel_flag | u(1) |
| complementary_data_flag | u(1) |
| if( !frame_packing_arrangement_cancel_flag ) { |  |
| frame_packing_arrangement_type | u(7) |
| quincunx_sampling_flag | u(1) |
| content_interpretation_type | u(6) |
| spatial_flipping_flag | u(1) |
| frame0_flipped_flag | u(1) |
| field_views_flag | u(1) |
| current_frame_is_frame0_flag | u(1) |
| frame0_self_contained_flag | u(1) |
| frame1_self_contained_flag | u(1) |
| if ( !quincunx_sampling_flag && frame_packing_arrangement_type != 5 ) { |  |
| frame0_grid_position_x | u(4) |
| frame0_grid_position_y | u(4) |
| frame1_grid_position_x | u(4) |
| frame1_grid_position_y | u(4) |
| } |  |
| frame_packing_arrangement_reserved_byte | u(8) |
| frame_packing_arrangement_repetition_period | ue(v) |
| } |  |
| upsampled_aspect_ratio_flag | u(1) |
| } |  |

complementary_data_flag equal to 1 specifies that the current coded video sequence is the complementary video data of another video sequence, which is basically frame-packed with half resolution 3D video. complementary_data_flag equal to 0 specifies that the current coded video sequence is the primary half resolution 3D video data of the full resolution 3D video. When frame_packing_arrangement is equal to 5, complementary_data_flag shall be equal to 0.

NOTE 1 –If both the coded video sequences with complementary_data_flag equal to 0 and the coded video sequences with complementary_data_flag equal to 1 are received, the decoder can decode full resolution 3D video. Otherwise, if only the coded video sequences with complementary_data_flag equal to 0 is received, the half resolution 3D video is decoded.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).