| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0294 |
| --- | --- |

| Title: | Non TE3: Simplification of Intra prediction based on differential picture in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krishna Rapaka Jianle Chen Xiang Li Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | krapaka@qti.qualcomm.com cjianle@qti.qualcomm.com lxiang@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

Difference Domain intra Prediction (DIP) is investigated in SHVC TE3. In this proposal, two simplification methods for DIP are presented. In the first method, Mode dependent Intra Smoothing (MDIS) is disabled for the DIP mode. For this configuration, it is reported that for AI cases, the average luma BD-Rate reduction for EL only and EL+BL are 1.83% and 0.83% respectively. In the second method, only two intra prediction modes, namely, horizontal and vertical are enabled for DIP and the best mode of these two is signaled using a flag. In addition, MDIS and pixel filtering in horizontal and vertical prediction modes are disabled. For this configuration, it is reported that for AI cases, the average luma BD-Rate reduction for EL only and EL+BL are 1.05% and 0.45% respectively.

# Technical description

## Difference Domain Intra Prediction

The Difference Domain intra Prediction (DIP) was proposed in [1][2] to improve the coding efficiency of SHVC. In the DIP, as shown in Figure 1, the difference between the pixels of current neighbors and that of collocated BL neighbors are used to generate a difference prediction based on the intra prediction mode. The generated difference prediction signal is added to the collocated BL block signal to form the final prediction.

The difference domain prediction mode is indicated by a flag intra_resi_pred_flag at CU level.

Figure 1 Difference Domain Intra Prediction

## Method 1: DIP without MDIS

In this method, Mode dependent Intra Smoothing (MDIS) is disabled in the DIP mode. It was observed that due to the high frequency nature of the difference signals, existing HEVC’s MDIS process on the reference pixels doesn’t help in the DIP mode. As a result of disabling MDIS filtering, the latency and complexity are reduced.

## Method 2: Simplified DIP with only two Intra Prediction Modes

In this method, only two intra prediction modes, namely, horizontal and vertical are enabled for DIP. The best modes of these two are signaled using a flag at PU level. In Addition, MDIS and pixel filtering for horizontal and vertical modes are disabled for DIP mode. As a result, the DIP process at the enhancement layer is shown as follows,

Pf(x,y) = B(x,y) + (Pe(xR,yR) - Pb(xR,yR))

where,

Pf(x,y) : Final Prediction Pixels for the enhancement layer block

B(x,y) : Collocated Base layer pixels (INTRA-BL Prediction)

Pe(xR,yR) :  Spatial reference pixels for the enhancement layer block based on intra prediction direction

Pb(xR,yR) :  Corresponding spatial reference pixels for the collocated base layer block based on intra prediction direction

Since MDIS and Intra Filtering for horizontal and vertical mode are disabled, the intra prediction pixel generation for Pe(xR,yR) and Pb(xR,yR) can be by-passed in DIP.

# Test Results

Two simplification methods mentioned in Section 1 are experimentally verified under SHVC common test conditions defined by AHG10. Thanks to LGE for crosschecking the tests.

Table 1:  Experimental results of DIP without MDIS over TE3 Anchor

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.6% | -0.3% | -0.5% |  |  |  |
| Class B | -1.1% | -0.4% | -0.6% | -0.7% | 0.1% | -0.1% |
| Overall (EL+BL) | -1.0% | -0.4% | -0.6% | -0.7% | 0.1% | -0.1% |
| Overall (EL) | -1.7% | -0.6% | -1.0% | -2.0% | 0.4% | -0.1% |
| Enc Time[%] | 160.0% |  |  | 151.2% |  |  |
| Dec Time[%] | 108.9% |  |  | 107.6% |  |  |
| Enc Mem[%] | 103.2% |  |  | 102.9% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Table 2:  Experimental results of Simplified two mode DIP over TE3 Anchor

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.2% | -0.2% |  |  |  |
| Class B | -0.5% | -0.2% | -0.3% | -0.4% | 0.2% | 0.1% |
| Overall (EL+BL) | -0.5% | -0.2% | -0.3% | -0.4% | 0.2% | 0.1% |
| Overall (EL) | -0.9% | -0.3% | -0.4% | -1.3% | 0.6% | 0.4% |
| Enc Time[%] | 120.6% |  |  | 116.6% |  |  |
| Dec Time[%] | 102.1% |  |  | 101.4% |  |  |
| Enc Mem[%] | 100.2% |  |  | 100.2% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# References

- J. Chen, K. Rapaka, X. Li, V. Seregin, L. Guo, M. Karczewicz, G. Van der Auwera, J. Sole, X. Wang, C. J. Tu, Y. Chen, “Description of scalable video coding technology proposal by Qualcomm (configuration 1) JCTVC-K0035”, 11th Meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Shanghai, China, 10-19 Oct., 2012

- J. Chen, K. Rapaka, X. Li, V. Seregin, L. Guo, M. Karczewicz, G. Van der Auwera, J. Sole, X. Wang, C. J. Tu, Y. Chen, “Description of scalable video coding technology proposal by Qualcomm (configuration 2) JCTVC-K0036”, 11th Meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Shanghai, China, 10-19 Oct., 2012.

- K. Rapaka, J. Chen, X. Li, M. Karczewicz, “TE3 : Results of Test 4.2.2 on Intra prediction based on differential picture”, 12th Meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, 14–23 Jan. 2013.

- J. Boyce, D. Hong, W. Jang, A. Abbas, S. Reddy, K. McCann, J.-H. Park, J. Kim, C. Kim, J.-H. Min, E. Alshina, A. Alshin, I.-K. Kim, T. Lee, B. Choi, Y. Piao, S. Jeong, S. Lee, Y. Cho, J. Y. Choi, “Description of low complexity scalable video coding technology proposal by Vidyo and Samsung JCTVC-K0045”, 11th Meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Shanghai, China, 10-19 Oct., 2012.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).