| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0295 |
| --- | --- |

| Title: | TE2: Cross-check results of test 3.1.1 on CU based IntraBL signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Zhuoyi Lyu Jinwen Zan Road Beiqing 156, Huawei Industrial Base, Haidian, Beijing, China | Tel: Email: | +86 10 59728335 lvzhuoyi@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This document reports the crosschecking procedures and results for SHVC [1] TE2 3.1.1 [2] described in JCTVC- L0284 [3]. The configurations specified in JCTVC-K1102 [2] are tested, and the RD data obtained matches the results provided by Qualcomm.

# Introduction

Qualcomm provided the source codes implemented onto SMuC-0.1.1. The verification is carried out by setting the following macros in TypeDef.h.

#define INTRA_BL_SKIP          1

#define SVC_DBF_INTRABL_BS   1

The running of the tests includes encoding, decoding, and collecting the run time of encoding and decoding processes. The anchor data used is from HM-8.1.

It is noted that there is a bug in SMuC-0.1.1, which employed forward DCT and inverse DST for IntraBL 4x4 blocks. It was agreed that this original version without bug fix is used for the TEs.

For the software provided in this cross-check, the bug is fixed with the following macros, meaning DCT has been used for both forward and inverse transforms.

#define INTRA_BL_BUGFIX   1

#if INTRA_BL_BUGFIX

#define INTRA_BL_DCT_FIX  1 ///< fix : 0: use DST forward transform: 1: use DCT inverse transform

#endif

# Results

## Test condition

The common test conditions specified in the JCTVC-K1102 [2] are used for the simulation. VS2010 x64 compiler is used to generate the executable files from the provided source code. Simulations were done on a cluster running Windows Server 2003 x64 OS.

## Test results

Tables 1 show the summary of results. It was confirmed that BD-Rate values and dQP code increment ratios are exactly same as those provided by the proponent. Detailed RD data can be found in the attached excel sheet.

Table 1: Summary results of test of JCTVC-K0035/K0036 against HM8.1 (anchor).

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -27.1% | -27.4% | -28.3% |  |  |  |
| Class B | -21.4% | -21.4% | -21.3% | -32.9% | -33.0% | -33.2% |
| Overall (EL+BL) | -23.1% | -23.1% | -23.3% | -32.9% | -33.0% | -33.2% |
| Overall (EL) | -35.2% | -35.5% | -35.8% | -58.7% | -58.2% | -58.6% |
| Enc Time[%] | 97.5% |  |  | 94.9% |  |  |
| Dec Time[%] | 100.2% |  |  | 96.0% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -18.5% | -7.9% | -9.5% |  |  |  | -21.5% | -12.1% | -11.8% |
| Class B | -15.7% | -9.2% | -7.8% | -26.3% | -20.0% | -18.6% | -19.6% | -10.8% | -8.0% |
| Overall (EL+BL) | -16.5% | -8.8% | -8.3% | -26.3% | -20.0% | -18.6% | -20.1% | -11.2% | -9.1% |
| Overall (EL) | -25.4% | -13.6% | -13.0% | -47.6% | -37.5% | -35.1% | -31.8% | -18.7% | -15.7% |
| Enc Time[%] | 96.7% |  |  | 98.8% |  |  | 94.9% |  |  |
| Dec Time[%] | 130.1% |  |  | 128.4% |  |  | 105.1% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -13.5% | -1.9% | -3.0% |  |  |  | -14.0% | -7.3% | -6.3% |
| Class B | -11.9% | -6.7% | -4.9% | -22.0% | -16.3% | -14.6% | -12.8% | -5.2% | -1.8% |
| Overall (EL+BL) | -12.3% | -5.4% | -4.3% | -22.0% | -16.3% | -14.6% | -13.2% | -5.8% | -3.1% |
| Overall (EL) | -19.7% | -8.3% | -7.0% | -40.9% | -30.8% | -27.8% | -21.2% | -11.0% | -7.1% |
| Enc Time[%] | 96.2% |  |  | 98.7% |  |  | 94.0% |  |  |
| Dec Time[%] | 135.9% |  |  | 135.2% |  |  | 110.7% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# Conclusion

This document presents the cross-verification results of SHVC-TE2 3.1.1 [2] based on a bug fixed version of SMuc-0.1.1. The cross-verification tasks were successfully completed and it was confirmed that the implementation conforms to the description in JCTVC-L0284 [3] and results exactly match those presented in the proponent.

# References

[1] A. Segall, J. Boyce, J.-R. Ohm, G.J. Sullivan, “Report of results of the joint call for proposals on scalable high efficiency video coding (SHVC)”, JCTVC-K1008, 11th JCT-VC meeting, Shanghai, China, October 2012.

[2] L. Guo, Y. He, D. Kwon, J. Zan, H. Lakshman, J. Kang, “Description of Tool Experiment A2: Inter-layer Texture Prediction Signaling in SHVC,” JCT-VC document JCTVC-K1102, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th meeting, Shanghai, China, October. 2012.

[3] L. Guo, V. Seregin, J. Chen, M. Karczewicz, “TE2: Results of test 3.1.1 on inter-layer texture prediction signaling”,  JCT-VC document JCTVC-L0284, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th meeting, Geneva, Switzerland, January 2013.