| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0296 |
| --- | --- |

| Title: | TE2: Cross-check results of test 3.3.1 on skipped slice in the enhancement layer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Dong Jiang Jinwen Zan 156 Beiqing Rd, Huawei Industrial Base Haidian District, Beijing 100095 P. R. China | Tel: Email: | +86 10 60614082 Dong.jiang@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This document reports the SHVC [1] TE2 [2] crosschecking procedures and results for the Enhancement Layer Skipped Slice proposed in JCTVC-L0055. The RD data we obtained match the results provided by the proponent.

# Introduction

InterDigital provided the source codes of their proposed codec described in JCTVC-K0034_r1, based on SMuC-0.1.1. The verification is carried out by setting the following macros in TypeDef.h.

Two configurations are tested about the proposed algorithm. One is based on the intraBL, the setting is as follows:

#define REF_IDX_FRAMEWORK                  0      ///< inter-layer reference framework

#if REF_IDX_FRAMEWORK

#define REF_IDX_ME_AROUND_ZEROMV        0      ///< added ME around zero MV for inter-layer reference picture

#else

#define INTRA_BL                              1      ///< inter-layer texture prediction

#endif

#define TE2_331_SKIP_SLICE                    1      ///< added skip slice for TE2 3.3.1

The other is based on the ref_idx_framework, the setting is as follows:

#define REF_IDX_FRAMEWORK                  1      ///< inter-layer reference framework

#if REF_IDX_FRAMEWORK

#define REF_IDX_ME_AROUND_ZEROMV        0      ///< added ME around zero MV for inter-layer reference picture

#else

#define INTRA_BL                              1      ///< inter-layer texture prediction

#endif

#define TE2_331_SKIP_SLICE                    1      ///< added skip slice for TE2 3.3.1

The running of the tests includes encoding, decoding, and collecting the run time of encoding and decoding processes. The anchor data used is from HM-8.1.

## Test condition

The two mandatory ref_idx configurations specified above are used for the simulation. VS2010 x64 compiler is used to generate the executable file from the provided source code. Simulations were done on computer cluster running Windows Server 2003 x64 OS.

## Test results

Tables 1 shows the summary of results of proposed algorithm based on intraBL. It was confirmed that BD-Rate values and dQP code increment ratios are exactly same as those provided by the proponent. Detailed RD data can be found in the attached excel sheet.

Table 1: Summary results of test of JCTVC-K0034_r1 against HM8.1 (intraBL on).

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -27.0% | -27.2% | -28.0% |  |  |  |  |  |  |
| Class B | -21.1% | -21.2% | -21.0% | -32.6% | -33.0% | -33.0% |  |  |  |
| Overall (EL+BL) | -22.8% | -22.9% | -23.0% | -32.6% | -33.0% | -33.0% |  |  |  |
| Overall (EL) | -34.8% | -35.2% | -35.3% | -58.3% | -58.4% | -58.5% |  |  |  |
| Enc Time[%] | 111.5% |  |  | 109.9% |  |  |  |  |  |
| Dec Time[%] | 106.9% |  |  | 108.2% |  |  |  |  |  |
| Enc Mem[%] | 109.6% |  |  | 108.0% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -18.5% | -7.8% | -9.3% |  |  |  | -21.5% | -12.0% | -11.7% |
| Class B | -15.5% | -9.2% | -7.8% | -26.1% | -20.3% | -18.8% | -19.4% | -11.2% | -8.2% |
| Overall (EL+BL) | -16.4% | -8.8% | -8.3% | -26.1% | -20.3% | -18.8% | -20.0% | -11.4% | -9.2% |
| Overall (EL) | -25.2% | -13.6% | -12.9% | -47.4% | -38.2% | -35.5% | -31.7% | -19.1% | -15.9% |
| Enc Time[%] | 108.2% |  |  | 98.9% |  |  | 122.5% |  |  |
| Dec Time[%] | 143.8% |  |  | 131.6% |  |  | 122.7% |  |  |
| Enc Mem[%] | 111.9% |  |  | 110.4% |  |  | 99.7% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -13.6% | -1.9% | -3.0% |  |  |  | -14.0% | -7.2% | -6.4% |
| Class B | -11.7% | -6.9% | -5.1% | -22.0% | -17.1% | -15.2% | -12.8% | -5.7% | -2.2% |
| Overall (EL+BL) | -12.3% | -5.5% | -4.5% | -22.0% | -17.1% | -15.2% | -13.1% | -6.2% | -3.4% |
| Overall (EL) | -19.6% | -8.6% | -7.2% | -41.3% | -32.7% | -29.1% | -21.1% | -11.6% | -7.6% |
| Enc Time[%] | 121.7% |  |  | 121.5% |  |  | 100.2% |  |  |
| Dec Time[%] | 163.6% |  |  | 161.0% |  |  | 108.0% |  |  |
| Enc Mem[%] | 111.9% |  |  | 110.4% |  |  | 99.6% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Tables 2 shows the summary of results of proposed algorithm based on ref_idx_framework. It was confirmed that BD-Rate values and dQP code increment ratios are exactly same as those provided by the proponent. Detailed RD data can be found in the attached excel sheet.

Table 2: Summary results of test of JCTVC-K0034_r1 against HM8.1 (ref_idx_framework on).

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -26.9% | -26.2% | -27.1% |  |  |  |  |  |  |
| Class B | -21.2% | -20.1% | -20.0% | -32.8% | -32.9% | -33.1% |  |  |  |
| Overall (EL+BL) | -22.8% | -21.9% | -22.0% | -32.8% | -32.9% | -33.1% |  |  |  |
| Overall (EL) | -34.8% | -33.7% | -33.9% | -58.5% | -58.2% | -58.7% |  |  |  |
| Enc Time[%] | 245.3% |  |  | 184.9% |  |  |  |  |  |
| Dec Time[%] | 98.8% |  |  | 91.4% |  |  |  |  |  |
| Enc Mem[%] | 109.6% |  |  | 108.0% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -17.3% | -4.6% | -6.1% |  |  |  | -20.3% | -8.3% | -7.3% |
| Class B | -14.2% | -5.2% | -3.3% | -25.4% | -18.8% | -16.9% | -17.7% | -5.8% | -1.2% |
| Overall (EL+BL) | -15.1% | -5.1% | -4.1% | -25.4% | -18.8% | -16.9% | -18.5% | -6.5% | -2.9% |
| Overall (EL) | -23.4% | -8.0% | -6.9% | -46.5% | -36.0% | -32.8% | -29.2% | -11.7% | -6.5% |
| Enc Time[%] | 104.4% |  |  | 100.8% |  |  | 107.2% |  |  |
| Dec Time[%] | 127.7% |  |  | 126.0% |  |  | 99.1% |  |  |
| Enc Mem[%] | 111.9% |  |  | 110.4% |  |  | 99.7% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -13.3% | -1.1% | -2.3% |  |  |  | -13.7% | -6.4% | -5.4% |
| Class B | -11.6% | -5.6% | -3.9% | -22.1% | -16.9% | -15.2% | -12.7% | -5.7% | -1.8% |
| Overall (EL+BL) | -12.1% | -4.3% | -3.5% | -22.1% | -16.9% | -15.2% | -13.0% | -5.9% | -2.9% |
| Overall (EL) | -19.4% | -6.8% | -5.7% | -41.4% | -32.2% | -29.1% | -20.9% | -11.2% | -6.9% |
| Enc Time[%] | 116.8% |  |  | 105.0% |  |  | 98.8% |  |  |
| Dec Time[%] | 149.6% |  |  | 132.4% |  |  | 98.6% |  |  |
| Enc Mem[%] | 111.9% |  |  | 110.4% |  |  | 99.6% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# Conclusion

The executables generated from the provided source code can be used for the encoding and decoding test. The decoded sequences match the reconstructed sequences. The RD data provided by the proponent are exactly the same as the results we obtained. Detailed RD data can be found in the attached excel sheet.

# References

[1] A. Segall, et al., “Report of results of the joint call for proposals on scalable high efficiency video coding (SHVC)”, JCTVC-K1008, 11th JCT-VC meeting, Shanghai, China, October, 2012.

[2] Liwei Guo, et al., “Description of Tool Experiment A2: Inter-layer Texture Prediction Signaling in SHVC,” JCT-VC document JCTVC-K1102, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th meeting, Shanghai, China, October. 2012.

[3] Y. He, X. Xiu, Y. He, and Y. Ye “TE2: Results of test 3.3.1 enhancement layer skipped slice,” JCTVC-L0055, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th meeting, Geneva, Switzerland, January 2013.