| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0297 |
| --- | --- |

| Title: | TE5: Results of test 5.2.9 on inter-layer motion prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra, Jie Zhao, Andrew Segall 5750 NW Pacific Rim Blvd. Camas WA 98607 USA | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com jzhao@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP Laboratories of America, Inc. |  |  |

_____________________________

# Abstract

This contribution reports results for TE5 test 5.2.9 on inter-layer motion prediction for the scalable extension of High efficiency video coding (HEVC) standard. In the proposed approach a merge list candidate is determined using the motion information of the co-located base layer prediction unit and added after the temporal merge list candidate. A second change, proposes that a primary motion vector predictor be determined based on the base layer motion information. If the primary motion vector predictor is not selected an AMVP process identical to the base layer AMVP process is used. The coding efficiency of the two proposed changes is measured separately as well as together. The average luma EL+BL BD rate with SMuC 0.1.1 as anchors is determined to be:

|  | Y-RA-2x | Y-RA-1.5x | Y-RA-SNR | Y-LDP-2x | Y-LDP-1.5x | Y-LDP-SNR |
| --- | --- | --- | --- | --- | --- | --- |
| Merge only | -1.9% | -2.0% | -2.0% | -1.1% | -1.1% | -1.3% |
| AMVP only | -1.3% | -1.3% | -1.3% | -0.8% | -0.7% | -0.8% |
| Merge+AMVP | -2.1% | -2.2% | -2.2% | -1.2% | -1.2% | -1.4% |

# Introduction

This contribution proposes the following changes to SMuC 0.1.1: (a) Inclusion of one merge list candidate after the temporal candidate (b) Inclusion of a primary motion vector predictor, based on motion information of co-located base layer prediction unit. The proposed system has the following characteristics:

- No modification of the base layer pruning process is made for the merge list construction process.

- No modification of the base layer redundancy checks are made for the merge list construction process.

- Same base layer motion information fetch mechanism is used for generation of base layer merge list candidate and primary motion vector predictor.

- Same motion vector scaling mechanism is used for generation of base layer merge list candidate and primary motion vector predictor.

- The pixel location used in fetching the base layer motion information is the same as that used for the temporal candidate.

# Algorithm

In this contribution the merge list and motion vector predictor are determined using uncompressed motion information from the base layer as described in test 5.2.9 of JCTVC-K1105 [1], JCTVC-K0031 [2].

Merge list changes for enhancement layer

A merge list candidate is added to the merge list based on the motion information corresponding to a co-located prediction unit (PU) in the base layer picture co-located with the current picture. The reference index of the base layer merge list candidates are set to point to enhancement layer picture co-located with the base-layer reference picture. The base layer reference picture corresponds to the reference picture of the base layer picture co-located with the enhancement layer picture under consideration. The same reference picture list is considered for the base layer picture while determining the reference picture as was being considered for the enhancement layer picture. The motion vector corresponding to the co-located PU in the base layer picture co-located with the current enhancement layer picture is scaled.

Motion vector predictor changes for enhancement layer

A primary motion vector predictor is generated for every reference picture using the base layer motion vector. More specifically, a base layer motion vector is derived from the co-located PU from the base layer picture co-located with the current enhancement layer picture. The derived motion vector is scaled and used as the primary motion vector predictor. A context coded flag mvp_lX_bl_flag is used to signal if the primary motion vector predictor is to be used for the current PU. If the mvp_lX_bl_flag is false the advanced motion vector prediction is used to generate the predictor.

# Experimental Setup

The test conditions used correspond to those outlined in JCTVC-K1105. The anchor software used is the SMuC 0.1.1. BD rates are measured for both merge list and AVMP list changes separately as well as together. The detailed BD rate results can be found in the attached excel sheets.

## Merge only changes:

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.9% | -3.3% | -3.3% |  |  |  | -1.9% | -3.5% | -3.8% |
| Class B | -1.9% | -2.8% | -2.9% | -2.0% | -2.7% | -3.0% | -2.0% | -2.9% | -3.1% |
| Overall (EL+BL) | -1.9% | -3.0% | -3.0% | -2.0% | -2.7% | -3.0% | -2.0% | -3.1% | -3.3% |
| Overall (EL) | -3.4% | -5.3% | -5.5% | -5.0% | -6.7% | -7.2% | -3.9% | -5.7% | -6.2% |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.1% | -1.9% | -1.8% |  |  |  | -1.3% | -2.0% | -2.0% |
| Class B | -1.1% | -1.4% | -1.6% | -1.1% | -1.4% | -1.5% | -1.3% | -1.6% | -1.8% |
| Overall (EL+BL) | -1.1% | -1.6% | -1.7% | -1.1% | -1.4% | -1.5% | -1.3% | -1.7% | -1.9% |
| Overall (EL) | -1.8% | -2.6% | -2.7% | -2.4% | -2.9% | -3.2% | -2.2% | -2.8% | -3.0% |

## AMVP only changes:

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.4% | -2.3% | -2.4% |  |  |  | -1.3% | -2.3% | -2.6% |
| Class B | -1.3% | -1.8% | -1.9% | -1.3% | -1.6% | -1.9% | -1.3% | -1.9% | -2.0% |
| Overall (EL+BL) | -1.3% | -1.9% | -2.0% | -1.3% | -1.6% | -1.9% | -1.3% | -2.0% | -2.2% |
| Overall (EL) | -2.3% | -3.4% | -3.6% | -3.4% | -3.8% | -4.4% | -2.6% | -3.6% | -4.0% |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.9% | -1.4% | -1.4% |  |  |  | -0.9% | -1.4% | -1.3% |
| Class B | -0.7% | -1.0% | -1.0% | -0.7% | -0.9% | -1.1% | -0.8% | -1.0% | -1.1% |
| Overall (EL+BL) | -0.8% | -1.1% | -1.1% | -0.7% | -0.9% | -1.1% | -0.8% | -1.1% | -1.2% |
| Overall (EL) | -1.3% | -1.8% | -1.8% | -1.6% | -1.9% | -2.2% | -1.4% | -1.8% | -1.9% |

## Merge + AMVP changes:

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -2.2% | -3.8% | -3.8% |  |  |  | -2.1% | -4.0% | -4.3% |
| Class B | -2.0% | -3.1% | -3.3% | -2.2% | -3.0% | -3.2% | -2.2% | -3.2% | -3.4% |
| Overall (EL+BL) | -2.1% | -3.3% | -3.4% | -2.2% | -3.0% | -3.2% | -2.2% | -3.4% | -3.7% |
| Overall (EL) | -3.7% | -5.9% | -6.1% | -5.5% | -7.2% | -7.8% | -4.2% | -6.3% | -6.8% |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.3% | -2.0% | -2.0% |  |  |  | -1.4% | -2.1% | -2.1% |
| Class B | -1.1% | -1.5% | -1.6% | -1.2% | -1.5% | -1.6% | -1.3% | -1.7% | -1.8% |
| Overall (EL+BL) | -1.2% | -1.7% | -1.7% | -1.2% | -1.5% | -1.6% | -1.4% | -1.8% | -1.9% |
| Overall (EL) | -1.9% | -2.8% | -2.9% | -2.5% | -3.0% | -3.3% | -2.2% | -2.9% | -3.0% |

Note, the use of a heterogeneous cluster for running simulations makes the timing data included in the excel sheets unreliable.

# Conclusion

This contribution reports the results for test 5.2.9 on inter-layer motion prediction and listed in TE5 description document JCTVC-K1105. The proposed changes do not impact the redundancy check and pruning process of the merge list construction and uses the same base layer motion information fetch mechanism for generation of base layer merge list candidate and primary motion vector predictor. The pixel location to be used in fetching the base layer motion information is the same as that used for the temporal candidate.

# References

- V. Seregin, P. Onno, S. Liu, T. Lee, C. Kim, H. Yang, “Description of Tool Experiment C5: Inter-layer syntax prediction using HEVC base layer,” JCTVC-K1105, Shanghai, CN, Oct. 2012

- K. Misra, J. Zhao, S. H. Kim, S. Deshpande, A. Segall “Description of scalable video coding technology proposal by Sharp,” JCTVC-K0031, Shanghai, CN, Oct. 2012

# Patent rights declaration(s)

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).