| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0300 |
| --- | --- |

| Title: | TE5: Cross-check report for TE5 test 5.2.8 on inter-layer motion vector prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kiran Misra, Andrew Segall 5750 NW Pacific rim Blvd., Camas WA 98607 USA | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP Laboratories of America, Inc. |  |  |

_____________________________

# Abstract

This is a cross-check report for TE5 test 5.2.8 (JCTVC-L0376) on inter-layer motion vector prediction. The source changes made on the SMuC 0.1.1 base source code was studied and found to be consistent with the TE5 test 5.2.8 description. Tests were carried out using conditions described in the document “Description of tool experiment C5: Inter-layer syntax prediction using HEVC base layer” (JCTVC-K1105). The performance of changes to the merge process and the AMVP process was measured separately using BD rate with SMuC 0.1.1 anchors and was found to be:

|  | Y-RA-2x | Y-RA-1.5x | Y-LDP-2x | Y-LDP-1.5x |
| --- | --- | --- | --- | --- |
| Merge only | -1.4% | -1.7% | -0.8% | -1.0% |
| AMVP only | -0.7% | -1.0% | -0.3% | -0.5% |

It is confirmed that all the rate-distortion results generated for the cross-check, match the results provided in JCTVC-L0376.

# Introduction

TE5 test 5.2.8 proposes predicted motion vectors (PMVs) in the merge list use the following ordering: left, up, inter-layer, upper left. The test also proposes that PMVs in the advanced motion vector prediction (AMVP) list use the following ordering: left blocks, upper blocks, inter-layer blocks, and temporal blocks. The number of list candidates for merge and AMVP are increased to 6 and 3 respectively for the enhancement layer.

The base layer motion information for an enhancement layer CU is determined by creating an up-sampled CU with the same dimensions as the enhancement layer CU. This up-sampled CU is populated using base layer data using the following three-step approach:

- A pre-determined mapping is used to copy data from the base layer into the up-sampled CU. Four MVs in the 8x8 pixels’ block of the enhancement layer are then replaced by the MV which is associated with the smallest RefIdx.

- If two or four 4x4 blocks in the 8x8 block have similar L0 MV and L1 MV, such 4x4 blocks are merged and get new L0 MV and L1 MV based on the average of MVs. The similarity is determined based on a threshold which depends on the spatial scale factor.

- Finally, each 4x4 block in the up-sampled CU undergoes the following processing. If the all neighboring blocks of a 4x4 block are coded by an intra mode, zero MV is assigned to the 4x4 block, otherwise the motion information determined in steps 1 and 2 are used for creating a motion field.

# Experimental Results

Test conditions outlined in “Description of tool experiment C5: Inter-layer syntax prediction using HEVC base layer” (JCTVC-K1105) were used to determine the BD rate with SMuC 0.1.1 anchors. The summary results are attached below. The detailed results can be found in the attached excel.

## Merge only changes:

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.2% | -2.1% | -2.1% |  |  |  |
| Class B | -1.5% | -2.3% | -2.4% | -1.7% | -2.6% | -2.8% |
| Overall (EL+BL) | -1.4% | -2.3% | -2.3% | -1.7% | -2.6% | -2.8% |
| Overall (EL) | -2.6% | -4.2% | -4.2% | -4.5% | -6.4% | -6.9% |
| Enc Time[%] | 94.5% |  |  | 92.3% |  |  |
| Dec Time[%] | 110.2% |  |  | 104.1% |  |  |
| Enc Mem[%] | 99.8% |  |  | 100.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.6% | -1.0% | -0.9% |  |  |  |
| Class B | -0.9% | -1.3% | -1.3% | -1.0% | -1.4% | -1.5% |
| Overall (EL+BL) | -0.8% | -1.2% | -1.2% | -1.0% | -1.4% | -1.5% |
| Overall (EL) | -1.3% | -2.0% | -2.0% | -2.3% | -3.0% | -3.0% |
| Enc Time[%] | 87.0% |  |  | 95.3% |  |  |
| Dec Time[%] | 99.0% |  |  | 103.4% |  |  |
| Enc Mem[%] | 99.8% |  |  | 100.5% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

## AMVP only changes:

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.6% | -1.1% | -1.1% |  |  |  |
| Class B | -0.8% | -1.2% | -1.2% | -1.0% | -1.2% | -1.4% |
| Overall (EL+BL) | -0.7% | -1.1% | -1.2% | -1.0% | -1.2% | -1.4% |
| Overall (EL) | -1.4% | -2.0% | -2.2% | -2.6% | -2.9% | -3.5% |
| Enc Time[%] | 96.8% |  |  | 94.0% |  |  |
| Dec Time[%] | 113.8% |  |  | 100.6% |  |  |
| Enc Mem[%] | 99.8% |  |  | 100.4% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.3% | -0.4% |  |  |  |
| Class B | -0.4% | -0.6% | -0.6% | -0.5% | -0.6% | -0.7% |
| Overall (EL+BL) | -0.3% | -0.5% | -0.6% | -0.5% | -0.6% | -0.7% |
| Overall (EL) | -0.5% | -0.9% | -0.9% | -1.0% | -1.2% | -1.4% |
| Enc Time[%] | 88.2% |  |  | 93.9% |  |  |
| Dec Time[%] | 100.7% |  |  | 101.9% |  |  |
| Enc Mem[%] | 99.9% |  |  | 100.4% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

Note, the timing information may be unreliable due to the heterogeneous nature of the cluster used.

# Conclusion

It is confirmed that all the rate-distortion results generated for the cross-check, match the results provided in JCTVC-L0376. The source code was studied and found to be consistent with the TE5 test 5.2.8 description and JCTVC-L0376.