| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0301 |
| --- | --- |

| Title: | AHG8: New 4:4:4 screen-content sequences for HEVC extension development |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Haoping Yu Huawei Technologies (USA) 2330 Central Expressway Santa Clara, CA95050 USA Robert Cohen Mitsubishi Electric Research Laboratories 201 Broadway Cambridge, MA 02139 U.S.A. Wen Gao (Huawei) Ya Cao (Huawei) Jing Ye (Huawei) Xian Wang (Huawei) Anthony Vetro (MERL) Huifang Sun (MERL) | Tel: Email: Tel: Email: | +1 408 330 4904 haoping.yu@huawei.com +1 617 621 7503 cohen@merl.com |
| Source: | Huawei Technologies and Mitsubishi Electric Research Laboratories |  |  |

_____________________________

# Abstract

To support further development of HEVC extensions, new 4:4:4 test sequences are proposed in this contribution. These sequences consist of computer screen content seen from typical software applications. They were produced by capturing computer DVI RGB output in the video format of 1280x720 at 60fps, with 8 bits per component. The original RGB sequences as well as their YUV version, color format conversion information, and copyright statement, have been uploaded to the folder /testsequences/FrExt-candidate-sequences/screen_content at ftp.tnt.uni-hannover.de.

# Sequence Description

In this contribution, the visual content of three different computer application scenarios was simulated and captured in three separate sequences. The three sequences are sc_map_1280x720_60_60bit, sc_programming_1280x720_60_8bit, and sc_wordEditing_1280x720_60_8bit. They have typical screen contents such as moving texts, moving graphics, switching windows, etc., often seen in Internet navigation, cloud computing, and word editing applications.

Sample frames of these sequences are shown below.

Figure 1. Frame 50 of sc_map_1280x720_60_60bit sequence

Figure 2. Frame 100 of sc_programming_1280x720_60_8bit sequence

Figure 3. Frame 580 of sc_wordEditing_1280x720_60_8bit sequence

Each of these sequences has 600 frames. The original RGB sequences were generated by recording the raw 24-bit RGB signals from a computer DVI output through a video capture card made by NCast. The recording was done by connecting the host computer DVI output to the capture card that was plugged in to a PCI-e slot on a separate computer. The DVI display format as well as the video capture format of the recording computer was set to 1280x720 at 60fps.

The YUV version of these sequences was made by converting the RGB data to ITU-R Rec. BT. 709 YCbCr 8-bit format through the following calculations:

t = 0.2126*R + 0.7152*G + 0.0722*B

Y   = floor (16 + 219 * t / 255 + 0.5)

Cb = floor (128 + 112 * (B – t) / (255 * (1 – 0.0722)) + 0.5)

Cr  = floor (128 + 112 * (R – t) / (255 * (1 – 0.2126)) + 0.5)

where t is a floating-point variable; R, G, B, Y, Cb, and Cr are integers, floor(x) is the largest integer of x, and single-precision floating-point processing was used in all calculations.

The RGB as well as YUV data were saved in 8-bit planar format.

For reference, the following calculations should be used to convert the above YCbCr back to RGB:

t = floor ((Y - 16) * 255 / 219+0.5)

R = clamp [t + (V-128) * 255 * (1-0.2126) / 112 + 0.5]

B = clamp [t + (U-128)*255*(1-0.0722)/112 + 0.5]

G = clamp [( t - 0.2126 * R - 0.0722 * B) / 0.7152 +0.5]

where clamp [x]= ( (x < 0) ? 0: ( (x > 255) ? 255: floor (x) ) ), and all calculations should be done in single-precision floating-point.

# Copyright information

The test sequence and all intellectual property rights therein remain the property of their respective copyright holders. This material can only be used for the purpose of academic research and development of standards. This material cannot be distributed with charge. The owner makes no warranties with respect to the material and expressly disclaims any warranties regarding its fitness for any purpose.

Owners of these sequences: Huawei Technologies, and MERL

Production: Huawei Technologies, and MERL

# Recommendation

These sequences provide some unique visual characteristics that are common in screen content. We recommend that they be adopted as test sequences in the further development of HEVC 4:4:4 extensions.

# Patent rights declaration(s)

Huawei may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MERL does not have any current or pending patent rights relating to the technology described in this contribution.