| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0303 |
| --- | --- |

| Title: | AHG8: P2M based dual-coder extension of HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tao Lin, Shuhui Wang, Peijun Zhang, Kailun Zhou | Tel: Email: | 86-21-5103-0961 lintao@tongji.edu.cn |
| Source: | Tongji University, Shanghai, China |  |  |

_____________________________

# Abstract

This contribution presents a Pseudo-2D-matching (P2M) tool based dual-coder extension of HEVC. P2M coder is essentially a 2D pixel data oriented dictionary-like coder. The dual-coder approach integrates P2M coder into traditional hybrid coder. It is reported that the BD-rate reduction is from 0% to 85% with an average of 35% in AI-main and from 0% to 73% with an average of 27% in LP-main to code the 24 test sequences of AHG8.

# Introduction

A Pseudo-2D-matching (P2M) coder is added to the traditional hybrid coder. Two coders are used to pre-code an LCU and the coder with minimum RD cost is selected as the final coder to code the LCU. The P2M based dual-coder technique is very effective to code screen contents.

# P2M Coding Tool

P2M coding is illustrated in Figure 1. The LCU size of Figure 1 is 16x16 pixels in packed pixel format.

LCU

0

LCU

1

LCU

2

LCU

h-1

LCU

h

LCU

h+1

LCU

h+2

LCU

2h-1

LCU

m

1st matched string (red) has 42 samples=14 pixels

2nd matched string (blue) has 33 samples

3rd matched string (green) has 45 samples

- P2M coding.

The LCU m of Figure 1 is coded using P2M coding tool. The searching-window (previously coded pixels) is treated as a long 1D string that has one LCU followed by another LCU in LCU coding order. Within an LCU, vertical scanning is employed to convert the packed pixels into a 1D sample string. Starting from the first sample of the LCU m, the encoder searches the optimal (usually longest) string in the searching-window that matches the sample string in LCU m. The string found in the searching-window is called matching string and the sample string in LCU m is called matched string. Figure 1 shows the first three matching (and matched) strings for LCU m:

- The 1st matching (and matched) string shown in red color has 42 samples (14 pixels) and the matching string is located in LCU 0 (the last 15 samples of the last column) and LCU 1 (the first 27 samples of the first column).

- The 2nd matching (and matched) string shown in blue color has 33 samples (11 pixels) and the matching string is located inside LCU 0 (the last 21 samples of the third column followed by the first 12 samples of the fourth column);

- The 3rd matching (and matched) string shown in green color has 45 samples (15 pixels) and the matching string is located in LCU h-1 (the last 39 samples of the last column) and LCU h (the first 6 samples of the first column).

# P2M based dual-coder syntax and semantics

## SPS

Dual-coder mode can be enabled in SPS. A syntax elements dual_coder_mode is added to SPS as shown in Figure 2 just before the log2_max_pic_order_cnt_lsb_minus4 syntax element.

Syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| <snip ……> |  |
| dual_coder_mode | ue(v) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| <snip ……> |  |
| rbsp_trailing_bits( ) |  |
| } |  |

- SPS syntax to enable dual-coder mode.

Semantics

dual_coder_mode equal to 0 specifies that the sequence is a traditional hybrid coding sequence and dual-coder mode is disabled. dual_coder_mode equal to 1 specifies that both P2M coder and traditional hybrid coder are used to code the sequence in an largest coding unit (LCU) adaptive way. dual_coder_mode equal to 2 is reserved.

## Largest Coding Unit (Coding Tree Unit)

P2M based dual-coder bitstream is a mix of P2M coder bitstream segment and traditional hybrid coder bitstream segment. For the decoder to parse the coder type (P2M coder or hybrid coder) properly, in the coding tree unit (CTU, synonymous with largest coding unit or LCU) layer, a syntax element ctu_coded_by_p2m_coder_flag is added to indicate whether the ctu is coded by P2M coder or not. The ctu layer syntax is shown in Figure 3.

Syntax

| coding_tree_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| xCtb = ( CtbAddrRS % PicWidthInCtbsY ) << Log2CtbSizeY |  |
| yCtb = ( CtbAddrRS / PicHeightInCtbsY ) << Log2CtbSizeY |  |
| CtbAddrInSliceSeg = CtbAddrInRS − slice_segment_address |  |
| if( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ) |  |
| sao( xCtb >> Log2CtbSizeY, yCtb >> Log2CtbSizeY ) |  |
| if(dual_coder_mode = = 0) |  |
| coding_quadtree( xCtb, yCtb, Log2CtbSizeY, 0 ) |  |
| else { |  |
| ctu_coded_by_p2m_coder_flag | ae(v) |
| if(cu_coded_by_p2m_coder_flag = = 0) //HYBRID_CODER |  |
| coding_quadtree( xCtb, yCtb, Log2CtbSizeY, 0 ) |  |
| else |  |
| p2m_coder( x0, y0, Log2CtbSizeY ) |  |
| } |  |
| } |  |

- ctu layer syntax.

Semantics

ctu_coded_by_p2m_coder_flag equal to 0 specifies that the ctu is coded by the hybrid coder, which can use any hybrid coding tools specified in this HEVC document. ctu_coded_by_p2m_coder_flag equal to 1 specifies that the ctu is coded by the P2M coder.

The p2m_coder() syntax is shown in Figure 4.

NOTE 1 – NumSamplesInCU is the total number of Y samples, U samples, and V samples in a coding unit (e.g. 16x16x3=768 for a coding tree unit of size 16x16 pixels in 444 color format, 64x64x3=12288 for a coding tree unit of size 64x64 pixels in 444 color format).

Syntax

| p2m_coder( x0, y0, Log2CtbSizeY ) { | Descriptor |
| --- | --- |
| decoded_sample_count = 0 |  |
| NumSamplesInCTU = (3 << (Log2CtbSizeY << 1)) |  |
| while(decoded_sample_count < NumSamplesInCTU) { |  |
| matching_string_flag | ae(v) |
| if(matching_string_flag = = 1) { |  |
| matching_string_distance_use_recent_8_flag | ae(v) |
| if(matching_string_distance_use_recent_8_flag = = 1) { |  |
| matching_string_distance_recent_8_idx | ae(v) |
| matching_string_distance_minus1 = Recent8Distances[matching_string_distance_recent_8_idx] |  |
| for( i = matching_string_distance_recent_8_idx; i > 0; i-- ) |  |
| Recent8Distances[i] = Recent8Distances[i-1] |  |
| Recent8Distances[0] = matching_string_distance_minus1 |  |
| } |  |
| else { |  |
| matching_string_distance_minus1 | ae(v) |
| for( i = 7; i > 0; i-- ) |  |
| Recent8Distances[i] = Recent8Distances[i-1] |  |
| Recent8Distances[0] = matching_string_distance_minus1 |  |
| } |  |
| matching_string_length_minus2 | ae(v) |
| decoded_sample_count += (matching_string_length_minus2 + 2) |  |
| } |  |
| else { |  |
| unmatchable_sample_residual_abs | ae(v) |
| unmatchable_sample_residual_sign | ae(v) |
| decoded_sample_count += 1 |  |
| } |  |
| } |  |
| } |  |

- Syntax of p2m_coder.

Semantics

matching_string_flag specifies whether the succeeding dictionary coding element is a matching string or an unmatchable (luma or chroma) sample residual.

If matching_string_flag = 1, the succeeding dictionary coding element is a matching string, otherwise, the succeeding dictionary coding element is an unmatchable (luma or chroma) sample residual.

matching_string_distance_use_recent_8_flag equal to 1 specifies that the current matching_string_distance_ minus1 is exactly the same as one of the eight most recently decoded matching_string_distance_minus1 stored in an eight-entry array named Recent8Distances[8] and the succeeding syntax element is an index of the array. matching_string_distance_use_recent_8_flag equal to 0 specifies that the succeeding syntax element is matching_string_distance_minus1.

matching_string_distance_recent_8_idx is an index to Recent8Distances[8].

matching_string_distance_minus1 + 1 specifies the distances between the first sample of current sample-string to be decoded/constructed and the first sample of matching sample-string previously decoded/constructed in the dictionary, which is actually DPB and the current decoded picture reordered in a “dictionary-scan” format defined as a column-wise-packed-pixel cu-by-cu format. The value of matching_string_distance_minus1 shall be in the range of 0 to 4194303, inclusive

matching_string_length_minus2 + 2 specifies the length of the matching sample-string. The value of matching_string_length_minus2 shall be in the range of 0 to 271, inclusive.

unmatchable_sample_residual_abs is the absolute difference between the unmatchable sample (y or u or v component of a pixel) to be decoded and its predictor that is the same component (y or u or v) of the previous pixel in the dictionary. The value of unmatchable_sample_residual_abs shall be in the range of 0 to (256<< bit_depth_luma_minus8) – 1 for luma component or (256<< bit_depth_chroma_minus8) – 1 for chroma component, inclusive.

unmatchable_sample_residual_sign specifies the sign of an unmatchable sample residual. If unmatchable_sample_residual_sign is equal to 0, the corresponding unmatchable sample residual has a positive value. Otherwise (unmatchable_sample_residual_sign is equal to 1), the corresponding unmatchable sample residual has a negative value.

# BD-rate Experimental Results

Reference: HM-range-extensions branch (revision 3055, based on HM-9.0.1).

Target: HM-range-extensions branch based P2M+hybrid dual-coder extension software in this proposal.

Sequences: 24 test sequences of AHG8.

See JCTVC-L0303-v1.xls and JCTVC-L0303-v1.ppt,

# Conclusions

In this document, we have proposed the P2M based dual-coder extension of HEVC. The modification to the existing version of HEVC is straightforward. Only SPS and CTU syntax need to be changed. The BD-rate reduction is very significant (xx% for AI-main and yy% for LP-main) for 24 screen content test sequences. We propose to adopt the P2M dual-coder extension.

# Patent rights declaration(s)

Tongji University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).